3GPP TSG-RAN5 Meeting #92-e	R5-21xxxx

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5015 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA RRC test case 8.6.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per test spec 36.523-1 section 8.6.4.5.3.2-2 timer duration is 5 seconds to ensure that the UE detects T310 which is more than required. As per 36.331 section 6.3.6, T310 itself can be defined for a maximum of 2s, so there is no need to turn off cell for 5Sec for UE to detect the T310 expiry. Hence timer duration has been reduced to 3 seconds |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reduced wait timer to 3 seconds to ensure that cell is turned on before ERRC perform cell scan |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.4.5.3.2-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s210767 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <START OF CHANGES>

8.6.4.5 Radio Link Failure logging / Reporting of ECGI of the PCel

8.6.4.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed intra-frequency handover and reported that the UE has radio link failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing previousPCellId }

}

8.6.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.7.4, 5.3.7.5, 5.3.11.3 and 5.6.5.3.

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 36.331, clause 5.3.7.5]

The UE shall:

...

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	upon T310 expiry; or

1>	upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected;

2>	store the following radio link failure information in the VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-Identity to the RPLMN;

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

NOTE:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration including the mobilityControlInfo message was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to rlf;

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report 48, hours after the radio link failure is detected, upon power off or upon detach.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN, set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

1>	if the rlf-Report is included in UEInformationResponse:

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers.

...

1>	if the logMeasReport is included in the UEInformationResponse:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.4.5.3	Test description

8.6.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.4.5.3.2	Test procedure sequence

Table 8.6.4.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2" and "T3" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.4.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | "Off" | No Cells are available. (NOTE 1). |
| T3 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | Only Cell 2 is available. (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |

Table 8.6.4.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 2 parameter according to the row "T1" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra-frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 6A | Wait for 1s to ensure that the UE completes the acquisition of essential SI | - | - | - | - |
| 7 | The SS changes Cell 1 and Cell 2 parameter according to the row "T2" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 8 | Wait for 3s to ensure that the UE detects T310 expiry. | - | - | - | - |
| 9 | The SS changes Cell 2 parameter according to the row "T3" in Table 8.6.4.5.3.2-1. | - | - | - | - |
| 10 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 2. | --> | RRCConnectionReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCConnectionReestablishment message on Cell 2. | <-- | RRCConnectionReestablishment | - | - |
| 12 | The UE transmits an RRCConnectionReestablishmentComplete message with radio link failure information on Cell 2. | --> | RRCConnectionReestablishmentComplete | - | - |
| 13 | The SS transmits an RRCConnectionReconfiguration message on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2. | --> | RRCConnectionReconfigurationComplete | - | - |
| 15 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 2? | --> | UEInformationResponse | 1 | P |
| 17 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | - |

8.6.4.5.3.3	Specific message contents

Table 8.6.4.5.3.3-0: Conditions for specific message contents
in Tables 8.6.4.5.3.3-2 and 8.6.4.5.3.3-10

| Condition | Explanation |
| --- | --- |
| Band > 64 | This condition applies if the band number is bigger than 64. |

Table 8.6.4.5.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.6.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.4.5.3.3-2: MeasConfig (Table 8.6.4.5.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObect[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-3: MeasurementReport (step 4, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.6.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.4.5.3.3-5: MobilityControlInfo (Table 8.6.4.5.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-6: RRCConnectionReestablishmentRequest (step 10, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-7: RRCConnectionReestablishmentComplete (step 12, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-8: RRCConnectionReconfiguration (step 13, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-9: UEInformationRequest (step 15, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| rlf-ReportReq-r9 | TRUE |  |  |
| logMeasReportReq-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.4.5.3.3-10: UEInformationResponse (step 16, Table 8.6.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) |  |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 | Not present |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity, otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Physical cell Identity of Cell 2 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 2 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 2 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Not present or the same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value |  |  |
| connectionFailureType-r10 | rlf |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<END OF CHANGES>