3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210779

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0018 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 6.1.1.2 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MCPTT TC 6.1.1.2 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s210780 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

1.1	Change 1	3

1.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	Nemergent Solutions Client v7.0	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.1.2 which is part of the ATS iwd-TTCN3-B2020-09_D21wk24 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fidel Liberal
	fidel.liberal@ehu.eus

## Verification Test Summary

Test Case: 	6.1.1.2

ATS Version:	ATS  iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

Change1 of R5s210398

## Change 2

Change2 of R5s210398

## Additional changes by MCC160

### cs_FloorCtrlMsg_QueuePositionInfo

According to 24.380 clause 8.2.12 User ID and SSRC of queued floor participant are applicable for off-network only and according to 36.579-1 Table 5.5.6.10-1 both fields are "not present".

NOTE: Further changes of floor control messages need to be discussed.

template (value) FloorControlMsg_Type cs_FloorCtrlMsg_QueuePositionInfo(O4_Type p_SSRC_Queued, B16_Type p_FloorInd, MCX_FloorControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { queuePositionInfoMessage := { commonHdr := cs_FloorControl_CommonHdr(fl_MCX_MediaControl_SubtypeTX('01001'B, p_AckRequest), tsc_MCX_SSRC_NW), // QueuePositionInfo subtype 24.380 clause 8.2.2.1 userId := omit, ssrcQueuedParticipant := omit, queuedUserId := omit, queueInfo := cs_FloorControl_QueueInfo('01'O, '00'O), trackInfo := omit, floorIndicator := cs_FloorControl_FloorIndicator(p_FloorInd) } };

type record FloorControl_QueuePositionInfo { FloorControl_CommonHdr commonHdr, FloorControl_UserID userId optional, RTP_SSRC ssrcQueuedParticipant optional, FloorControl_QueuedUserID queuedUserId optional, FloorControl_QueueInfo queueInfo optional, FloorControl_TrackInfo trackInfo optional, FloorControl_FloorIndicator floorIndicator optional };

### f_MCPTT_SdpAnswer_TX

f_MCPTT_SdpAnswer_TX does not support option b.iii yet, where the client implicitly requests a grant, which is not even accepted.

type enumerated MCX_MediaControlConditionSdpAnswer_Type { implicitFloorRequestGranted, /* if requested in the SDP offer, the implicit floor request gets granted in the SDP answer (conditions IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED */ implicitFloorRequestNoted, /* if requested in the SDP offer, the implicit floor request gets accepted but not granted (condition IMPLICIT_GRANT_REQUESTED only) */ implicitFloorRequestRejected /* even when if requested in the SDP offer, the SDP answer does neither contain the mc_implicit_request nor the mc_granted parameter */ };

function f_MCPTT_SdpAnswer_TX(SDP_Message p_SDP_Offer, MCX_MediaControlConditionSdpAnswer_Type p_MediaControlCondition := implicitFloorRequestNoted, template (omit) bitstring p_EncodedMikeyMsg := omit) runs on MCX_IMS_PTC return template (value) SDP_Message { /* NOTE: SDP answer may contain security key in case of a first-to-answer call */ var charstring v_FmtAudio := f_MCX_SdpOffer_GetFmtAudio(p_SDP_Offer); var SDP_media_desc v_MediaDescAudio := valueof(f_MCX_SDP_GetAudioDescription(p_SDP_Offer)); var template (omit) SDP_media_desc v_MediaDescApplOffer := f_MCX_SDP_GetMediaControlDescription(p_SDP_Offer); var template (value) SemicolonParam_List v_FmtpParamsFloorControl := {}; var template (value) SDP_attribute_list v_FurtherAudioAttributes := {}; var MCPTT_SDP_FloorControlParameters_Type v_FloorControlParameters; if (isvalue(v_MediaDescApplOffer)) { v_FloorControlParameters := f_MCX_SDP_MediaDescription_FloorControlDecodeParameters(valueof(v_MediaDescApplOffer)); v_FloorControlParameters.mc_granted := omit; v_FloorControlParameters.mc_ssrc := omit; if (isvalue(v_FloorControlParameters.mc_priority)) { // mc_priority: if a value is provided in the offer: "3" or the value provided in the offer, whichever is the lower value; otherwise not present if (v_FloorControlParameters.mc_priority > 3) { v_FloorControlParameters.mc_priority := 3; } } if (isvalue(v_FloorControlParameters.mc_implicit_request)) { // implicit grant requested if (p_MediaControlCondition == implicitFloorRequestRejected) { // IMPLICIT_GRANT_REQUESTED == false v_FloorControlParameters.mc_implicit_request := omit; } else { // IMPLICIT_GRANT_REQUESTED == true v_FloorControlParameters.mc_ssrc := oct2int(f_MCX_IMS_PTC_SSRC_UE_Get()); if (p_MediaControlCondition == implicitFloorRequestGranted) { // IMPLICIT_FLOOR_GRANTED == true v_FloorControlParameters.mc_granted := true; } } } v_FmtpParamsFloorControl := fl_MCX_SDP_MediaDescription_FloorControlEncodeParameters(v_FloorControlParameters); } if (f_SDP_AttributeList_CheckAttribute(v_MediaDescAudio.attributes, cr_SDP_Attribute_recvonly)) { v_FurtherAudioAttributes := { cs_SDP_Attribute_sendonly }; // According to 24.379 clause 6.2.2 (SDP answer generation) } else if (f_SDP_AttributeList_CheckAttribute(v_MediaDescAudio.attributes, cr_SDP_Attribute_sendonly)) { v_FurtherAudioAttributes := { cs_SDP_Attribute_recvonly }; // According to 24.379 clause 6.2.2 (SDP answer generation) } return fl_MCPTT_BuildSDP_TX(v_FmtAudio, v_FmtpParamsFloorControl, p_EncodedMikeyMsg, v_FurtherAudioAttributes); }

function f_MCX_CallSetupMO_Send200OK_ReceiveACK(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, MCX_MediaControlConditionSdpAnswer_Type p_MediaControlCondition := implicitFloorRequestNoted charstring p_CalleeUri := tsc_MCPTT_PublicServiceId_A, template (omit) bitstring p_EncodedMikeyMsg := omit) runs on IMS_PTC { var INVITE_Request v_InviteRequest := p_InviteRequestWithSdp.Invite; var SDP_Message v_SDP_Offer := p_InviteRequestWithSdp.SdpOffer; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(p_InviteRequestWithSdp.RoutingInfo); var template (value) SDP_Message v_SDP_Answer := f_MCPTT_SdpAnswer_TX(v_SDP_Offer, p_MediaControlCondition , p_EncodedMikeyMsg); f_MCX_InviteMO_Send200OK(v_InviteRequest.msgHeader, v_RoutingInfo_DL, p_CalleeUri, v_SDP_Answer); f_MCX_InviteMO_ReceiveACK(p_CalleeUri, v_InviteRequest); }

Further changes are done, where f_MCPTT_SdpAnswer_TX and f_MCX_CallSetupMO_Send200OK_ReceiveACK are used.

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SS_Log_Coversheet.txt

- Signalling-TC6.1.1.2.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

| MCC160 Comment | SS log in R5s210780 withdrawn, replaced by SS log in R5s211167, which is accepted. |
| --- | --- |

# References

| [1] | R5s210780: Supporting information for the addition of MCPTT test case 6.1.1.2 to the MCX ATS |
| --- | --- |