3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210782

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1828 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Re-verification of NR PDCP test case 7.1.3.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify NR PDCP test case 7.1.3.5.5 to the 5G/ENDC ATS for FR1 path, using IWD 21wk24. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR PDCP test case 7.1.3.5.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.5.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	5

2.4 Change 4	8

2.5 Change 5	10

3	Branches executed	11

4	Execution Log File	11

4.1	MediaTek MT6885	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.3.5.5 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.3.5.5

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6885

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | cs_NR_SS_ReConfig_NotACK_NextRLC_PDU |
| --- | --- |
| Reason for change | As per the current implementtaion of the template cs_NR_SS_ReConfig_NotACK_NextRLC_PDU, UL & DL RLC SN Size is set as 12bit assuming the bearer is SRB. This template is used in TC 7.1.3.5.5 for the DRB and the SN size shall be wrongly set as 12 instead of 18 for the DRB. |
| Summary of change | New template is defined with name cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB, to correct the SN size to 18 bit for the DRB |
| TTCN module | NR_ASP_templates_RB_Config.ttcn |
| MCC160 Comment | Accepted in principle but different implementation |

Before Change

template (value) NR_RadioBearer_Type cs_NR_SS_ReConfig_NotACK_NextRLC_PDU( template (value) NR_RadioBearerId_Type p_RB) :=

cs_NR_SS_RadioBearer_Config(p_RB,

omit,

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(cs_38508_UL_RLC_SRB, cs_38508_DL_RLC_SRB, cs_NR_RLC_TestModeConfig_NotACK_NextRLC_PDU),

omit,

omit),

omit);

After Change

template (value) NR_RadioBearer_Type cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB( template (value) NR_RadioBearerId_Type p_RB) :=

cs_NR_SS_RadioBearer_Config(p_RB,

omit,

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(f_NR_UL_AM_RLC(tue), f_NR_DL_AM_RLC(true), cs_NR_RLC_TestModeConfig_NotACK_NextRLC_PDU),

omit,

omit),

omit);

MCC160 Implementation

//New Template

Template (value) NR_RadioBearer_Type cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB( template (value) NR_RadioBearerId_Type p_RB,

boolean p_IsFR1) :=

cs_NR_SS_RadioBearer_Config(p_RB,

omit,

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(f_NR_UL_AM_RLC(p_IsFR1), f_NR_DL_AM_RLC(p_IsFR1), cs_NR_RLC_TestModeConfig_NotACK_NextRLC_PDU),

omit,

omit),

omit);

## 2.2 Change 2

| Function name | f_NR_SS_ConfigRB_NotACK_NextRLC_PDU |
| --- | --- |
| Reason for change | As per the current implementtaion of the template cs_NR_SS_ReConfig_NotACK_NextRLC_PDU, UL & DL RLC SN Size is set as 12bit assuming the bearer is SRB. This template is used in TC 7.1.3.5.5 for the DRB and the SN size shall be wrongly set as 12 instead of 18 for the DRB. |
| Summary of change | Modified f_NR_SS_ConfigRB_NotACK_NextRLC_PDU to use the new defined template cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB, see change 1. |
| TTCN module | NR_ConfigurationSteps.ttcn |
| MCC160 Comment | Accepted in principle but different implementation |

Before Change

function f_NR_SS_ConfigRB_NotACK_NextRLC_PDU (NR_CellId_Type p_NR_CellId,

template (value) NR_RadioBearerId_Type p_RB,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now

) runs on NR_BASE_PTC

{

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU(p_RB) }, p_TimingInfo); //71355@wk24

}

After Change

function f_NR_SS_ConfigRB_NotACK_NextRLC_PDU (NR_CellId_Type p_NR_CellId,

template (value) NR_RadioBearerId_Type p_RB,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now

) runs on NR_BASE_PTC

{

f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(p_RB) }, p_TimingInfo); //71355@wk24

}

MCC160 Implementation

function f_TC_7_1_3_5_5_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCell, integer p_DRB_Id) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo;

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

var template (value) RadioBearerConfig  v_RadioBearerConfig;

var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit;

var template (value) CellGroupConfig v_SCGConfig:= f_NR_GetSCGConfigDef(nr_Cell1);

var NR_DRB_COMMON_IND v_Primary;

var NR_DRB_COMMON_IND v_Secondary;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var boolean v_IsENDC:= f_GetTestcaseAttrib_ENDC(testcasename());

var template (value) RlcBearerRouting_Type v_RlcBearerRoutingPcell;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var boolean v_PduRlcPrimary:= false;

var boolean v_PduRlcSecondary:= false;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_PSCell); //@sic R5s210782 sic@

timer t_WaitForPdu:= 5.0;

…

//@siclog "Step 2A" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_CommonRadioBearerConfig(p_PSCell, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(cs_NR_RadioBearerId_DRB(tsc_NR_DRB2),v_IsFR1) }, cs_TimingInfo_Now); //@sic R5s210782 sic@

//@siclog "Step 3" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[0], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

…

//@siclog "Step 8AA" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_CommonRadioBearerConfig(p_PSCell, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(cs_NR_RadioBearerId_DRB(tsc_NR_DRB2),v_IsFR1) }, cs_TimingInfo_Now); //@sic R5s210782 sic@

…

}

## 2.3 Change 3

| Function name | f_TC_7_1_3_5_5_NR_TestBody |
| --- | --- |
| Reason for change | As per Steps 4B/4C and Steps 8E/8F, Retransmisson for PDCP PDU in the AM RLC primary entity /Secondary Entity shall be handled in the test case /TTCN. However, this is not correct, since the retransmitted AM RLC SDU shall be discarded by the SS lower layers as duplicate PDUs. |
| Summary of change | Removed Steps 4B/4C and 8E/8F from the TTCN. Note : An associated prose CR on 38.523-1 will be raised at RAN5#92. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted in principle conditional to prose CR agreement at RAN5#92 |

Before Change

function f_TC_7_1_3_5_5_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCell, integer p_DRB_Id) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo;

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

var template (value) RadioBearerConfig  v_RadioBearerConfig;

var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit;

var template (value) CellGroupConfig v_SCGConfig:= f_NR_GetSCGConfigDef(nr_Cell1);

var NR_DRB_COMMON_IND v_Primary;

var NR_DRB_COMMON_IND v_Secondary;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var boolean v_IsENDC:= f_GetTestcaseAttrib_ENDC(testcasename());

var template (value) RlcBearerRouting_Type v_RlcBearerRoutingPcell;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var boolean v_PduRlcPrimary:= false;

var boolean v_PduRlcSecondary:= false;

timer t_WaitForPdu:= 5.0;

//Secondary Entity

if (v_IsENDC)

{

v_RlcBearerRoutingPcell:= crs_RlcBearerRouting_Eutra(eutra_Cell1);

}

else

{

v_RlcBearerRoutingPcell:= cs_RlcBearerRouting_NR(nr_Cell1);

}

p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc;

//Generate a SDU with PDU sizes 48, 64, 72, 94 bytes

v_SduList := f_GenerateSDU (p_NR_PDCP_State, {48, 64, 72, 94}); //@sic R5s210376 sic@

//@siclog "Step 2" siclog@

//SS transmits to activate PDCP Duplication for split DRB.

f_NR_SS_CommonCellConfig(p_PSCell,

cas_NR_MacCommandTrigger_REQ(p_PSCell,

cs_TimingInfo_Now,

cs_NR_MAC_DuplicationActDeact(int2bit(1,8))));

//@siclog "Step 2A" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_ConfigRB_NotACK_NextRLC_PDU(p_PSCell, cs_NR_RadioBearerId_DRB(tsc_NR_DRB2));

//@siclog "Step 3" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[0], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

//@siclog "Step 4-4A" siclog@

//EXCEPTION: Steps 4-4A below occur in any sequence

interleave {

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary{} //@sic R5s210376 sic@

//Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary{} //@sic R5s210376 sic@

};

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[0],"Step 4");

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[0],"Step 4A");

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

//@sic R5-213461 sic@

//EXCEPTION: Steps 4B-4C below may occur in any sequence. It is possible that only one of steps 4B or 4C occur, wait for 5 sec to allow these steps to take place.

//@siclog "Step 4B-4C" siclog@

//Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

//Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

t_WaitForPdu.start;

alt {

//Does UE transmit a PDCP Data PDU on on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{

if (not (v_PduRlcPrimary))

{

v_PduRlcPrimary:= true;

if (not(v_PduRlcSecondary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C: Second Retransmisson for PDCP PDU in the AM RLC primary entity");

}

}

//Does UE transmit a PDCP Data PDU on on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary

{

if(not(v_PduRlcSecondary))

{

v_PduRlcSecondary:= true;

if(not(v_PduRlcPrimary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C: Second Retransmisson for PDCP PDU in the AM RLC secondary entity");

}

}

[] t_WaitForPdu.timeout

{

if((not(v_PduRlcPrimary))and(not(v_PduRlcSecondary)))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C");

}

}

};

//@siclog "Step 5" siclog@

//SS transmits to deactivate PDCP duplication for split DRB

f_NR_SS_CommonCellConfig(p_PSCell,

cas_NR_MacCommandTrigger_REQ(p_PSCell,

cs_TimingInfo_Now,

cs_NR_MAC_DuplicationActDeact()));

//@siclog "Step 6" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity.

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[1], v_TimingInfo);

//@siclog "Step 7" siclog@

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

f_NR_PDCP_Recv(p_NR_PDCP_State,p_PSCell,v_SduList[1], cr_RlcBearerRouting_NR(p_PSCell));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

//@siclog "Step 8-8A" siclog@

//The SS transmits an NR RRCReconfiguration message to activate parameters for PdcpDuplication

if (v_IsENDC){ //@sic R5-206329 sic@

v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id,cs_NR_PDCP_Config_UlPath(infinity_, len18bits, len18bits,-,-,-,true))};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList);

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig);

}

else { //@sic R5-206329 sic@

v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(infinity_, len18bits, len18bits,-,-,-,true))};

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList);

v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2);

//SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB.

f_NR_SendRRCReconfiguration (p_PSCell, -,-, v_V1530Ext);

}

//@siclog "Step 8AA" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_ConfigRB_NotACK_NextRLC_PDU(p_PSCell, cs_NR_RadioBearerId_DRB(tsc_NR_DRB2));

//@siclog "Step 8B" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[2], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //@sic R5s210376 sic@

//@siclog "Step 8C-8D" siclog@

interleave {

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary{} //@sic R5s210376 sic@

//Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , v_RlcBearerRoutingPcell, -, {v_ExpectedPdu})) -> value v_Secondary{} //@sic R5s210376 sic@

}

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[2],"Step 8C");

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[2],"Step 8D");

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

//@sic R5-213461 sic@

//EXCEPTION: Steps 8E-8F below may occur in any sequence. It is possible that only one of steps 8E or 8F occur, wait for 5 sec to allow these steps to take place.

//@siclog "Step 8E-8F" siclog@

//Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

//Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

v_PduRlcPrimary:= false;

v_PduRlcSecondary:= false;

t_WaitForPdu.start;

alt {

//Does UE transmit a PDCP Data PDU on on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{

if(not(v_PduRlcPrimary))

{

v_PduRlcPrimary:= true;

if(not(v_PduRlcSecondary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F: Second Retransmisson for PDCP PDU in the AM RLC primary entity");

}

}

//Does UE transmit a PDCP Data PDU on on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary

{

if(not(v_PduRlcSecondary))

{

v_PduRlcSecondary:= true;

if(not(v_PduRlcPrimary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F: Second Retransmisson for PDCP PDU in the AM RLC secondary entity");

}

}

[] t_WaitForPdu.timeout

{

if((not(v_PduRlcPrimary))and(not(v_PduRlcSecondary)))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F");

}

}

};

//@siclog "Step 9" siclog@

//The SS stops allocating any UL grant for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s210376 sic@

}

//@siclog "Step 10" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity..

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[3], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 11");

t_WaitForPdu.stop; //@sic R5s210376 sic@

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 PDCP DATA PDU not received");

}

}

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

//@siclog "Step 13" siclog@

// Does UE transmits a PDCP Data PDU on the AM RLC primary or secondary entity  in next five seconds?

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

//@siclog "Step 11-12" siclog@

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , v_RlcBearerRoutingPcell, -, {v_ExpectedPdu})) -> value v_Secondary

{ //UE transmits a PDCP Data PDU on the AM RLC secondary entity.

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[] t_WaitForPdu.timeout{

//Do nothing

}

}

//Coordination message to trigger the EUTRA Postamble

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

}

After Change

function f_TC_7_1_3_5_5_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCell, integer p_DRB_Id) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo;

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

var template (value) RadioBearerConfig  v_RadioBearerConfig;

var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit;

var template (value) CellGroupConfig v_SCGConfig:= f_NR_GetSCGConfigDef(nr_Cell1);

var NR_DRB_COMMON_IND v_Primary;

var NR_DRB_COMMON_IND v_Secondary;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var boolean v_IsENDC:= f_GetTestcaseAttrib_ENDC(testcasename());

var template (value) RlcBearerRouting_Type v_RlcBearerRoutingPcell;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var boolean v_PduRlcPrimary:= false;

var boolean v_PduRlcSecondary:= false;

timer t_WaitForPdu:= 5.0;

//Secondary Entity

if (v_IsENDC)

{

v_RlcBearerRoutingPcell:= crs_RlcBearerRouting_Eutra(eutra_Cell1);

}

else

{

v_RlcBearerRoutingPcell:= cs_RlcBearerRouting_NR(nr_Cell1);

}

p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc;

//Generate a SDU with PDU sizes 48, 64, 72, 94 bytes

v_SduList := f_GenerateSDU (p_NR_PDCP_State, {48, 64, 72, 94}); //@sic R5s210376 sic@

//@siclog "Step 2" siclog@

//SS transmits to activate PDCP Duplication for split DRB.

f_NR_SS_CommonCellConfig(p_PSCell,

cas_NR_MacCommandTrigger_REQ(p_PSCell,

cs_TimingInfo_Now,

cs_NR_MAC_DuplicationActDeact(int2bit(1,8))));

//@siclog "Step 2A" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_ConfigRB_NotACK_NextRLC_PDU(p_PSCell, cs_NR_RadioBearerId_DRB(tsc_NR_DRB2));

//@siclog "Step 3" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[0], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

//@siclog "Step 4-4A" siclog@

//EXCEPTION: Steps 4-4A below occur in any sequence

interleave {

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary{} //@sic R5s210376 sic@

//Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary{} //@sic R5s210376 sic@

};

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[0],"Step 4");

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[0],"Step 4A");

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

//@sic R5-213461 sic@

//EXCEPTION: Steps 4B-4C below may occur in any sequence. It is possible that only one of steps 4B or 4C occur, wait for 5 sec to allow these steps to take place.

//@siclog "Step 4B-4C" siclog@

//Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

//Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

/* REMOVED Steps 4B/4C

t_WaitForPdu.start;

alt {

//Does UE transmit a PDCP Data PDU on on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{

if (not (v_PduRlcPrimary))

{

v_PduRlcPrimary:= true;

if (not(v_PduRlcSecondary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C: Second Retransmisson for PDCP PDU in the AM RLC primary entity");

}

}

//Does UE transmit a PDCP Data PDU on on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary

{

if(not(v_PduRlcSecondary))

{

v_PduRlcSecondary:= true;

if(not(v_PduRlcPrimary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C: Second Retransmisson for PDCP PDU in the AM RLC secondary entity");

}

}

[] t_WaitForPdu.timeout

{

if((not(v_PduRlcPrimary))and(not(v_PduRlcSecondary)))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4B-4C");

}

}

}; */

//@siclog "Step 5" siclog@

//SS transmits to deactivate PDCP duplication for split DRB

f_NR_SS_CommonCellConfig(p_PSCell,

cas_NR_MacCommandTrigger_REQ(p_PSCell,

cs_TimingInfo_Now,

cs_NR_MAC_DuplicationActDeact()));

//@siclog "Step 6" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity.

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[1], v_TimingInfo);

//@siclog "Step 7" siclog@

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

f_NR_PDCP_Recv(p_NR_PDCP_State,p_PSCell,v_SduList[1], cr_RlcBearerRouting_NR(p_PSCell));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

//@siclog "Step 8-8A" siclog@

//The SS transmits an NR RRCReconfiguration message to activate parameters for PdcpDuplication

if (v_IsENDC){ //@sic R5-206329 sic@

v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id,cs_NR_PDCP_Config_UlPath(infinity_, len18bits, len18bits,-,-,-,true))};

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList);

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig);

}

else { //@sic R5-206329 sic@

v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(infinity_, len18bits, len18bits,-,-,-,true))};

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList);

v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2);

//SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB.

f_NR_SendRRCReconfiguration (p_PSCell, -,-, v_V1530Ext);

}

//@siclog "Step 8AA" siclog@

//@sic R5-213461 sic@

//The SS is configured not to send RLC ACK for the next PDU on split DRB

f_NR_SS_ConfigRB_NotACK_NextRLC_PDU(p_PSCell, cs_NR_RadioBearerId_DRB(tsc_NR_DRB2));

//@siclog "Step 8B" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[2], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //@sic R5s210376 sic@

//@siclog "Step 8C-8D" siclog@

interleave {

//Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary{} //@sic R5s210376 sic@

//Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , v_RlcBearerRoutingPcell, -, {v_ExpectedPdu})) -> value v_Secondary{} //@sic R5s210376 sic@

}

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[2],"Step 8C");

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[2],"Step 8D");

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

//@sic R5-213461 sic@

//EXCEPTION: Steps 8E-8F below may occur in any sequence. It is possible that only one of steps 8E or 8F occur, wait for 5 sec to allow these steps to take place.

//@siclog "Step 8E-8F" siclog@

//Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity?

//Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity?

v_PduRlcPrimary:= false;

v_PduRlcSecondary:= false;

/* REMOVED Steps 8E/8F

t_WaitForPdu.start;

alt {

//Does UE transmit a PDCP Data PDU on on the AM RLC primary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{

if(not(v_PduRlcPrimary))

{

v_PduRlcPrimary:= true;

if(not(v_PduRlcSecondary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F: Second Retransmisson for PDCP PDU in the AM RLC primary entity");

}

}

//Does UE transmit a PDCP Data PDU on on the AM RLC secondary entity?

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id ,v_RlcBearerRoutingPcell , -, {v_ExpectedPdu})) -> value v_Secondary

{

if(not(v_PduRlcSecondary))

{

v_PduRlcSecondary:= true;

if(not(v_PduRlcPrimary)) {

repeat;

}

}

else{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F: Second Retransmisson for PDCP PDU in the AM RLC secondary entity");

}

}

[] t_WaitForPdu.timeout

{

if((not(v_PduRlcPrimary))and(not(v_PduRlcSecondary)))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8E-8F");

}

}

}; */

//@siclog "Step 9" siclog@

//The SS stops allocating any UL grant for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s210376 sic@

}

//@siclog "Step 10" siclog@

//The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity..

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PSCell);

f_NR_PDCP_Send(p_NR_PDCP_State,p_PSCell,v_SduList[3], v_TimingInfo);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 11");

t_WaitForPdu.stop; //@sic R5s210376 sic@

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 PDCP DATA PDU not received");

}

}

//@siclog "Step 12" siclog@

//The SS resumes normal UL grant allocation for the AM RLC secondary entity. @sic R5-204429 sic@

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

//@siclog "Step 13" siclog@

// Does UE transmits a PDCP Data PDU on the AM RLC primary or secondary entity  in next five seconds?

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , cr_RlcBearerRouting_NR(p_PSCell), -, {v_ExpectedPdu})) -> value v_Primary

{ //UE transmits a PDCP Data PDU on the AM RLC primary entity.

f_CheckReceivedPdcpSdu(v_Primary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

//@siclog "Step 11-12" siclog@

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PSCell, p_NR_PDCP_State.DRB_Id , v_RlcBearerRoutingPcell, -, {v_ExpectedPdu})) -> value v_Secondary

{ //UE transmits a PDCP Data PDU on the AM RLC secondary entity.

f_CheckReceivedPdcpSdu(v_Secondary,v_SduList[3],"Step 13");

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[] t_WaitForPdu.timeout{

//Do nothing

}

}

//Coordination message to trigger the EUTRA Postamble

if (v_IsENDC)

{

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

}

# Branches executed

The test case was executed on EN-DC band combination 3_n78 with integrity algorithms eia2 and nia2 and ciphering algorithms eea2 and nea2.

# Execution Log File

## MediaTek MT6885

The MediaTek MT6885 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_3_5_5_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_3_5_5_PIXIT.html

# References

| [1] | R5s210783 : EN-DC FR1 : Supporting information for re-verification of NR PDCP test case 7.1.3.5.5 |
| --- | --- |