3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210784

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4629 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC test case TC 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Annex C.1 from 36.331, UE is allowed to signal different values for FDD &TDD on FGI bits 103 & 113. But ttcn is not considering this and validating the corresponding pics parameter against wrong variable. v_FGIsFromPICS_r10 is the and(bitwise) version of v_FGIsFromPICS_r10_FDD & v_FGIsFromPICS_r10_TDD. Using this variable to compare against FGI bits which can have different values for FDD & TDD doesn’t serve the spec. requirement. Consider UE reports FGI 113 only for FDD but not TDD and pc_UL_CA_2Carriers=TRUE. The TC will fail this check because v_FGIsFromPICS_r10[12]=0 (v_FGIsFromPICS_r10_FDD[12]=1 and4b v_FGIsFromPICS_r10_TDD[12]=0). It is the same case for FGI 103. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FGI bits 103 & 113 are explicitly checked against FDD & TDD parameters. Also removed similar checks under FDD & TDD specific if conditions, which will be redundant after the proposed change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TC 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_EUTRA_CheckRel10_FGIs |
| --- | --- |
| Reason for change | According to Annex C.1 from 36.331, UE is allowed to signal different values for FDD &TDD on FGI bits 103 & 113. But ttcn is not considering this and validating the corresponding pics parameter against wrong variable. v_FGIsFromPICS_r10 is the and(bitwise) version of v_FGIsFromPICS_r10_FDD & v_FGIsFromPICS_r10_TDD. Using this variable to compare against FGI bits which can have different values for FDD & TDD doesn’t serve the spec. requirement. Consider UE reports FGI 113 only for FDD but not TDD and pc_UL_CA_2Carriers=TRUE. The TC will fail this check because v_FGIsFromPICS_r10[12]=0 (v_FGIsFromPICS_r10_FDD[12]=1 and4b v_FGIsFromPICS_r10_TDD[12]=0). It is the same case for FGI 103. |
| Summary of change | FGI bits 103, 113 are explicitly checked against FDD & TDD parameters. Also removed similar checks under FDD & TDD specific if conditions, which will be redundant after the proposed change. |
| TTCN module | LTE\8_5\ RRC_OtherFunctions.ttcn |
| MCC160 Comment | Accepted in principle. TF160 implementation see below |

Before change:

function fl_EUTRA_CheckRel10_FGIs(UE_EUTRA_Capability p_ReceivedCapabilityMsg,

boolean p_Rel15 := false)

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg;

var UE_EUTRA_Capability_v1020_IEs v_UE_EUTRA_Capability_v1020_IEs;

var UE_EUTRA_Capability_v1060_IEs v_UE_EUTRA_Capability_v1060_IEs;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddFDD_Mode_v1060;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddTDD_Mode_v1060;

var B32_Type v_FGIsRcvd;

var B32_Type v_FGIsFromPICS_r10 := f_DeriveFeatrGrpCapFromPics_r10();

var B32_Type v_FGIsFromPICS_r10_FDD := f_DeriveFeatrGrpCapFromPics_r10_FDD();  // @sic R5s150521 sic@

var B32_Type v_FGIsFromPICS_r10_TDD := f_DeriveFeatrGrpCapFromPics_r10_TDD();  // @sic R5s150521 sic@

var boolean v_UE_CatsForFGI_103_104 := pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14; // @sic R5s196979 sic@

if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1020_IEs := v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension;

}

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1060_IEs := v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension;  // @sic R5-144330 sic@

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddFDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060;

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddTDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060;

}

// check FGIs 101-116 if present

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs)) {

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_Capability_v1020_IEs.featureGroupIndRel10_r10;

if (v_FGIsFromPICS_r10 != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10", 101, v_FGIsFromPICS_r10, v_FGIsRcvd);  // @sic R5-132034 sic@

}

// check Setting of FGI 113 against pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2):

// IF FGI 113 is set to '1' THEN pc_UL_CA_2Carriers shall be set to TRUE

if (((v_FGIsFromPICS_r10[12]) == '1'B) and not pc_UL_CA_2Carriers){

f_SetVerdict(fail, __FILE__, __LINE__, " FGI 113 set to '1' and pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2) is set to false "); // @sic R5-171568, R5-172901, R5-173043, R5s170573, R5-198842 sic@

}

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1'

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[2] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[3] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// FGIs not reported: error in Rel9, all 1s in future releases assumed  (36.331 Annex B1)

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10 capabilities are not reported");  // @sic R5-133612 sic@

}

//check RAT specific FGIs 101-116 FDD and TDD

if (pc_eFDD and pc_eTDD) {  // @sic R5s150637 sic@

// none, one or 2 sets of FGIs may be provided

// check FGIs 101-116 FDD  if present

if (isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@ @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_FDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_FDD", 101, v_FGIsFromPICS_r10_FDD, v_FGIsRcvd);

}

// check Setting of FGI 113 against pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2):

// IF FGI 113 is set to '1' THEN pc_UL_CA_2Carriers shall be set to TRUE

if (((v_FGIsFromPICS_r10_FDD[12]) == '1'B) and not pc_UL_CA_2Carriers){

f_SetVerdict(fail, __FILE__, __LINE__, " FGI 113 set to '1' and pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2) is set to false "); // @sic R5-171568, R5-172901, R5-173043, R5s170573, R5-198842, R5s201315 sic@

}

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1'

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[2] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[3] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// if FDD specific FGIs are not reported the FDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_FDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101F cc. not consistent with common values", 101, v_FGIsFromPICS_r10_FDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

// check FGIs 101-116 TDD if present

if (isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@  @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_TDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_TDD", 101, v_FGIsFromPICS_r10_TDD, v_FGIsRcvd);

}

// check Setting of FGI 113 against pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2):

// IF FGI 113 is set to '1' THEN pc_UL_CA_2Carriers shall be set to TRUE

if (((v_FGIsFromPICS_r10_TDD[12]) == '1'B) and not pc_UL_CA_2Carriers){

f_SetVerdict(fail, __FILE__, __LINE__, " FGI 113 set to '1' and pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2) is set to false "); // @sic R5-171568, R5-172901, R5-173043, R5s170573, R5s170758, R5-198842, R5s201315 sic@

}

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1'

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[2] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[3] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// if TDD specific FGIs are not reported the TDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_TDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101T cc. not consistent with common values", 101, v_FGIsFromPICS_r10_TDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

}

else { // in this case only 1 RAT is active and no RAT specific FGIs should be reported

if ((isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) or ((isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)))) { // @sic R5-144330 sic@  @sic R5s150521 sic@ @sic R5s150637 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10_FDD or TDD capabilities are reported, but should not as only 1 RAT is supported");  // @sic R5-133612 sic@

}

}

}

After change:

function fl_EUTRA_CheckRel10_FGIs(UE_EUTRA_Capability p_ReceivedCapabilityMsg,

boolean p_Rel15 := false)

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg;

var UE_EUTRA_Capability_v1020_IEs v_UE_EUTRA_Capability_v1020_IEs;

var UE_EUTRA_Capability_v1060_IEs v_UE_EUTRA_Capability_v1060_IEs;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddFDD_Mode_v1060;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddTDD_Mode_v1060;

var B32_Type v_FGIsRcvd;

var B32_Type v_FGIsFromPICS_r10 := f_DeriveFeatrGrpCapFromPics_r10();

var B32_Type v_FGIsFromPICS_r10_FDD := f_DeriveFeatrGrpCapFromPics_r10_FDD();  // @sic R5s150521 sic@

var B32_Type v_FGIsFromPICS_r10_TDD := f_DeriveFeatrGrpCapFromPics_r10_TDD();  // @sic R5s150521 sic@

var boolean v_UE_CatsForFGI_103_104 := pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14; // @sic R5s196979 sic@

if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1020_IEs := v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension;

}

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1060_IEs := v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension;  // @sic R5-144330 sic@

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddFDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060;

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddTDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060;

}

// check FGIs 101-116 if present

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs)) {

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_Capability_v1020_IEs.featureGroupIndRel10_r10;

if (v_FGIsFromPICS_r10 != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10", 101, v_FGIsFromPICS_r10, v_FGIsRcvd);  // @sic R5-132034 sic@

}

// check Setting of FGI 113 against pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2):

// IF FGI 113 is set to '1' THEN pc_UL_CA_2Carriers shall be set to TRUE

if (((v_FGIsFromPICS_r10_FDD[12] == '1'B) or (v_FGIsFromPICS_r10_TDD[12] == '1'B)) and not pc_UL_CA_2Carriers){

f_SetVerdict(fail, __FILE__, __LINE__, " FGI 113 set to '1' and pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2) is set to false "); // @sic R5-171568, R5-172901, R5-173043, R5s170573, R5-198842 sic@

}

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1'

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10_FDD[2] != '1'B) and (v_FGIsFromPICS_r10_TDD[2] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[3] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// FGIs not reported: error in Rel9, all 1s in future releases assumed  (36.331 Annex B1)

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10 capabilities are not reported");  // @sic R5-133612 sic@

}

//check RAT specific FGIs 101-116 FDD and TDD

if (pc_eFDD and pc_eTDD) {  // @sic R5s150637 sic@

// none, one or 2 sets of FGIs may be provided

// check FGIs 101-116 FDD  if present

if (isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@ @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_FDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_FDD", 101, v_FGIsFromPICS_r10_FDD, v_FGIsRcvd);

}

// Removed code, since FDD and TDD relevant checks for FGI 113 & 103 are added above

// check for FGI 104 is redundant here

}

else {

// if FDD specific FGIs are not reported the FDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_FDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101F cc. not consistent with common values", 101, v_FGIsFromPICS_r10_FDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

// check FGIs 101-116 TDD if present

if (isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@  @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_TDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_TDD", 101, v_FGIsFromPICS_r10_TDD, v_FGIsRcvd);

}

// Removed code, since FDD and TDD relevant checks for FGI 113 & 103 are added above

// check for FGI 104 is redundant here

}

else {

// if TDD specific FGIs are not reported the TDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_TDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101T cc. not consistent with common values", 101, v_FGIsFromPICS_r10_TDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

}

else { // in this case only 1 RAT is active and no RAT specific FGIs should be reported

if ((isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) or ((isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)))) { // @sic R5-144330 sic@  @sic R5s150521 sic@ @sic R5s150637 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10_FDD or TDD capabilities are reported, but should not as only 1 RAT is supported");  // @sic R5-133612 sic@

}

}

}

TF160 implementation:

function fl_EUTRA_CheckRel10_FGIs(UE_EUTRA_Capability p_ReceivedCapabilityMsg,

boolean p_Rel15 := false)

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg;

var UE_EUTRA_Capability_v1020_IEs v_UE_EUTRA_Capability_v1020_IEs;

var UE_EUTRA_Capability_v1060_IEs v_UE_EUTRA_Capability_v1060_IEs;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddFDD_Mode_v1060;

var UE_EUTRA_CapabilityAddXDD_Mode_v1060 v_UE_EUTRA_CapabilityAddTDD_Mode_v1060;

var B32_Type v_FGIsRcvd;

var B32_Type v_FGIsFromPICS_r10 := f_DeriveFeatrGrpCapFromPics_r10();

var B32_Type v_FGIsFromPICS_r10_FDD := f_DeriveFeatrGrpCapFromPics_r10_FDD();  // @sic R5s150521 sic@

var B32_Type v_FGIsFromPICS_r10_TDD := f_DeriveFeatrGrpCapFromPics_r10_TDD();  // @sic R5s150521 sic@

var boolean v_UE_CatsForFGI_103_104 := pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14; // @sic R5s196979 sic@

if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1020_IEs := v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension;

}

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension)) { // @sic R5s141016 sic@

v_UE_EUTRA_Capability_v1060_IEs := v_UE_EUTRA_Capability_v1020_IEs.nonCriticalExtension;  // @sic R5-144330 sic@

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddFDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.fdd_Add_UE_EUTRA_Capabilities_v1060;

}

if (isvalue (v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060)) { // @sic R5s141016 sic@

v_UE_EUTRA_CapabilityAddTDD_Mode_v1060 := v_UE_EUTRA_Capability_v1060_IEs.tdd_Add_UE_EUTRA_Capabilities_v1060;

}

// check FGIs 101-116 if present

if (isvalue (v_UE_EUTRA_Capability_v1020_IEs)) {

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_Capability_v1020_IEs.featureGroupIndRel10_r10;

if (v_FGIsFromPICS_r10 != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10", 101, v_FGIsFromPICS_r10, v_FGIsRcvd);  // @sic R5-132034 sic@

}

// check Setting of FGI 113 against pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2):

// IF FGI 113 is set to '1' in one RAT at least THEN pc_UL_CA_2Carriers shall be set to TRUE

// It is assumed that the common value will always be one of the RAT specific values !!!

// E.g. common value of '1' and RAT specific values of '0' will never occur

// Otherwise FGI reporting and FGI PICS setting would both be consistently incorrect !!!      if (((v_FGIsFromPICS_r10_FDD[12] == '1'B) or (v_FGIsFromPICS_r10_TDD[12] == '1'B)) and not pc_UL_CA_2Carriers){

f_SetVerdict(fail, __FILE__, __LINE__, " FGI 113 set to '1' and pc_UL_CA_2Carriers (36.523-2 Table A.4.3.3-2) is set to false "); // @sic R5-171568, R5-172901, R5-173043, R5s170573, R5-198842 sic@

}

if (p_Rel15) {

// check Setting of FGI 103 and FGI 104 against setting of UE categories

// IF (pc_ue_Category_8 or pc_ue_Category_11 or pc_ue_Category_12 or pc_ue_CategoryDL_11 or pc_ue_CategoryDL_12 or pc_ue_CategoryDL_14) THEN  FGI 103 resp 104 shall be set to '1' in one RAT at least

// It is assumed that the common value will always be one of the RAT specific values !!!

// E.g. common value of '1' and RAT specific values of '0' will never occur

// Otherwise FGI reporting and FGI PICS setting would both be consistently incorrect !!!              if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10_FDD[2] != '1'B) and (v_FGIsFromPICS_r10_TDD[2] != '1'B)){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 103 not set to '1' "); // @sic R5-196979 sic@

}

if (v_UE_CatsForFGI_103_104 and (v_FGIsFromPICS_r10[3] != '1'B) and (v_FGIsFromPICS_r10_FDD[3] != '1'B) and (v_FGIsFromPICS_r10_TDD[3] != '1'B){ // @sic R5s200875 sic@

f_SetVerdict(fail, __FILE__, __LINE__, " v_UE_CatsForFGI_103_104 TRUE and FGI 104 not set to '1' "); // @sic R5-196979 sic@

}

}

}

else {

// FGIs not reported: error in Rel9, all 1s in future releases assumed  (36.331 Annex B1)

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10 capabilities are not reported");  // @sic R5-133612 sic@

}

//check RAT specific FGIs 101-116 FDD and TDD

if (pc_eFDD and pc_eTDD) {  // @sic R5s150637 sic@

// none, one or 2 sets of FGIs may be provided

// check FGIs 101-116 FDD  if present

if (isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@ @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_FDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_FDD", 101, v_FGIsFromPICS_r10_FDD, v_FGIsRcvd);

}

// Removed code, since FDD and TDD relevant checks for FGI 113 & 103 are added above

// check for FGI 104 is redundant here

}

else {

// if FDD specific FGIs are not reported the FDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_FDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101F cc. not consistent with common values", 101, v_FGIsFromPICS_r10_FDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

// check FGIs 101-116 TDD if present

if (isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)) { // @sic R5-144330 sic@  @sic R5s150521 sic@

// check match between PICS settings and received value

v_FGIsRcvd := v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060;

if (v_FGIsFromPICS_r10_TDD != v_FGIsRcvd) {

fl_FGI_CheckFailure ("featureGroupIndRel10_r10_TDD", 101, v_FGIsFromPICS_r10_TDD, v_FGIsRcvd);

}

// Removed code, since FDD and TDD relevant checks for FGI 113 & 103 are added above

// check for FGI 104 is redundant here

}

else {

// if TDD specific FGIs are not reported the TDD PICS setting must be as per the common values

if (v_FGIsFromPICS_r10_TDD != v_FGIsFromPICS_r10) { // @sic R5s150521 sic@

fl_FGI_CheckFailure ("FGI PICS 101T cc. not consistent with common values", 101, v_FGIsFromPICS_r10_TDD, v_FGIsFromPICS_r10); // @sic R5-133715 sic@

}

}

}

else { // in this case only 1 RAT is active and no RAT specific FGIs should be reported

if ((isvalue (v_UE_EUTRA_CapabilityAddFDD_Mode_v1060.featureGroupIndRel10_v1060)) or ((isvalue (v_UE_EUTRA_CapabilityAddTDD_Mode_v1060.featureGroupIndRel10_v1060)))) { // @sic R5-144330 sic@  @sic R5s150521 sic@ @sic R5s150637 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "featureGroupIndRel10_r10_FDD or TDD capabilities are reported, but should not as only 1 RAT is supported");  // @sic R5-133612 sic@

}

}

}