3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210787

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1831 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-14-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some issues introduced during the implemnentation of R5-213460 for test case 7.1.3.5.1: - In the prose of this test case note 1 states : Note 1 PDCP Data PDU contents are checked to verify that the UL PDU is same as the DL PDU. According to the Note in TS 38.323 [19] clause 5.3 in case of PDCP SDUs being discarded it is up to the UE implementation which SN to be used and therefore the SN cannot be checked. The implementation of R5-213460 is not inline with this – the variables v_ExpectedPdu1 and v_ExpectedPdu2 are created with SN set to the same as the matching DL PDUs, but UE may not take into account the discarded PDUs when counting UL SN. The current implementation of v_ExpectedPdu3 allows either v_ExpectedPdu1 or v_ExpectedPdu2 to be received, but does not allow for a list containing both ( “()” brackets are used instead of “{}”) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When generating v_ExpectedPdu1 and v_ExpectedPdu2, use f_NR_PDC_PDU_Get() with no sequence number specified. Then set the expected SDU data from elements 4 and 5 of v_SduList[] (similar to previous implementation in wk12) - Redefine v_ExpectedPdu1 and v_ExpectedPdu2 to be single PDUs (NR_PDCP_PDU_Type). Then combine them in v_ExpectedPdu3 to be allow both entries in the NR_PDCP_PDUList_Type, in either order. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.1.ENDC, 7.1.3.5.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_5_1_NR_TestBody() |
| --- | --- |
| Reason for change | Some issues introduced during the implemnentation of R5-213460 for test case 7.1.3.5.1: - In the prose of this test case note 1 states : Note 1 PDCP Data PDU contents are checked to verify that the UL PDU is same as the DL PDU. According to the Note in TS 38.323 [19] clause 5.3 in case of PDCP SDUs being discarded it is up to the UE implementation which SN to be used and therefore the SN cannot be checked. The implementation of R5-213460 is not inline with this – the variables v_ExpectedPdu1 and v_ExpectedPdu2 are created with SN set to the same as the matching DL PDUs, but UE may not take into account the discarded PDUs when counting UL SN. - The current implementation of v_ExpectedPdu3 allows either v_ExpectedPdu1 or v_ExpectedPdu2 to be received, but does not allow for a list containing both ( “()” brackets are used instead of “{}”) |
| Summary of change | - When generating v_ExpectedPdu1 and v_ExpectedPdu2, use f_NR_PDC_PDU_Get() with no sequence number specified. Then set the expected SDU data from elements 4 and 5 of v_SduList[] (similar to previous implementation in wk12) - Redefine v_ExpectedPdu1 and v_ExpectedPdu2 to be single PDUs (NR_PDCP_PDU_Type). Then combine them in v_ExpectedPdu3 to be allow both entries in the NR_PDCP_PDUList_Type, in either order. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | - Accepted in principle but implemented differently. - Accepted partially: Accepted: Redefine v_ExpectedPdu1 and v_ExpectedPdu2 to be single PDU Rejected: combine them in v_ExpectedPdu3 to be allow both entries in the NR_PDCP_PDUList_Type, in either order. This is not per prose |

Before Change

function f_TC_7_1_3_5_1_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfok; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var float v_T_reordering; var integer k; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Stop UL Grants f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 1" siclog@ All PDCP PDUs use same SDU v_IPData v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 78, 94, 110}); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); //@siclog "Step 3" siclog@ for (k:= 0; k <= 2; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[k], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 20*(k+2)); } //@siclog "Step 4" siclog@ Wait Discard timer to expire. @sic R5s190225, R5s190770 sic@ v_T_reordering := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, 500.0); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfok, float2int(v_T_reordering) - 20); //@siclog "Step 5" SS transmits a PDCP Data PDU containing one PDCP SDU @sic R5s190225 sic@ v_TimingInfok := v_TimingInfo2; for (k:= 0; k <= 1; k:= k+1) { f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[k+3], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,20*(k+1)); } v_TimingPreviousSlot:= f_NR_GetCurrentTiming(nr_Cell1); //@sic R5-213462 sic@ //@siclog "Step 6" The SS resumes normal UL grant allocation. v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfok, 0); // @sic R5s190590 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL)); //@siclog "Step 7" siclog@ Does UE transmit a PDCP Data PDU # 5 of size 94 bytes? (Note 2) //@sic R5-213462 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= (v_ExpectedPdu1,v_ExpectedPdu2); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } }

After Change

function f_TC_7_1_3_5_1_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfok; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var float v_T_reordering; var integer k; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Stop UL Grants f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 1" siclog@ All PDCP PDUs use same SDU v_IPData v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 78, 94, 110}); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); //@siclog "Step 3" siclog@ for (k:= 0; k <= 2; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[k], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 20*(k+2)); } //@siclog "Step 4" siclog@ Wait Discard timer to expire. @sic R5s190225, R5s190770 sic@ v_T_reordering := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, 500.0); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfok, float2int(v_T_reordering) - 20); //@siclog "Step 5" SS transmits a PDCP Data PDU containing one PDCP SDU @sic R5s190225 sic@ v_TimingInfok := v_TimingInfo2; for (k:= 0; k <= 1; k:= k+1) { f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[k+3], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,20*(k+1)); } v_TimingPreviousSlot:= f_NR_GetCurrentTiming(nr_Cell1); //@sic R5-213462 sic@ //@siclog "Step 6" The SS resumes normal UL grant allocation. v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfok, 0); // @sic R5s190590 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL)); //@siclog "Step 7" siclog@ Does UE transmit a PDCP Data PDU # 5 of size 94 bytes? (Note 2) //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size); //REMOVED f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu1.DataPduSN18Bits.SDU := v_SduList[3]; v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; //REMOVED f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2.DataPduSN18Bits.SDU := v_SduList[4]; v_ExpectedPdu3:= ({v_ExpectedPdu1,v_ExpectedPdu2}, { v_ExpectedPdu2,v_ExpectedPdu1}); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } }

MCC160 Implementation

template (present) NR_PDCP_PDU_Type cr_NR_PDCP_PDU_12B(template (present) B12_Type p_PDCP_SN_12B,

template (present) NR_PDCP_SDU_Type p_NR_PDCP_SDU := ?) := // @sic R5s210787 sic@

{  /* @status    APPROVED (ENDC, NR5GC) */

DataPduSN12Bits := {

D_C             := tsc_NR_PDCP_D_C_Data,

Reserved        := tsc_B3_Reserved,

SequenceNumber  := p_PDCP_SN_12B,

SDU             := p_NR_PDCP_SDU,

MAC_I           := *

}

};

template (present) NR_PDCP_PDU_Type cr_NR_PDCP_PDU_18B(template (present) B18_Type p_PDCP_SN_18B,

template (present) NR_PDCP_SDU_Type p_NR_PDCP_SDU := ?) := // @sic R5s210787 sic@

{  /* @status    APPROVED (ENDC, NR5GC) */

DataPduSN18Bits := {

D_C             := tsc_NR_PDCP_D_C_Data,

Reserved        := tsc_B5_Reserved,

SequenceNumber  := p_PDCP_SN_18B,

SDU             := p_NR_PDCP_SDU,

MAC_I           := *

}

};

function f_NR_PDCP_PDU_Get (NR_PDCP_SN_Size_Type p_NR_PDCP_SN_Size,

template(omit) integer p_PDCP_RX_SN := omit,

template (present) NR_PDCP_SDU_Type p_NR_PDCP_SDU := ?) return template (present) NR_PDCP_PDU_Type

{

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

select (p_NR_PDCP_SN_Size) {

case (PDCP_SNLength12) {

if (isvalue(p_PDCP_RX_SN)) {

v_ExpectedPdu := cr_NR_PDCP_PDU_12B(int2bit(valueof(p_PDCP_RX_SN),12),p_NR_PDCP_SDU); //@sic R5s210787 sic@

} else {

v_ExpectedPdu := cr_NR_PDCP_PDU_12B(?, p_NR_PDCP_SDU); //@sic R5s190203 R5s210787 sic@

}

}

case (PDCP_SNLength18) {

if (isvalue(p_PDCP_RX_SN)) {

v_ExpectedPdu := cr_NR_PDCP_PDU_18B(int2bit(valueof(p_PDCP_RX_SN),18),p_NR_PDCP_SDU); //@sic R5s210787 sic@

}else {

v_ExpectedPdu := cr_NR_PDCP_PDU_18B(?, p_NR_PDCP_SDU); //@sic R5s210787 sic@

}

}

case else {

FatalError(__FILE__, __LINE__, "NR_PDCP_SN_Size incorrect");

}

}

return v_ExpectedPdu;

}

function f_TC_7_1_3_5_1_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var SubFrameTiming_Type v_TimingInfo1;

var SubFrameTiming_Type v_TimingInfo2;

var SubFrameTiming_Type v_TimingInfok;

var SubFrameTiming_Type v_TimingUL;

var SubFrameTiming_Type v_TimingCurrentSlot;

var SubFrameTiming_Type v_TimingPreviousSlot;

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2;

var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3;

var NR_DRB_COMMON_IND v_ReceivedPdu1;

var NR_DRB_COMMON_IND v_ReceivedPdu2;

var NR_DRB_COMMON_IND v_ReceivedPdu3;

var float v_T_reordering;

var integer k;

p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc;

//Stop UL Grants

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 1" siclog@ All PDCP PDUs use same SDU v_IPData

v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 78, 94, 110});

…

//@siclog "Step 7" siclog@ Does UE transmit a PDCP Data PDU # 5 of size 94 bytes? (Note 2)

//@sic R5-213462 R5s210673 sic@

v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, -,v_SduList[3]); // @sic R5s210787 sic@

v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, -, v_SduList[4]); // @sic R5s210787 sic@

v_ExpectedPdu3:= {v_ExpectedPdu1,v_ExpectedPdu2};

alt {

// receive PDU1

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 //@sic R5s210673 sic@

{

v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame;

// receive PDU2

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; //@sic R5s210673 sic@

v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8");

}

// receive PDU1 and PDU2

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8");

}

}