3GPP TSG-RAN5 Meeting #2020-TTCN email	R5s210805

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1839 | rev | - | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Currently TC fails at Step 18, upon reception of the short message UE acquires SIB1 but in SIB1 'schedulingInfoList' valuetag for SIB4 and SIB2 is not modified therefore UE considers the old SIB4 and SIB2 as valid. It is necessary to modify the valuetag so that UE can read new SIB4 and SIB2 from next modification period. (Ref 38.331 5.2.2.2.1 SIB validity) - Test Steps 15-21 are not logged in TTCN |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Incremented value tag so that UE reads modified SIB4(Cell 1) and SIB2(Cell3) 2. Corrected TTCN steps logging syntax for Step 15 to Step 21 |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

2.2 Change 2	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

2.1 Change 1

| Function name | f_TC_8_1_1_2_1_NR5GC |
| --- | --- |
| Reason for change | Currently TC fails at Step 18, upon reception of the short message UE acquires SIB1 but in SIB1 'schedulingInfoList' valuetag for SIB4 and SIB2 is not modified therefore UE considers the old SIB4 and SIB2 as valid. It is necessary to modify the valuetag so that UE can read new SIB4 and SIB2 from next modification period. (Ref 38.331 5.2.2.2.1 SIB validity) |
| Summary of change | Incremented value tag so that UE reads modified SIB4(Cell 1) and SIB2(Cell3) |
| TTCN module | NR5GC\8_1_1\RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, subject to prose CR approval |

Before change:

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC

{

….

//@sic R5-213466 sic@

//@siclog "Step 18" siclog

//The SS changes NR Cell 3 to high priority in SIB4.

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7;

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]});

//@siclog "Step 18A" siclog

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7);

//@siclog "Step 19" siclog

//The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.

f_NR_ModifySysinfo(nr_Cell1);

f_NR_ModifySysinfo(nr_Cell3);

//@siclog "Step 20" siclog

//Wait for 2.1* modification period to allow the new system information to take effect.

f_NR_WaitModificationPeriods (nr_Cell1);

//@siclog "Step 21" siclog

//Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3?

f_NR5GC_CheckRRCIdle(nr_Cell3);

}…

After change:

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC

{

….

//@sic R5-213466 sic@

//@siclog "Step 18" siclog

//The SS changes NR Cell 3 to high priority in SIB4.

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7;

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]});

//@siclog "Step 18A" siclog

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7);

//@siclog "Step 19" siclog

//The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.

f_NR_IncrementValueTag(nr_Cell1, sibType4); //Anritsu TTCN CR for 8.1.1.2.1

f_NR_ModifySysinfo(nr_Cell1);

f_NR_IncrementValueTag(nr_Cell3, sibType2); //Anritsu TTCN CR for 8.1.1.2.1

f_NR_ModifySysinfo(nr_Cell3);

//@siclog "Step 20" siclog

//Wait for 2.1* modification period to allow the new system information to take effect.

f_NR_WaitModificationPeriods (nr_Cell1);

//@siclog "Step 21" siclog

//Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3?

f_NR5GC_CheckRRCIdle(nr_Cell3);

}…

2.2 Change 2

| Function name | fl_TC_8_1_1_2_1_TestBody |
| --- | --- |
| Reason for change | Test Steps 15-21 are not logged in TTCN |
| Summary of change | Corrected TTCN steps logging syntax for Step 15 to Step 21 |
| TTCN module | NR5GC\8_1_1\RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before change:

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Step 13" siclog@

//The SS transmits a Paging message including a matched ng-5G-S-TMSI on NR Cell 1.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

//@siclog "Step 14" siclog@

//Check: Does UE send RRCSetupRequest in 5 second?

f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, - , "Step 14");  //  @sic R5s201387 sic@

//@siclog "Step 15" siclog

//@sic R5-202619 sic@

//void

//@siclog "Step 16" siclog

//After expiry of connEstFailOffsetValidity, the SS waits 35s to allow UE to perform cell reselection.

f_Delay(35.0);//@sic R5-201216 sic@

//@siclog "Step 17" siclog

//Check: Does the test result of generic test procedue step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 1?

f_NR5GC_CheckRRCIdle(nr_Cell1);// @sic R5-198911 sic@

//@sic R5-213466 sic@

//@siclog "Step 18" siclog

//The SS changes NR Cell 3 to high priority in SIB4.

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7;

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]});

//@siclog "Step 18A" siclog

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7);

//@siclog "Step 19" siclog

//The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.

f_NR_ModifySysinfo(nr_Cell1);

f_NR_ModifySysinfo(nr_Cell3);

//@siclog "Step 20" siclog

//Wait for 2.1* modification period to allow the new system information to take effect.

f_NR_WaitModificationPeriods (nr_Cell1);

//@siclog "Step 21" siclog

//Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3?

f_NR5GC_CheckRRCIdle(nr_Cell3);

}

After change:

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Step 13" siclog@

//The SS transmits a Paging message including a matched ng-5G-S-TMSI on NR Cell 1.

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

//@siclog "Step 14" siclog@

//Check: Does UE send RRCSetupRequest in 5 second?

f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, - , "Step 14");  //  @sic R5s201387 sic@

//@siclog "Step 15" siclog@

//@sic R5-202619 sic@

//void

//@siclog "Step 16" siclog@

//After expiry of connEstFailOffsetValidity, the SS waits 35s to allow UE to perform cell reselection.

f_Delay(35.0);//@sic R5-201216 sic@

//@siclog "Step 17" siclog@

//Check: Does the test result of generic test procedue step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 1?

f_NR5GC_CheckRRCIdle(nr_Cell1);// @sic R5-198911 sic@

//@sic R5-213466 sic@

//@siclog "Step 18" siclog@

//The SS changes NR Cell 3 to high priority in SIB4.

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7;

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]});

//@siclog "Step 18A" siclog@

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7);

//@siclog "Step 19" siclog@

//The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification.

f_NR_ModifySysinfo(nr_Cell1);

f_NR_ModifySysinfo(nr_Cell3);

//@siclog "Step 20" siclog@

//Wait for 2.1* modification period to allow the new system information to take effect.

f_NR_WaitModificationPeriods (nr_Cell1);

//@siclog "Step 21" siclog@

//Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3?

f_NR5GC_CheckRRCIdle(nr_Cell3);

}