3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210806

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1840 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.2 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1, 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	9

4	Execution Log Files	10

4.1	HiSilicon Balong 5000	10

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_4_2_NR5GC |
| --- | --- |
| Reason for change | - The UE will try to camp on NR CELL1 after step 2 because the PLMN change does not take effect - After the emergency call is released, the exact behaviour of the UE is unspecified, as UE may or may not perform an implicit deregistration. |
| Summary of change | - modify Sysinfo and wait for some time before power on UE at step2 - perform postamble in new STATE_REGISTERED_DEREGISTERED_IDLE Note : A prose CR associated with change 2 will be raised at RAN5#92. |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment | - Rejected. The UE is turned off; so although there is an error in the TTCN - we do need to ensure new sys info is sent , we don't need to page the UE, or wait. - Accepted in princeple, but implemented as per agreed prose CR R5-214620.Implemented rightly as: f_NR_Postamble(nr_Cell1, STATE_EMERGENCY_RELEASE_IDLE); |

Before Change

function f_TC_11_4_2_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-15. f_UT_USIM_Insert(UT, "USIM configuration 15 specified in table 6.4.1-15 TS 38.508-1"); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_TestBody_Set(true); fl_TC_11_4_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_11_4_2_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-15. f_UT_USIM_Insert(UT, "USIM configuration 15 specified in table 6.4.1-15 TS 38.508-1"); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_ModifySysinfo(nr_Cell1, false); f_NR_WaitModificationPeriods(nr_Cell1); f_NR_TestBody_Set(true); fl_TC_11_4_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_REGISTERED_DEREGISTERED_IDLE); }

MCC160 proposed implementation

function f_TC_11_4_2_NR5GC() runs on NR5GC_PTC { var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-15. f_UT_USIM_Insert(UT, "USIM configuration 15 specified in table 6.4.1-15 TS 38.508-1"); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. Prior to being switched off the UE is registered on PLMN1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); f_NR_TestBody_Set(true); fl_TC_11_4_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_EMERGENCY_RELEASE_IDLE); }

## Change 2

| Function name | fl_TC_11_4_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | After the emergency call is released, the exact behaviour of the UE is unspecified, as UE may or may not perform an implicit deregistration. |
| Summary of change | Remove steps 8b2/8b3, to be handled with new postamble sequences. Note : Prose CRs for 38.523-1 and 38.508-1 will be raised at RAN5#92. |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_11_4_2_NR5GC_TestBody() runs on NR5GC_PTC { timer t_Wait := 10.0; //@siclog "Step 1" siclog@ //The SS configures:- "NR Cell 1 as "Serving cell". f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ //Switch the UE on. f_UT_SwitchOnUE(UT); //@siclog "Step 3" siclog@ //Make the UE attempt an IMS emergency call dialling a number which is stored on the ME. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 4" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? f_NR5GC_EmergencyCallLimitedService(nr_Cell1); //@siclog "Step 5-6" siclog@ //Make the UE release the emergency call. f_NR5GC_EmergencyCallRelease(nr_Cell1); //@siclog "Step 7" siclog@ //Start Timer. t_Wait.start; //@siclog "Step 8a1-8b3" siclog@ //EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 8a1" siclog@ //The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) { //@siclog "Step 8a2" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); //@siclog "Step 8a3" siclog@ //Stop Timer. t_Wait.stop; } //@siclog "Step 8b1" siclog@ //Timer expires. [] t_Wait.timeout { //@siclog "Step 8b2" siclog@ //The SS transmits a DEREGISTRATION REQUEST message with Deregistration type IE set to "re-registration required". SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_REQUEST_MT(crs_DeregisterType('0'B, '0'B,'01'B), cs_GMM_GSM_Cause (omit, '00001011'B), -,-, -)))); //@siclog "Step 8b3" siclog@ //The UE transmits a DEREGISTRATION ACCEPT message. SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))); } } //@siclog "Step 9" siclog@ //The SS releases RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 10" siclog@ //Check: Does the UE transmit an RRCSetupRequest message in the next 30sec? f_NR_CheckNoRRCSetupReq (nr_Cell1, 30.0); }

After Change

function fl_TC_11_4_2_NR5GC_TestBody() runs on NR5GC_PTC { timer t_Wait := 10.0; //@siclog "Step 1" siclog@ //The SS configures:- "NR Cell 1 as "Serving cell". f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ //Switch the UE on. f_UT_SwitchOnUE(UT); //@siclog "Step 3" siclog@ //Make the UE attempt an IMS emergency call dialling a number which is stored on the ME. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 4" siclog@ //Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? f_NR5GC_EmergencyCallLimitedService(nr_Cell1); //@siclog "Step 5-6" siclog@ //Make the UE release the emergency call. f_NR5GC_EmergencyCallRelease(nr_Cell1); //@siclog "Step 7" siclog@ //Start Timer. t_Wait.start; //@siclog "Step 8a1-8b3" siclog@ //EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 8a1" siclog@ //The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) { //@siclog "Step 8a2" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); //@siclog "Step 8a3" siclog@ //Stop Timer. t_Wait.stop; } //@siclog "Step 8b1" siclog@ //Timer expires. [] t_Wait.timeout { //@siclog "Step 8b2" siclog@ //The SS transmits a DEREGISTRATION REQUEST message with Deregistration type IE set to "re-registration required". //Void //@siclog "Step 8b3" siclog@ //The UE transmits a DEREGISTRATION ACCEPT message. //Void } }

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_2_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_2_PIXIT.xml

# References

| [1] | R5s210807 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.2 |
| --- | --- |