3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210810

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1842 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.8 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.8 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	10

3	Branches executed	11

4	Execution Log Files	11

4.1	HiSilicon Balong 5000	11

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.8 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.8.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_11_4_8_NR5GC |
| --- | --- |
| Reason for change | - Current guard timer of 300s is not sufficient for the test case to execute - Since IMS MO (non-emergency) call shall be established in this test case, the specific function for IMS1 PTC should be used. |
| Summary of change | - increase guard timer to 900s - Start case specific function for IMS1 PTC f_TC_11_4_8_NR5GC_IMS1() |
| TTCN module | NR5GC_Testsuite |
| MCC160 Comment | Accepted |

Before Change

testcase TC_11_4_8_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_8_NR5GC()); v_IMS2.start(f_TC_11_4_8_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change

testcase TC_11_4_8_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_8_NR5GC()); v_IMS1.start(f_TC_11_4_8_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_8_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 2

| Function name | fl_TC_11_4_8_NR5GC_TestBody |
| --- | --- |
| Reason for change | - UE is switch OFF/Power On at the end of preamble, as a result, UE shall do IMS de-register/register correspondingly. But current TTCN does not implement it - A run-time error occurs while callimng f_Build_ExtdEmergNumList because the input p_NoOfNums is not incorrect - T3512 value is included in Registration Accept at step 13, but according to 38-508 Table 4.7.1-7 it is not present except for INITIAL registration. - Registration procedure fail because IE guti and taiList are incorrect. They are gotten from NR Cell 1 - Some step logging is incorrecdt |
| Summary of change | - replace f_UT_SwitchOnUE with f_NR5GC_SwitchOnAndResetIMS - correct the input p_NoOfNums - modify the 1st input parameter of f_Get_NG_RegistrationAcceptMsg to Mobility because this is a Mobility registration procedure - get PLMN and TAI from NR Cell12 for step 13 - correct logging |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_11_4_8_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var TrackingAreaCode v_TAC_1 := f_NR_CellInfo_GetTAC (nr_Cell1); var NAS_PlmnId v_PLMN_1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_1); var template (value) NG_MobileIdentity v_GUTI_1 := f_NG_GutiParameters2MobileIdentity (v_PLMN_1, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1)); var template (value) NG_TrackingAreaIdList v_TAIList_1 := cs_NG_TAIListNonConsecutive(v_PLMN_1, {bit2oct(v_TAC_1)}); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var EmergencyNumList v_LocalNumList := {'120'H, '121'H, '122'H, '123'H, '124'H, '125'H, '126'H, '127'H, '128'H, '129'H}; var EmergencyNumList v_ExtdNumList := {'20'H, '21'H, '22'H, '23'H, '24'H, '25'H, '26'H, '27'H, '28'H, '29'H, '30'H, '31'H, '32'H, '33'H, '34'H, '35'H, '36'H, '37'H, '130'H, '4001'H}; var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 var integer i; //@siclog "Step 1" siclog@ f_UT_SwitchOnUE(UT); //@siclog "Step 2" siclog@ f_NR5GC_RRC_IdleState1N_Def (nr_Cell1); //@siclog "Steps 3 - 7" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 4"); //@siclog "Steps 8 - 12" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 9"); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 11" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (10, v_LocalNumList); // FFS Prose says 20 numbers: 10 ...37, 130, 4001 v_XtdEmergNumList := f_Build_ExtdEmergNumList (30, v_ExtdNumList, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell12, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 14 - 18" siclog@ for (i := 0; i < 10; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_LocalNumList[i]), "Step 15, i = " & int2str(i)); } //@siclog "Steps 19 - 23" siclog@ for (i := 0; i < 20; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_ExtdNumList[i]), "Step 20, i = " & int2str(i)); } }

After Change

function fl_TC_11_4_8_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_1 := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC_1 := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN_1 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_1); var template (value) NG_MobileIdentity v_GUTI_1 := f_NG_GutiParameters2MobileIdentity (v_PLMN_1, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1)); var template (value) NG_TrackingAreaIdList v_TAIList_1 := cs_NG_TAIListNonConsecutive(v_PLMN_1, {bit2oct(v_TAC_1)}); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var EmergencyNumList v_LocalNumList := {'120'H, '121'H, '122'H, '123'H, '124'H, '125'H, '126'H, '127'H, '128'H, '129'H}; var EmergencyNumList v_ExtdNumList := {'20'H, '21'H, '22'H, '23'H, '24'H, '25'H, '26'H, '27'H, '28'H, '29'H, '30'H, '31'H, '32'H, '33'H, '34'H, '35'H, '36'H, '37'H, '130'H, '4001'H}; var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 var integer i; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_NR5GC_RRC_IdleState1N_Def (nr_Cell1); //@siclog "Steps 3 - 7" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 4"); //@siclog "Steps 8 - 12" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 9"); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 13" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (10, v_LocalNumList); // FFS Prose says 20 numbers: 10 ...37, 130, 4001 v_XtdEmergNumList := f_Build_ExtdEmergNumList (20, v_ExtdNumList, '0'B); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTI_1, v_TAIList_1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell12, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 14 - 18" siclog@ for (i := 0; i < 10; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_LocalNumList[i]), "Step 15, i = " & int2str(i)); } //@siclog "Steps 19 - 23" siclog@ for (i := 0; i < 20; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_ExtdNumList[i]), "Step 20, i = " & int2str(i)); } }

## Change 3

| Function name | f_TC_11_4_8_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Same as change 2.1 |
| Summary of change | Implement the IMS de-register/register caused by the switch-off/Power on in middle of test case |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment | Accepted in principle |

Before Change

function f_TC_11_4_8_NR5GC_IMS1()runs on IMS_PTC { // Initial PDU session - for e.g Registration and 'normal' calls var integer v_MaxMOCalls := 2; //2 IMS MO calls to be initiated var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s210603 sic@ var integer i; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); for (i:=0; i< v_MaxMOCalls; i:=i+1) { fl_IMS_NR5GC_MOSpeechCall(v_OtherCalleeUri); } f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_4_8_NR5GC_IMS1()runs on IMS_PTC { // Initial PDU session - for e.g Registration and 'normal' calls var integer v_MaxMOCalls := 2; //2 IMS MO calls to be initiated var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s210603 sic@ var integer i; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Deregistration(); f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); for (i:=0; i< v_MaxMOCalls; i:=i+1) { fl_IMS_NR5GC_MOSpeechCall(v_OtherCalleeUri); } f_IMS_IpcanReleaseWithOptionalDeregistration(); }

MCC160 Implementation:

function f_TC_11_4_8_NR5GC_IMS1()runs on IMS_PTC

{ // Initial PDU session - for e.g Registration and 'normal' calls

var integer v_MaxMOCalls := 2; //2 IMS MO calls to be initiated

var template (present) SipUrl v_OtherCalleeUri := cr_SipUrl_Common((c_sipScheme, c_telScheme), ?); // @sic R5s210603 sic@

var integer i;

f_IMS_PTC_Init(PDN_1);

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IpcanReleaseWithOptionalDeregistration();

f_IMS_GlobalRelease();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

for (i:=0; i< v_MaxMOCalls; i:=i+1) {

fl_IMS_NR5GC_MOSpeechCallAndRelease(v_OtherCalleeUri);

}

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_8_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_8_PIXIT.xml

# References

| [1] | R5s210811 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.8 |
| --- | --- |