3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210820

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1846 | rev | - | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.5.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS38.523-1 9.1.5.1.10.3.1, The NGC cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in38.508-1 [4], except replacing NRf3 with NRf1,so NRf1 for NGC CellI should be used. In function f_NR5GC_Init, the logic in case NR5GC_NAS_ReplaceNRf2_NRf1 and NR5GC_NAS_ReplaceNRf3_NRf1 are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use NRf1 to replace NRf3 in case NR5GC_NAS_ReplaceNRf3_NRf1, use NRf1 to replace NRf2 in case NR5GC_NAS_ReplaceNRf2_NRf1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will pass using incorrect frequency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections requireds	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk24  related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections requireds

## Change 1

| Function name | f_NR5GC_Init |
| --- | --- |
| Reason for change | According to TS38.523-1 9.1.5.1.10.3.1, The NGC cells are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in38.508-1 [4], except replacing NRf3 with NRf1,so NRf1 for NGC CellI should be used. In function f_NR5GC_Init, the logic in case NR5GC_NAS_ReplaceNRf2_NRf1 and NR5GC_NAS_ReplaceNRf3_NRf1 are not correct. |
| Summary of change | Use NRf1 to replace NRf3 in case NR5GC_NAS_ReplaceNRf3_NRf1, use NRf1 to replace NRf2 in case NR5GC_NAS_ReplaceNRf2_NRf1 |
| TTCN module | NR5GC_CommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_NR5GC_Init(NR_SysinfoCombination_Type p_NR_SysinfoCombination,

NR5GC_NAS_CellSpecialFreq p_NAS_CellSpecialFreq := NR5GC_NAS_NormalFreq

) runs on NR5GC_PTC

{ //@sic R5-195681: added p_NAS_CellSpecialFreq sic@

var template(value) NR_Frequencyf1tof4_Type v_NR_PrimaryFrequencyf1Tof4;

var template(value) NR_Frequencyf1tof4_Type v_NR_SecondaryFrequencyf1tof4;

if (not f_GetTestcaseAttrib_HighestCarrierBandwidth(testcasename())) {

v_NR_PrimaryFrequencyf1Tof4 := f_NR_GetFrequencySA_f1tof4(px_NR_PrimaryBand);

v_NR_SecondaryFrequencyf1tof4 := f_NR_GetFrequencySA_f1tof4(px_NR_SecondaryBand);

} else { //@sic R5s190826/827 sic@

v_NR_PrimaryFrequencyf1Tof4 := f_NR_GetHighestBandwidthFrequency(px_NR_PrimaryBand);

v_NR_SecondaryFrequencyf1tof4 := f_NR_GetHighestBandwidthFrequency(px_NR_SecondaryBand);

}

select (p_NAS_CellSpecialFreq) {

case(NR5GC_NAS_ReplaceNRf2_NRf1) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f1 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f2;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f1 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f2;

}

case(NR5GC_NAS_ReplaceNRf3_NRf1) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f1 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f1 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3;

}

case(NR5GC_NAS_ReplaceNRf3_NRf2) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f2;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f2;

}

case(NR5GC_NAS_ReplaceNRf4_NRf3){

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f4 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f4 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3;

}

}

fl_NR_InitCommon( v_NR_PrimaryFrequencyf1Tof4, px_NR_PrimaryBand, v_NR_SecondaryFrequencyf1tof4, px_NR_SecondaryBand, p_NR_SysinfoCombination);

}

After change

function f_NR5GC_Init(NR_SysinfoCombination_Type p_NR_SysinfoCombination,

NR5GC_NAS_CellSpecialFreq p_NAS_CellSpecialFreq := NR5GC_NAS_NormalFreq

) runs on NR5GC_PTC

{ //@sic R5-195681: added p_NAS_CellSpecialFreq sic@

var template(value) NR_Frequencyf1tof4_Type v_NR_PrimaryFrequencyf1Tof4;

var template(value) NR_Frequencyf1tof4_Type v_NR_SecondaryFrequencyf1tof4;

if (not f_GetTestcaseAttrib_HighestCarrierBandwidth(testcasename())) {

v_NR_PrimaryFrequencyf1Tof4 := f_NR_GetFrequencySA_f1tof4(px_NR_PrimaryBand);

v_NR_SecondaryFrequencyf1tof4 := f_NR_GetFrequencySA_f1tof4(px_NR_SecondaryBand);

} else { //@sic R5s190826/827 sic@

v_NR_PrimaryFrequencyf1Tof4 := f_NR_GetHighestBandwidthFrequency(px_NR_PrimaryBand);

v_NR_SecondaryFrequencyf1tof4 := f_NR_GetHighestBandwidthFrequency(px_NR_SecondaryBand);

}

select (p_NAS_CellSpecialFreq) {

case(NR5GC_NAS_ReplaceNRf2_NRf1) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f2 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f1;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f2 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f1;

}

case(NR5GC_NAS_ReplaceNRf3_NRf1) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f1;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f1;

}

case(NR5GC_NAS_ReplaceNRf3_NRf2) {

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f2;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f2;

}

case(NR5GC_NAS_ReplaceNRf4_NRf3){

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f4 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoDL_f3;

v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f4 := v_NR_PrimaryFrequencyf1Tof4.FrequencyInfoUL_f3;

}

}

fl_NR_InitCommon( v_NR_PrimaryFrequencyf1Tof4, px_NR_PrimaryBand, v_NR_SecondaryFrequencyf1tof4, px_NR_SecondaryBand, p_NR_SysinfoCombination);

}