3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210838

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1855 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of 7.1.3.4.1 - Using the type NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot at the steps 15,30 and 31 - At step 15, UE loopback is expected on NR Cell 2 as per prose , but TTCN is expecting loop pack PDCP Data PDU on NR Cell 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the test case 7.1.3.4.1 , it is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs at steps 15,30 and 31 - At step 15 of the test case 7.1.3.4.1 , TTCN is corrected to receive the Looped back PDCP Data PDU on NR Cell2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.ENDC, 7.1.3.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_7_1_3_4_1_NR_Step10to23	2

1.2	Correction to function f_TC_7_1_3_4_1_NR_Step25to30	12

# Corrections required

## Correction to function f_TC_7_1_3_4_1_NR_Step10to23

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | In the current TTCN implementation of 7.1.3.4.1 - Using the type NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot at the steps 15 - At step 15, UE loopback is expected on NR Cell 2 as per prose , but TTCN is expecting loop pack PDCP Data PDU on NR Cell 1 |
| Summary of change | - For the test case 7.1.3.4.1 , it is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs at steps 15 - At step 15 of the test case 7.1.3.4.1 , TTCN is corrected to receive the Looped back PDCP Data PDU on NR Cell2 |
| TTCN module | NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | - Accepted. Implemented by R5s210719 - Accepted |

Before Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; // var TimingInfo_Type v_TimingInfo; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC - only one grant for PDCP STATUS REPORT f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } } else{ //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if ( p_WaitforRRCConCmp) { // if NR5GC @sic R5s210282 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell2); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); }; //@siclog "Step 12" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) //and PDU Type =000, FMC field = 3 DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@ //@siclog "Step 13" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell2); //@siclog "Step 15" siclog@ //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= (v_ExpectedPdu1, v_ExpectedPdu2); v_ExpectedPdu5:= (v_ExpectedPdu2, v_ExpectedPdu3); v_ExpectedPdu6:= (v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; } // receive PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3 { v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } //@siclog "Step 16" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData); //@siclog "Step 17" siclog@ //The UE sends the PDCP Data PDU#5 f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2)); //@siclog "Step 20" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#7 p_NR_PDCP_State.TX_NEXT:= 7; p_NR_PDCP_State.RX_NEXT:= 7; //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData); //@siclog "Step 21" siclog@ //Check: Does the UE transmit a PDCP DATA PDU#7 on NR Cell 2? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell2, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } [] t_Wait.timeout{} } //@siclog "Step 21A" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5-190752 sic@ //@siclog "Step 22" siclog@ //The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); }

After Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; // var TimingInfo_Type v_TimingInfo; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; //WA#WI=861440 var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; //WA#WI=861440 var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; //WA#WI=861440 var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC - only one grant for PDCP STATUS REPORT f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } } else{ //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if ( p_WaitforRRCConCmp) { // if NR5GC @sic R5s210282 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell2); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); }; //@siclog "Step 12" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) //and PDU Type =000, FMC field = 3 DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@ //@siclog "Step 13" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell2); //@siclog "Step 15" siclog@ //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //WA#WI=861440 f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //WA#WI=861440 f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //WA#WI=861440 f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= {v_ExpectedPdu1, v_ExpectedPdu2}; //WA#WI=861440 v_ExpectedPdu5:= {v_ExpectedPdu2, v_ExpectedPdu3}; //WA#WI=861440 v_ExpectedPdu6:= {v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3}; //WA#WI=861440 alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 //WA#WI=861440 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 //WA#WI=861440 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; } // receive PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3 //WA#WI=861440 { v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 //WA#WI=861440 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 //WA#WI=861440 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; //WA#WI=861440 v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 //WA#WI=861440 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } //@siclog "Step 16" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData); //@siclog "Step 17" siclog@ //The UE sends the PDCP Data PDU#5 f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2)); //@siclog "Step 20" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#7 p_NR_PDCP_State.TX_NEXT:= 7; p_NR_PDCP_State.RX_NEXT:= 7; //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData); //@siclog "Step 21" siclog@ //Check: Does the UE transmit a PDCP DATA PDU#7 on NR Cell 2? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell2, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } [] t_Wait.timeout{} } //@siclog "Step 21A" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5-190752 sic@ //@siclog "Step 22" siclog@ //The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); }

## Correction to function f_TC_7_1_3_4_1_NR_Step25to30

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | In the current TTCN implementation Using the type NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot at the steps 30 and 31 |
| Summary of change | For the test case 7.1.3.4.1 , it is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs at steps 30 and 31 |
| TTCN module | \ttcn\develop\NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted. Implemented by R5s210719 |

Before Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 24" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ //EXCEPTION: Steps 25 and 26 can occur in any order. (Note 7) interleave {//@sic R5s201484 sic@ //@siclog "Step 25" siclog@ //The UE transmits a NR RRCReconfigurationComplete message.(Note 4) []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } } else{ //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); // @sic R5s210282 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit); }; //@siclog "Step 27" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6 on NR Cell 1. DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell1, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(6)})); //@siclog "Step 28" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog "Step 28A" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData); //@sic R5-190752 sic@ //@siclog "Step 28B" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? t_Wait.start; //@sic R5-190752 sic@ alt { [] DRB.receive((car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1)))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28B"); } [] t_Wait.timeout{} } //@siclog "Step 29" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#6 p_NR_PDCP_State.TX_NEXT:= 6; p_NR_PDCP_State.RX_NEXT:= 6; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData); //@siclog "Steps 30 - 31" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#6.(Note 9) //Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#7(Note 9). //@sic R5-213462 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= (v_ExpectedPdu1,v_ExpectedPdu2); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } } }

After Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; //WA#WI=861440 var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; //WA#WI=861440 var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 24" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ //EXCEPTION: Steps 25 and 26 can occur in any order. (Note 7) interleave {//@sic R5s201484 sic@ //@siclog "Step 25" siclog@ //The UE transmits a NR RRCReconfigurationComplete message.(Note 4) []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } } else{ //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); // @sic R5s210282 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit); }; //@siclog "Step 27" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6 on NR Cell 1. DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell1, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(6)})); //@siclog "Step 28" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog "Step 28A" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData); //@sic R5-190752 sic@ //@siclog "Step 28B" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? t_Wait.start; //@sic R5-190752 sic@ alt { [] DRB.receive((car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1)))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28B"); } [] t_Wait.timeout{} } //@siclog "Step 29" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#6 p_NR_PDCP_State.TX_NEXT:= 6; p_NR_PDCP_State.RX_NEXT:= 6; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData); //@siclog "Steps 30 - 31" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#6.(Note 9) //Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#7(Note 9). //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //WA#WI=861440 f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); //WA#WI=861440 f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {v_ExpectedPdu1,v_ExpectedPdu2}; //WA#WI=861440 alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 //WA#WI=861440 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; //WA#WI=861440 v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } } }