3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210839

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1858 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The R5s201052 introduced 100ms delay in steps 9-10 in order to combat the situation where UL grant is disabled with explicit timing which gets activated after enabling the UL grant on SR with timing NOW. Thus leading to situation where UL grant is stopped when new RRC connection is expected. The solution works, however we propose a more robust enhancement that will avoid artificial delay (f_Delay(0.100)) introduced by R5s201052, will increase readability of the step 9-10 implementation and will make the usability of f_NR_RRCRelease_Local more universal by extending it of the return parameter carrying the timing used for the essential SS configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1 extends the f_NR_RRCRelease_Local of return parameter to provide timing of SS configurations. - Change 2 removes the artificial delay of 100ms in steps 9-10 and ensures that UL grant is started on SR after all the SS configuration performed by f_NR_RRCRelease_Local is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less robust and readable implementation of steps 9-10 in test 9.1.7.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_RRCRelease_Local |
| --- | --- |
| Reason for change | f_NR_RRCRelease_Local needs to be extended of return parameter for providing the timing information of the last SS configuration. |
| Summary of change | f_NR_RRCRelease_Local has been extended of return parameter for providing the timing information of the last SS configuration. |
| TTCN module | 9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment | Accepted. In addition the parameter p_CnfFlag is deleted, according to 38.523-3 ASP definition: when there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. See additional changes below in f_NR_UE_DeRegisterOnSwitchOff. |

Before Change

function f_NR_RRCRelease_Local(NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingAtT, template (omit) boolean p_CnfFlag := omit ) runs on NR_BASE_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs and (default) DRB * => this will in fact happen in the future */ /// @sic R5s190984: added p_CnfFlag sic@ var SubFrameTiming_Type v_Timing := p_TimingAtT; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_CellId); /* Step 1 At T: Send RRCRelease(done already), stop UL grants */ f_NR_ULGrantConfiguration_Stop(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 2 at T+5ms: release security */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 5); f_NR_SS_RRC_ReleaseSecurity(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 3 at T+10ms: Release DRX configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 10); f_NR_SS_DrxCtrlConfig(p_NR_CellId, cs_NR_DrxCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 4: At T + 15ms: Release measurement gap configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 15); f_NR_SS_MeasGapCtrlConfig(p_NR_CellId, cs_NR_MeasGapCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 5 at T+55ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 55); f_NR_SS_SRBs_DRBs_Release(p_NR_CellId, cs_TimingInfo_NR(v_Timing), v_DrbIdList); /* Step 6 at T+60ms: Configure SRBs */ // DRBs to be reconfigure during RRCReconfiguration v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 60); f_NR_SS_SRBs_DRBs_Config(p_NR_CellId, cs_TimingInfo_NR(v_Timing), omit, p_CnfFlag); //@sic R5s190109 change 23 sic@ }

After Change

function f_NR_RRCRelease_Local(NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingAtT, template (omit) boolean p_CnfFlag := omit ) runs on NR_BASE_PTC return SubFrameTiming_Type { /* NOTE: the function schedules all steps to release and reconfigure the SRBs and (default) DRB * => this will in fact happen in the future */ /// @sic R5s190984: added p_CnfFlag sic@ var SubFrameTiming_Type v_Timing var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_CellId); /* Step 1 At T: Send RRCRelease(done already), stop UL grants */ f_NR_ULGrantConfiguration_Stop(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 2 at T+5ms: release security */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 5); f_NR_SS_RRC_ReleaseSecurity(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 3 at T+10ms: Release DRX configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 10); f_NR_SS_DrxCtrlConfig(p_NR_CellId, cs_NR_DrxCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 4: At T + 15ms: Release measurement gap configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 15); f_NR_SS_MeasGapCtrlConfig(p_NR_CellId, cs_NR_MeasGapCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 5 at T+55ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 55); f_NR_SS_SRBs_DRBs_Release(p_NR_CellId, cs_TimingInfo_NR(v_Timing), v_DrbIdList); /* Step 6 at T+60ms: Configure SRBs */ // DRBs to be reconfigure during RRCReconfiguration v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 60); f_NR_SS_SRBs_DRBs_Config(p_NR_CellId, cs_TimingInfo_NR(v_Timing), omit, p_CnfFlag); //@sic R5s190109 change 23 sic@ return v_Timing; }

MCC160 proposed implementation

function f_NR_UE_DeRegisterOnSwitchOff(NR_CellId_Type p_NR_CellId,

NG_ALL_STATE_Type p_UE_State,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2,

boolean p_USIM_Rmvd := false,

B2_Type p_AccessType := '01'B) runs on NR5GC_PTC

{

var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_MSG_Indication_Type v_DetachReq;

var boolean v_RxdMsg := false;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var SubFrameTiming_Type v_TimingOfLocalRelease;

timer t_T3540 := f_NR_SetTimerToleranceMax (p_NR_CellId, nasTimer, 10.0);

if (p_UE_State == STATE_CONNECTED_3A_T3540){ // @sic R5-201202, R5-201333 sic@

t_T3540.start;

// release local configuration

f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId));

t_T3540.timeout;

// Now continue with RRC_IDLE procedure

}

if (pc_SwitchOnOff) { // The UE can actually be switched off

f_IP_ChangeNrCell(IP, p_NR_CellId);  // @sic R5s210603 sic@

select (p_UE_State) { // @sic R5-195325 sic@

case (STATE_IDLE_1A, STATE_CONNECTED_3A_T3540) {

// Already switched off in calling function

f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?);

v_DetachReq := fl_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, v_ReceivedMsg); // @sic R5-204389 sic@

v_RxdMsg := true; // @sic R5s190930 sic@

f_NR_RRCRelease(p_NR_CellId);

}

case (STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@

v_DetachReq := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?);

v_RxdMsg := true;

f_NR_RRCRelease(p_NR_CellId);

}

case (STATE_INACTIVE_2A){ // @sic R5-195325 sic@

f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@

v_ReceivedMsgTemplate := f_NR_RRC_Resume_DefWithNas(p_NR_CellId, tsc_NR_RRC_TI_Def, ?, ?);

if (not isvalue(v_ReceivedMsgTemplate)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

} else {

v_DetachReq := fl_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, valueof(v_ReceivedMsgTemplate)); // @sic R5-204389 sic@

v_RxdMsg := true; // @sic R5s190930 sic@

f_NR_RRCRelease(p_NR_CellId);

}

}

case (STATE_CONNECTED_3A){ // @sic R5-201202, R5-202562 sic@

f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s201132 sic@

// DRB already configured, so the UE won't send any NAS msg until it's completed IMS Deregistration

alt { // @sic R5-216151 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp

{

if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(),

v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed");

}

f_NR5GC_PDUSessionRelease(p_NR_CellId,

v_GSM_MobilityInfo.SessionId);

// Now wait for deregister msg

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

v_DetachReq := v_ReceivedAsp.Signalling.Nas[0];

}

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

v_DetachReq := v_ReceivedAsp.Signalling.Nas[0];

}

}

v_RxdMsg := true;

f_NR_RRCRelease(p_NR_CellId);

}

case(STATE_DEREGISTERED) { // @sic R5-201202 sic@

f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@

// don't need to release the local configuration as RRC Release is always done before this function is called @sic R5s201228 sic@

v_TimingOfLocalRelease := f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId));  //@sic R5s190984 sic@

f_Delay(0.1*int2float(f_SubFrameTiming_Duration(v_TimingOfLocalRelease, f_NR_GetCurrentTiming(p_NR_CellId)))); //@sic R5s210839 sic@

}

}

if (v_RxdMsg) { // @sic R5s190930 sic@

if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, p_AccessType))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

}

} else {

f_UT_PowerOffUE(UT);

}

// Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE

f_NR5GC_MobileInfo_Init();

}

## Change 2

| Function name | fl_TC_9_1_7_1_Body |
| --- | --- |
| Reason for change | In order to avoid artificial delay of 100ms used in step 9-10 the timing information provided by f_NR_RRCRelease_Local is used to start the UL grant on SR after all the SS configuration from f_NR_RRCRelease_Local is completed. |
| Summary of change | Removed f_Delay(0.100) from step 9-10 implementation and started UL grant on SR after f_NR_RRCRelease_Local is completed. |
| TTCN module | 9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment | Accepted. In addition, the parameter p_CnfFlag is deleted and UL grant start need not be configured twice. See proposed implementation below |

Before Change

//@siclog "Step 8F - 8H" siclog@ //@sic R5-199099, R5-201241 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B)) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA), tsc_CnfReq); //@sic R5s200087, R5s200662 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_Delay(0.100); //@sic R5s201052 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@

After Change

//@siclog "Step 8F - 8H" siclog@ //@sic R5-199099, R5-201241 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B)) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA), tsc_CnfReq); //@sic R5s200087, R5s200662 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //REMOVED f_Delay(0.100); //@sic R5s201052 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@

MCC160 proposed implementation

function f_NR_RRC_ConnEst_DefWithNas(NR_CellId_Type p_NR_CellId,

RRC_TransactionIdentifier p_RRC_TI,

template (present) EstablishmentCause p_EstablishmentCause,

template (present) SecurityHeaderType p_ExpectedSecurityStatus := tsc_SHT_IntegrityProtected,

template InitialUE_Identity p_InitialUE_Identity := ?,

template(present) RRCSetupComplete_IEs.selectedPLMN_Identity p_SelectedPLMN_Identity := ?,

template RRCSetupComplete_IEs.ng_5G_S_TMSI_Value p_Ng_5G_S_TMSI_Value:= *,

template RRCSetupComplete_v1610_IEs p_RRCSetupComplete_v1610_IEs := *,

boolean p_StartUL_Grant := true

)

runs on NR_BASE_PTC return NG_NAS_MSG_Indication_Type

{ //@sic R5s210839: added p_StartUL_Grant sic@

var NG_NAS_MSG_Indication_Type v_NasInd;

// receive RRC Connection Request on SRB0 (CCCH) and switch on UL grant:

f_NR_RRC_SetupRequest_Def(p_NR_CellId, p_EstablishmentCause, p_InitialUE_Identity, p_StartUL_Grant);

// send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments:

f_NR_RRCSetup_Def (p_NR_CellId);

// UE shall establish SRB1 acc. to 38.331 cl. 5.3.3.1

// UE enters RRC_CONNECTED state

// receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_NasInd := f_NR_RRCSetupComplete_Def(p_NR_CellId,

cr_38508_RRCSetupComplete(p_RRC_TI, p_SelectedPLMN_Identity,- , p_Ng_5G_S_TMSI_Value, p_RRCSetupComplete_v1610_IEs),

p_ExpectedSecurityStatus

);

return v_NasInd;

}

//----------------------------------------------------------------------------

/*

* @desc      Wait for RRC Setup Request sent by the UE; straight forward case without watchdog timer

* @param     p_NR_CellId

* @param     p_EstablishmentCause (default value: cr_NR_EstablishmentCause_Any)

* @param     p_InitialUE_Identity (default value: ?)

* @status    APPROVED (IMS, NR5GC, NR5GC_IRAT, POS)

*/

function f_NR_RRC_SetupRequest_Def(NR_CellId_Type p_NR_CellId,

template (present) EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ? ,

boolean p_StartUL_Grant := true

) runs on NR_BASE_PTC

{ //@sic R5s210839: added p_StartUL_Grant sic@

timer t_Dummy; // timer which is not started

f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Dummy, p_EstablishmentCause, p_InitialUE_Identity, p_StartUL_Grant);

}

//----------------------------------------------------------------------------

/*

* @desc      Common function to wait for RRC Setup Request sent by the UE;

*            when a running timer is handed over, this is used as watchdog.

* @param     p_NR_CellId

* @param     p_Timer

* @param     p_EstablishmentCause (default value: cr_NR_EstablishmentCause_Any)

* @param     p_InitialUE_Identity (default value: ?)

* @return    boolean

* @status    APPROVED (IMS, NR5GC, NR5GC_IRAT, POS)

*/

function f_NR_RRC_SetupRequest_Common(NR_CellId_Type p_NR_CellId,

timer p_Timer,

template  EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ?,

boolean p_StartUL_Grant := true)

runs on NR_BASE_PTC return boolean

{ /* common procedure to receive the initial RRC setup request for an expected RRC connection establishment procedure;

when RRC setup is not explicitly sent from TTCN but is preconfigured at the SS to be sent together with the contention resolution,

the default UL grant configuration is configured at the SS */

//@sic R5s210839: added p_StartUL_Grant sic@

var boolean v_TimerIsRunning := p_Timer.running;

var template (omit)  NR_UplinkTimeAlignment_Synch_Type v_PUCCH_Synch;

if (not f_NR_EarlyContentionResolution() and p_StartUL_Grant) {

v_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Stop;  /* No automatic UL time alignment:

if the SS would send MAC PDU with TA while the waits for contention resolution, it will consider contention resolutions as being fail

=> in case of automatic handling of the RRCConnectionSetup in Msg4 (RRCConnectionSetup) UL time alignment shall be started not before RRCConnectionSetupComplete has been received */

f_NR_ULGrantConfiguration_Start(p_NR_CellId, -, -, -, v_PUCCH_Synch, -, -, tsc_NoCnfReq ); //@sic R5s200161 sic@

} // else: UL grant configuration is done just before sending of RRCSetup

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(p_EstablishmentCause, p_InitialUE_Identity)))

{

p_Timer.stop;       /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */

return true;

}

[v_TimerIsRunning] p_Timer.timeout

{

}

}

return false;

}

…

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NG_NAS_GutiParameters_Type v_GutiParams_A := f_NR5GC_CellInfo_GetGutiParameters(v_NGC_NASCellA);

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_NGC_NASCellA, tsc_NR_BWP_Id);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA));

var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA);

var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)});

var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); //Get the QosFlow ID information from the DRB ID

var NG_MobileIdentity v_GUTIToSend;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var template (present) NG_MobileIdentity v_S_TMSI_A := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_A);

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var integer v_Count := 0; //@sic R5-194869 sic@

var integer v_PSI := valueof(v_QosFlowId.PDU_SessionId); //@sic R5s200087 sic@

var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data

var boolean v_Result := false; //@sic R5s200087 sic@

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

timer t_T3517 := 15.0; //@sic R5-194869 sic@

timer t_T3525 := 60.0; //@sic R5-194869 sic@

timer t_Wait; //@sic R5-201241 sic@

//@siclog "Steps 1" siclog@

…

if(not v_Result) //@sic R5s200087 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed");

}

//@siclog "Step 9 - 10" siclog@

//@sic R5-199099 sic@

for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) {

t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0));

alt{

[]SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected,

cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@

cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext),

v_S_TMSI_A,

cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@

omit,omit,*))))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry");

}

[]t_T3517.timeout

{   //@sic R5s210839 sic@

v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA)); //@sic R5s200087, R5s200662 sic@

f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

//UL grant already configured with activation time

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 sic@

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),

tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B,

-,

-)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

v_Result := false; //@sic R5s200087 sic@

if(v_PSI <8){ //@sic R5s200087 sic@

v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI);

}

else //@sic R5s200087 sic@

{

v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@

}

if(not v_Result) //@sic R5s200087 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed");

}

}

}

…