3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210840

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4637 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_ReturnCIOT_RRCConnectionSetupComplete_v8a0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For CIoT optimisation TCs, ttcn is currently populating expected values for up_CIoT_EPS_Optimisation_r13 & cp_CIoT_EPS_Optimisation_r13 fields in rrcConnectionSetupComplete message, based on pics parameters. According to TS 36.331 section 5.3.3.4, UE shall include support for up-CIoT-EPS-Optimisation & cp-CIoT-EPS-Optimisation in rrcConnectionSetupComplete message only if received from upper layers. Also TS 24.301 section 5.3.15 states, “Broadcast system information may provide information about support of CIoT EPS optimizations. At reception of new broadcast system information, the lower layers deliver it to the EMM layer in the UE. The information provided by lower layers is per PLMN and used by the UE to determine whether certain CIoT EPS optimizations are supported in the cell. The UE shall not attempt to use CIoT EPS optimizations which are indicated as not supported.” For CP CIoT TCs SIB2 are transmitted with support for only cp_CIoT_EPS_Optimisation_r13 & similarly for UP CIoT TCs SIB2 are transmitted with support for only up_CIoT_EPS_Optimisation_r13. Testcase will fail when the UE supports both CP & UP CIoT Optimisations (i.e., pc_Control_Plane_CIoT_Optimisation =TRUE and pc_User_Plane_CIoT_Optimisation=TRUE) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Expected values for up_CIoT_EPS_Optimisation_r13 & cp_CIoT_EPS_Optimisation_r13 fields are populated based on only testcase type rather than both testcase & pics parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Testcases 23.1.x, 23.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_ReturnCIOT_RRCConnectionSetupComplete_v8a0 |
| --- | --- |
| Reason for change | For CIoT optimisation TCs, ttcn is currently populating expected values for up_CIoT_EPS_Optimisation_r13 & cp_CIoT_EPS_Optimisation_r13 fields in rrcConnectionSetupComplete message, based on pics parameters. According to TS 36.331 section 5.3.3.4, UE shall include support for up-CIoT-EPS-Optimisation & cp-CIoT-EPS-Optimisation in rrcConnectionSetupComplete message only if received from upper layers. Also TS 24.301 section 5.3.15 states, “Broadcast system information may provide information about support of CIoT EPS optimizations. At reception of new broadcast system information, the lower layers deliver it to the EMM layer in the UE. The information provided by lower layers is per PLMN and used by the UE to determine whether certain CIoT EPS optimizations are supported in the cell. The UE shall not attempt to use CIoT EPS optimizations which are indicated as not supported.” For CP CIoT TCs SIB2 are transmitted with support for only cp_CIoT_EPS_Optimisation_r13 & similarly for UP CIoT TCs SIB2 are transmitted with support for only up_CIoT_EPS_Optimisation_r13. Testcase will fail when the UE supports both CP & UP CIoT Optimisations (i.e., pc_Control_Plane_CIoT_Optimisation =TRUE and pc_User_Plane_CIoT_Optimisation=TRUE) |
| Summary of change | Expected values for up_CIoT_EPS_Optimisation_r13 & cp_CIoT_EPS_Optimisation_r13 fields are populated based on only testcase type rather than both testcase & pics parameters. |
| TTCN module | EUTRA_RRC_Templates.ttcn |
| MCC160 Comment | Accepted in principle, but implemented differently, the function fl_InitialiseCIOT_SIB2 shall be corrected to follow SIB2 content specified in TS 36.508 Table 4.4.3.3-1: SystemInformationBlockType2. See proposed implementation below. |

Before change:

function fl_ReturnCIOT_RRCConnectionSetupComplete_v8a0() return template RRCConnectionSetupComplete_v8a0_IEs

{ //@sic R5-172999 sic@

var template RRCConnectionSetupComplete_v8a0_IEs  v_RRCConnectionSetupComplete_v8a0 := *;

var template (omit) RRCConnectionSetupComplete_UP_CIoT_Type v_UP_CIoT_Type := omit;

var template (omit) RRCConnectionSetupComplete_CP_CIoT_Type v_CP_CIoT_Type := omit;

if (f_GetTestcaseAttrib_CIOT_UP(testcasename()) or f_GetTestcaseAttrib_CIOT_CP(testcasename())) {

if (pc_User_Plane_CIoT_Optimisation) {

v_UP_CIoT_Type := true_;

}

if (pc_Control_Plane_CIoT_Optimisation) {

v_CP_CIoT_Type := true_;

}

//@sic R5-177082 sic@

v_RRCConnectionSetupComplete_v8a0 := cr_RRCConnectionSetupComplete_v8a0_IEs(cr_RRCConnectionSetupComplete_v1020_IEs(cr_RRCConnectionSetupComplete_v1130_IEs(cr_RRCConnectionSetupComplete_v1250_IEs(cr_RRCConnectionSetupComplete_v1320_IEs(v_UP_CIoT_Type, v_CP_CIoT_Type, *)))));  //@sic R5-177082 R5s180379 sic@

}

return v_RRCConnectionSetupComplete_v8a0

}

After change:

function fl_ReturnCIOT_RRCConnectionSetupComplete_v8a0() return template RRCConnectionSetupComplete_v8a0_IEs

{ //@sic R5-172999 sic@

var template RRCConnectionSetupComplete_v8a0_IEs  v_RRCConnectionSetupComplete_v8a0 := *;

var template (omit) RRCConnectionSetupComplete_UP_CIoT_Type v_UP_CIoT_Type := omit;

var template (omit) RRCConnectionSetupComplete_CP_CIoT_Type v_CP_CIoT_Type := omit;

if (f_GetTestcaseAttrib_CIOT_CP(testcasename())) {

v_CP_CIoT_Type := true_;

v_RRCConnectionSetupComplete_v8a0 := cr_RRCConnectionSetupComplete_v8a0_IEs(cr_RRCConnectionSetupComplete_v1020_IEs(cr_RRCConnectionSetupComplete_v1130_IEs(cr_RRCConnectionSetupComplete_v1250_IEs(cr_RRCConnectionSetupComplete_v1320_IEs(v_UP_CIoT_Type, v_CP_CIoT_Type, *)))));  //@sic R5-177082 R5s180379 sic@

}

if (f_GetTestcaseAttrib_CIOT_UP(testcasename())) {

v_UP_CIoT_Type := true_;

v_RRCConnectionSetupComplete_v8a0 := cr_RRCConnectionSetupComplete_v8a0_IEs(cr_RRCConnectionSetupComplete_v1020_IEs(cr_RRCConnectionSetupComplete_v1130_IEs(cr_RRCConnectionSetupComplete_v1250_IEs(cr_RRCConnectionSetupComplete_v1320_IEs(v_UP_CIoT_Type , v_CP_CIoT_Type, *)))));  //@sic R5-177082 R5s180379 sic@

}

return v_RRCConnectionSetupComplete_v8a0

}

MCC160 proposed implementation

function fl_InitialiseCIOT_SIB2() return template(omit) CIOT_EPS_OptimisationInfo_r13

{

var template (omit) CIOT_EPS_OptimisationInfo_r13 v_CIOT_EPS_OptimisationInfo := omit;

var boolean v_CIoT_UP := f_GetTestcaseAttrib_CIOT_UP( testcasename());

var boolean v_CIoT_CP := f_GetTestcaseAttrib_CIOT_CP( testcasename());

if (v_CIoT_UP or v_CIoT_CP) {

v_CIOT_EPS_OptimisationInfo := cs_CIOT_EPS_OptimisationInfo_OneEntry( true_, true_ );

}

return v_CIOT_EPS_OptimisationInfo;

}