3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210842

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1860 | rev | - | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.5.8.2.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5GC test case 8.1.5.8.2.1 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.5.8.2.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections requireds	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	12

2.4	Change 4	13

3	Branches executed	13

4	Execution Log Files	14

4.1	HiSilicon Balong 5000	14

5	References	14

# Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk24  related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections requireds

## Change 1

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | 1 SS configuration to add Scell should be set after RRCReconfiguration message,before RRCReconfigurationComplete message. |
| Summary of change | 1 Calculate v_TimingInfo to send RRCReconfiguration message and v_TimingInfo2 to config SS. |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type     p_PCell,

NR_CellId_Type     p_SCell) runs on NR5GC_PTC

{

var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1);

var template(value) CellGroupConfig v_SCGConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA)));

var template(value) RadioBearerConfig v_RadioBearerConfig;

var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell);

var integer i;

// @siclog "Steps 1-3" siclog@

// The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH.

// Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3)

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))),

car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete),

16,   //converted in slots inside the function

"Step 3");

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_Now);

// @siclog "Steps 4-5" siclog@

//The SS transmits an RRCReconfiguration message to release SCell(Cell 3)

//UE transmits an RRCReconfigurationComplete message

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);

if ( pc_inactiveState ) {

// @siclog "Step6a1" siclog@

// IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(p_PCell);

// @siclog "Step6a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

// @siclog "Step6a2" siclog@

// The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(p_PCell, RRC_INACTIVE);

// @siclog "Step6a3" siclog@

// The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(p_PCell);

// The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 10 to schedule PUSCH.  (Note 1, Note 2)

// Check: Does the UE transmit an RRCResumeComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3)

v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell);                          // condition RESUME

v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig;  // condition SCell_Add

v_SCGConfig.sCellToAddModList  := v_SCGConfigCA.sCellToAddModList;        // condition SCell_Add

for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

//@sic R5s210620 sic@

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_Now, f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_Now);

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)),

car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)),

16,                  //converted in slots inside the function

"Step 6a7");

//DRBs are not reconfigured in SS: that is ok as they are not used

f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig);

}

}

After change

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type     p_PCell,

NR_CellId_Type     p_SCell) runs on NR5GC_PTC

{

var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1);

var template(value) CellGroupConfig v_SCGConfig;

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA)));

var template(value) RadioBearerConfig v_RadioBearerConfig;

var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit;

var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList();

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell);

var integer i;

var SubFrameTiming_Type v_TimingInfo;

var SubFrameTiming_Type v_TimingInfo2;

// @siclog "Steps 1-3" siclog@

// The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH.

// Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3)

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500);

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2));

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_TimingInfo), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))),

car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete),

16,   //converted in slots inside the function

"Step 3");

// @siclog "Steps 4-5" siclog@

//The SS transmits an RRCReconfiguration message to release SCell(Cell 3)

//UE transmits an RRCReconfigurationComplete message

f_NR5GC_Rel_1SCell(p_PCell, p_SCell);

if ( pc_inactiveState ) {

// @siclog "Step6a1" siclog@

// IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig.

f_NR_RRC_InactiveDef(p_PCell);

// @siclog "Step6a2" siclog@

//Wait 1s to let UE enter RRC_INACTIVE state.

f_Delay(1.0);

// @siclog "Step6a2" siclog@

// The SS transmits a Paging message including a matched ng-5G-S-TMSI.

f_NR_Paging_Def(p_PCell, RRC_INACTIVE);

// @siclog "Step6a3" siclog@

// The UE transmits an RRCResumeRequest message.

f_NR_RRC_ResumeRequest_Def(p_PCell);

// The SS transmits an RRCResume message with sCellToAddModList to add SCell(Cell 3)

// The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 10 to schedule PUSCH.  (Note 1, Note 2)

// Check: Does the UE transmit an RRCResumeComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3)

v_SCGConfig := f_NR_GetSCGConfigResume(p_PCell);                          // condition RESUME

v_SCGConfig.mac_CellGroupConfig := v_SCGConfigCA.mac_CellGroupConfig;  // condition SCell_Add

v_SCGConfig.sCellToAddModList  := v_SCGConfigCA.sCellToAddModList;        // condition SCell_Add

for (i:=0; i < lengthof(v_ExistingDRBs); i:=i+1) {

v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit);

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList);

v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500);

v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later

f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell));

f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2));

f_NR5GC_RRC_ProcedureDelay(p_PCell,

cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)),

car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, - crs_MacBearerRouting(p_PCell)),

16,                  //converted in slots inside the function

"Step 6a7");

//DRBs are not reconfigured in SS: that is ok as they are not used

f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig);

}

}

## Change 2

| Function name | f_NR5GC_RRC_ProcedureDelay |
| --- | --- |
| Reason for change | 1 According to TS38.523-1 Table 8.1.5.8.2.1.3.2-3, multiple by 10 is not needed; 2 For change 1.1; 3 mod v_MaxNumberSlots is not needed, mode here will made HSFN to 0 when calculate timing in next function f_NR_ConvertTimingInSlots_To_Timing; 4 Using T1 as the time to enable the UL HARQ,just to make sure the configuration can take effect in time. |
| Summary of change | 1 Modify the value of v_Nslots; 2 Using the time calculated in f_TC_8_1_5_8_2_x_NR5GC_TestBody to send RRCReconfiguration message; 3 Delete mod v_MaxNumberSlots; 4 Using T1 as the time to enable the UL HARQ. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | 1. Accepted 2. Accepted – see proposed implementation below 3. Accepted 4. Accepted |

Before change

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep

) runs on NR_BASE_PTC return NR_SRB_COMMON_IND

{

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*10*v_SlotsNumberPerSubframe + 1;

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

timer t_Watchdog;

v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_T1Slots := (((v_T1.HSFN.Number * 10240) + (v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

v_K1 := 7;

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

v_K1 := 7;

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

v_K1 := 5;

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

v_K1 := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

//Calculate T3

v_T3Slots := (v_T1Slots + v_Nslots + v_Delta1) mod v_MaxNumberSlots;

v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe);

//T1: send DL Msg

p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

SRB.send(p_SRB_COMMON_REQ);

//Enable HARQ ack/nack reporting

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)   );

//T3 configure SS with periodic UL Grant at T3

f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity));

// Start WatchDog timer

v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId);

t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0);  /* 50ms added to be at the safe side */

…

return v_ReceivedNR_SRB_COMMON_IND;

}

After change

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep

) runs on NR_BASE_PTC return NR_SRB_COMMON_IND

{

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1;

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

timer t_Watchdog;

//v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_T1 := valueof(p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame);

v_T1Slots := (((v_T1.HSFN.Number * 10240) + (v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

v_K1 := 7;

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

v_K1 := 7;

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

v_K1 := 5;

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

v_K1 := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

//Calculate T3

v_T3Slots := (v_T1Slots + v_Nslots + v_Delta1);

v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe);

//T1: send DL Msg

//p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

SRB.send(p_SRB_COMMON_REQ);

//Enable HARQ ack/nack reporting

//f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable));

f_NR_SS_SystemIndCtrlConfig (p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable),cs_TimingInfo_NR(v_T1));

//T3 configure SS with periodic UL Grant at T3

f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity));

…

return v_ReceivedNR_SRB_COMMON_IND;

}

MCC160 proposed implementation

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep

) runs on NR_BASE_PTC return NR_SRB_COMMON_IND

{

var integer v_T1Slots;

var integer v_T2Slots;

var integer v_T3Slots;

var integer v_T4Slots;

var SubFrameTiming_Type v_T1; // Send DL Msg

var SubFrameTiming_Type v_T2; // HARQ ACK receive

var SubFrameTiming_Type v_T3; // Start UL Grant start

var SubFrameTiming_Type v_T4; // UL RRC PDU received

var SubFrameTiming_Type v_Tnow;

var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId);

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId);

var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1;  //@sic R5s210842 change 2.1 sic@

var integer v_Delta1;

var integer v_Delta2;

var integer v_GrantPeriodicity;

var integer v_DL_RetransmissionCnt := 0;

var NR_SYSTEM_IND v_SYS_IND;

var boolean v_T2Flag := false;

var boolean v_T4Flag := false;

var integer v_K1;

var integer v_K2 := 4;

var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND;

var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe;     // Maximum number per SFN complete cycle

timer t_Watchdog;

//@sic R5s210842 sic@

if (f_TimingInfo_IsNow(p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame)) {

v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId);

p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1;

} else {

v_T1 := valueof(p_SRB_COMMON_REQ.Common.TimingInfo.SubFrame);

}

v_T1Slots := (((v_T1.HSFN.Number * 10240) + (v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1);

// Initialise GrantPeriodicity, Delta1, K1

if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){

v_GrantPeriodicity := 1; // Grant repeated in every slot

v_Delta1 := 0;

v_K1 := 7;

} else { // TDD

select(v_SCS) {

case(kHz15) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5;

v_K1 := 7;

}

case(kHz30) {

v_GrantPeriodicity := 10; //Grant repeated every 10 slots

v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10;

v_K1 := 5;

}

case(kHz120) {

v_GrantPeriodicity := 5; //Grant repeated every 5 slots

v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5;

v_K1 := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); }

}

}

//Calculate T3

v_T3Slots := v_T1Slots + v_Nslots + v_Delta1;  //@sic R5s210842 change 2.3 sic@

v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe);

//T1: send DL Msg

SRB.send(p_SRB_COMMON_REQ);

## Change 3

| Function name | f_NR_SS_SystemIndCtrlConfig |
| --- | --- |
| Reason for change | For change 2.4 |
| Summary of change | Add a new parameter to pass in the timing infor. |
| TTCN module | NR_ASP_Templates_SysInd.ttcn |
| MCC160 Comment | Accepted but the added parameter shall be declared as template(omit) |

Before change

function f_NR_SS_SystemIndCtrlConfig(NR_CellId_Type p_NR_CellId,

template (value) NR_System_IndicationControl_Type p_SystemIndCtrl) runs on NR_BASE_PTC

{

var template (value) NR_SYSTEM_CTRL_REQ v_NR_SYSTEM_CTRL_REQ := cas_NR_SystemIndCtrl_REQ(p_NR_CellId, p_SystemIndCtrl);

var boolean v_CnfFlag := valueof(v_NR_SYSTEM_CTRL_REQ.Common.ControlInfo.CnfFlag);

SYS.send(v_NR_SYSTEM_CTRL_REQ);

if (v_CnfFlag) {

SYS.receive(car_NR_SystemIndCtrl_CNF(p_NR_CellId));

}

}

After change

function f_NR_SS_SystemIndCtrlConfig(NR_CellId_Type p_NR_CellId,

template (value) NR_System_IndicationControl_Type p_SystemIndCtrl,

template (present) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now) runs on NR_BASE_PTC

{

var template (value) NR_SYSTEM_CTRL_REQ v_NR_SYSTEM_CTRL_REQ := cas_NR_SystemIndCtrl_REQ_815821(p_NR_CellId, p_SystemIndCtrl,p_TimingInfo);

var boolean v_CnfFlag := valueof(v_NR_SYSTEM_CTRL_REQ.Common.ControlInfo.CnfFlag);

SYS.send(v_NR_SYSTEM_CTRL_REQ);

if (v_CnfFlag) {

SYS.receive(car_NR_SystemIndCtrl_CNF(p_NR_CellId));

}

}

## Change 4

| Function name | cas_NR_SystemIndCtrl_REQ |
| --- | --- |
| Reason for change | For change 3 |
| Summary of change | Add a new parameter to pass in the timing infor. |
| TTCN module | NR_ASP_Templates_SysInd.ttcn |
| MCC160 Comment | Accepted but the added parameter shall be declared as template(omit) |

Before change

template (value) NR_SYSTEM_CTRL_REQ cas_NR_SystemIndCtrl_REQ(NR_CellId_Type p_NR_CellId,

template (value) NR_System_IndicationControl_Type p_SystemIndCtrl) :=

{ /* @status    APPROVED (ENDC, NR5GC) */

Common := cs_NR_ReqAspCommonPart(p_NR_CellId, -, -, cs_TimingInfo_Now),

Request := {

SystemIndCtrl := p_SystemIndCtrl

}

};

After change

template (value) NR_SYSTEM_CTRL_REQ cas_NR_SystemIndCtrl_REQ(NR_CellId_Type p_NR_CellId,

template (value) NR_System_IndicationControl_Type p_SystemIndCtrl,

template (present) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now) :=

{ /* @status    APPROVED (ENDC, NR5GC) */

Common := cs_NR_ReqAspCommonPart(p_NR_CellId, -, -, p_TimingInfo),

Request := {

SystemIndCtrl := p_SystemIndCtrl

}

};

# Branches executed

The test case was executed on NR band CA_n41C.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_1_5_8_2_1_NR5GC_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

# References

| [1] | R5s210843: Supporting information for agreement of NR5GC test case 8.1.5.8.2.1 in FR1. |
| --- | --- |