3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210861

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1868 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_UT_ReadAllowedNSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function, the assignment v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); ensures that the empty lines from the list are already removed. Thus, the actual string value would be available at v_ListOfLines[0], rather than v_ListOfLines[1]. However, in the current TTCN implementation of this function, the variable v_CharStringList is assigned the output of f_StringSplit(v_ListOfLines[1], {","});. This needs to be changed to f_StringSplit(v_ListOfLines[0], {","}); |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of assignment v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, assigned v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_UT_ReadAllowedNSSAI	3

# Corrections required

## function f_UT_ReadAllowedNSSAI

| Function name | f_UT_ReadAllowedNSSAI |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the assignment v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); ensures that the empty lines from the list are already removed. Thus, the actual string value would be available at v_ListOfLines[0], rather than v_ListOfLines[1]. However, in the current TTCN implementation of this function, the variable v_CharStringList is assigned the output of f_StringSplit(v_ListOfLines[1], {","});. This needs to be changed to f_StringSplit(v_ListOfLines[0], {","}); |
| Summary of change | Instead of assignment v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, assigned v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |
| TTCN module | Common\Common\UppwerTesterFunctions.ttcn |
| MCC160 Comment | Rejected. The AT command +C5GNSSAIRDP is sent with the parameter <nssai_type>: set to ‘3’. This will return the stored default configured NSSAI, rejected NSSAI(s), configured NSSAI(s) and allowed NSSAI(s). As the allowed NSSAI is always provided in the 2nd line, an index of [1] is correct. |

Before Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); // @sic R5s210431 sic@ }<<SKIPPED CODE>>

After Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); // @sic R5s210431 sic@ }<<SKIPPED CODE>>