3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210868

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4640 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA CA test frequencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2021-09-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From 36.508 Table 6.2.3.2-2: Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation and E-UTRA PCell and PSCell for DC Inter-band operation (two bands), f1 dl test frequency is currently incorrect in the function f_EUTRA_CA_InitFrequenciesInterBand() |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct f1 dl test Frequency for ChBandwidth 10 for CA band combination (30A-66A) and (30A_66A_SwitchAllocation). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail test cases in these CA band combinations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_EUTRA_CA_InitFrequenciesInterBand() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_CA_InitFrequenciesInterBand() |
| --- | --- |
| Reason for change | From 36.508 Table 6.2.3.2-2: Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation and E-UTRA PCell and PSCell for DC Inter-band operation (two bands), f1 dl test frequency is currently incorrect in the function f_EUTRA_CA_InitFrequenciesInterBand() |
| Summary of change | Correct f1 dl test Frequency for ChBandwidth 10 for CA band combination (30A-66A) and (30A_66A_SwitchAllocation) |
| TTCN module | Common\EUTRA\ EUTRA_CellInfoInit_CA.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_EUTRA_CA_InitFrequenciesInterBand(CA_BandCombination_Type p_CA_FrequencyBand) return Frequency_fList_CAInterBand_Type

{ // @sic R5-155910 New CA band combination pixit sic@

var template(value) Frequency_fList_CAInterBand_Type v_Freq := cs_FrequencyCAInterBand_Dummy;

//@sic R5s150438 sic@

select (p_CA_FrequencyBand) {

case (CA_1A_3A){ // CA_1A-3A @sic R5-145181 sic@

v_Freq.f1.dl_CarrierFreq := 300;

v_Freq.f1.ul_CarrierFreq := 18300;

v_Freq.f2.dl_CarrierFreq := 550;

v_Freq.f2.ul_CarrierFreq := 18550;

v_Freq.f5.dl_CarrierFreq := 1575;

v_Freq.f5.ul_CarrierFreq := 19575;

v_Freq.f6.dl_CarrierFreq := 1900;

v_Freq.f6.ul_CarrierFreq := 19900;

}

case (CA_1A_3A_SwitchAllocation){  // CA_1A-3A Switch Allocation @sic R5-145663 sic@

v_Freq.f5.dl_CarrierFreq := 300;

v_Freq.f5.ul_CarrierFreq := 18300;

v_Freq.f6.dl_CarrierFreq := 550;

v_Freq.f6.ul_CarrierFreq := 18550;

v_Freq.f1.dl_CarrierFreq := 1575;

v_Freq.f1.ul_CarrierFreq := 19575;

v_Freq.f2.dl_CarrierFreq := 1900;

v_Freq.f2.ul_CarrierFreq := 19900;

}<<Skiped Code>>

case (CA_30A_66A){// CA_30A-66A @sic R5-180749 sic@

v_Freq.f1.dl_CarrierFreq := 2355;

v_Freq.f1.ul_CarrierFreq := 27710;

v_Freq.f5.dl_CarrierFreq := maxEARFCN;

v_Freq.f5.dl_CarrierFreq_v9e0 := 66886;

v_Freq.f5.ul_CarrierFreq := maxEARFCN;

v_Freq.f5.ul_CarrierFreq_v9e0 := 132422;

v_Freq.f5_DL_ChBandwidth := n100;

v_Freq.f5_UL_ChBandwidth := n100;

v_Freq.f6.dl_CarrierFreq := maxEARFCN;

v_Freq.f6.dl_CarrierFreq_v9e0 := 67036;

v_Freq.f6.ul_CarrierFreq := maxEARFCN;

v_Freq.f6.ul_CarrierFreq_v9e0 := 132572;

v_Freq.f6_DL_ChBandwidth := n100;

v_Freq.f6_UL_ChBandwidth := n100;

}<<Skiped Code>>

case (CA_30A_66A_SwitchAllocation){// CA_30A-66A Switched Allocation @sic R5-180749 sic@

v_Freq.f5.dl_CarrierFreq := 2355;

v_Freq.f5.ul_CarrierFreq := 27710;

v_Freq.f1.dl_CarrierFreq := maxEARFCN;

v_Freq.f1.dl_CarrierFreq_v9e0 := 66886;

v_Freq.f1.ul_CarrierFreq := maxEARFCN;

v_Freq.f1.ul_CarrierFreq_v9e0 := 132422;

v_Freq.f1_DL_ChBandwidth := n100;

v_Freq.f1_UL_ChBandwidth := n100;

v_Freq.f2.dl_CarrierFreq := maxEARFCN;

v_Freq.f2.dl_CarrierFreq_v9e0 := 67036;

v_Freq.f2.ul_CarrierFreq := maxEARFCN;

v_Freq.f2.ul_CarrierFreq_v9e0 := 132572;

v_Freq.f2_DL_ChBandwidth := n100;

v_Freq.f2_UL_ChBandwidth := n100;

}}<<Skiped Code>>

After change

function f_EUTRA_CA_InitFrequenciesInterBand(CA_BandCombination_Type p_CA_FrequencyBand) return Frequency_fList_CAInterBand_Type

{ // @sic R5-155910 New CA band combination pixit sic@

var template(value) Frequency_fList_CAInterBand_Type v_Freq := cs_FrequencyCAInterBand_Dummy;

//@sic R5s150438 sic@

select (p_CA_FrequencyBand) {

case (CA_1A_3A){ // CA_1A-3A @sic R5-145181 sic@

v_Freq.f1.dl_CarrierFreq := 300;

v_Freq.f1.ul_CarrierFreq := 18300;

v_Freq.f2.dl_CarrierFreq := 550;

v_Freq.f2.ul_CarrierFreq := 18550;

v_Freq.f5.dl_CarrierFreq := 1575;

v_Freq.f5.ul_CarrierFreq := 19575;

v_Freq.f6.dl_CarrierFreq := 1900;

v_Freq.f6.ul_CarrierFreq := 19900;

}

case (CA_1A_3A_SwitchAllocation){  // CA_1A-3A Switch Allocation @sic R5-145663 sic@

v_Freq.f5.dl_CarrierFreq := 300;

v_Freq.f5.ul_CarrierFreq := 18300;

v_Freq.f6.dl_CarrierFreq := 550;

v_Freq.f6.ul_CarrierFreq := 18550;

v_Freq.f1.dl_CarrierFreq := 1575;

v_Freq.f1.ul_CarrierFreq := 19575;

v_Freq.f2.dl_CarrierFreq := 1900;

v_Freq.f2.ul_CarrierFreq := 19900;

}<<Skiped Code>>

case (CA_30A_66A){// CA_30A-66A @sic R5-180749 sic@

v_Freq.f1.dl_CarrierFreq := 9820;

v_Freq.f1.ul_CarrierFreq := 27710;

v_Freq.f5.dl_CarrierFreq := maxEARFCN;

v_Freq.f5.dl_CarrierFreq_v9e0 := 66886;

v_Freq.f5.ul_CarrierFreq := maxEARFCN;

v_Freq.f5.ul_CarrierFreq_v9e0 := 132422;

v_Freq.f5_DL_ChBandwidth := n100;

v_Freq.f5_UL_ChBandwidth := n100;

v_Freq.f6.dl_CarrierFreq := maxEARFCN;

v_Freq.f6.dl_CarrierFreq_v9e0 := 67036;

v_Freq.f6.ul_CarrierFreq := maxEARFCN;

v_Freq.f6.ul_CarrierFreq_v9e0 := 132572;

v_Freq.f6_DL_ChBandwidth := n100;

v_Freq.f6_UL_ChBandwidth := n100;

}<<Skiped Code>>

case (CA_30A_66A_SwitchAllocation){// CA_30A-66A Switched Allocation @sic R5-180749 sic@

v_Freq.f5.dl_CarrierFreq := 9820;

v_Freq.f5.ul_CarrierFreq := 27710;

v_Freq.f1.dl_CarrierFreq := maxEARFCN;

v_Freq.f1.dl_CarrierFreq_v9e0 := 66886;

v_Freq.f1.ul_CarrierFreq := maxEARFCN;

v_Freq.f1.ul_CarrierFreq_v9e0 := 132422;

v_Freq.f1_DL_ChBandwidth := n100;

v_Freq.f1_UL_ChBandwidth := n100;

v_Freq.f2.dl_CarrierFreq := maxEARFCN;

v_Freq.f2.dl_CarrierFreq_v9e0 := 67036;

v_Freq.f2.ul_CarrierFreq := maxEARFCN;

v_Freq.f2.ul_CarrierFreq_v9e0 := 132572;

v_Freq.f2_DL_ChBandwidth := n100;

v_Freq.f2_UL_ChBandwidth := n100;

}}<<Skiped Code>>