3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210871

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1876 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 3, the timing for downlink data is scheduled using short DRX cycle. Since no data is sent before this step, the drx-InactivityTimer is never activated yet (i.e. short cdrx condition not met) so the UE will still using long DRX cycle. This can cause data to be sent outside the DRX onDuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Using LongDrxCycle instead of ShortDrxCycle for timing calculation at step 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.3.ENDC, 7.1.1.5.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_5_3_NR_TestBody() |
| --- | --- |
| Reason for change | At step 3, the timing for downlink data is scheduled using short DRX cycle. Since no data is sent before this step, the drx-InactivityTimer is never activated yet (i.e. short cdrx condition not met) so the UE will still using long DRX cycle. This can cause data to be sent outside the DRX onDuration |
| Summary of change | Using LongDrxCycle instead of ShortDrxCycle for timing calculation at step 3. |
| TTCN module | DRX_MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_7_1_1_5_3_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_OnDurationTimer := 20; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) ………………….. // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 3" siclog@ t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Choose the next subframe when the onDurationTimer is started. v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); ………………………….. }

After Change

function f_TC_7_1_1_5_3_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_OnDurationTimer := 20; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) ……………………. // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@siclog "Step 3" siclog@ t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Choose the next subframe when the onDurationTimer is started. v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); …………………………… }