3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210872

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1877 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of 8.1.5.4.1 SS transmits 10 IP packets on each DRB (using QFI2 for DRB1 and QFI5 for DRB2) . At step2 ,SS may receive the set of loop back packets in the same slot or in multiple slots |  |  |  |  |  |  |  |  |
| Summary of change: |  | TTCN is modified to handle set of loop back packets in the same slot or in multiple slots |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to module RRC_Others_NR5GC	2

1.2	Correction to function f_TC_8_1_5_4_1_NR5GC_TestBody	3

# Corrections required

## Correction to module RRC_Others_NR5GC

| Module name | RRC_Others_NR5GC |
| --- | --- |
| Reason for change | NR_DRB_COMMON_IND data type has to be used for the handling of multiple IP packets in the same slot or in multiple slots .Importing NR_ASP_DrbDefs is required for the compilation purpose |
| Summary of change | Import of NR_ASP_DrbDefs is added in the module RRC_Others_NR5GC |
| TTCN module | NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR approval. |

Before Change:

import from NG_NAS_MsgContainers all; import from NAS_CommonTypeDefs all; import from NAS_CommonTemplates all; import from NR_SecurityFunctions all; import from EUTRA_NR_Parameters all; import from NR_ASP_Templates_CommonPart all; import from NR_DCI_Configuration all;

After Change:

import from NG_NAS_MsgContainers all; import from NAS_CommonTypeDefs all; import from NAS_CommonTemplates all; import from NR_SecurityFunctions all; import from EUTRA_NR_Parameters all; import from NR_ASP_Templates_CommonPart all; import from NR_DCI_Configuration all; import from NR_ASP_DrbDefs all;

## Correction to function f_TC_8_1_5_4_1_NR5GC_TestBody

| Function name | f_TC_8_1_5_4_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of 8.1.5.4.1 The SS transmits 10 IP packets on each DRB (using QFI2 for DRB1 and QFI5 for DRB2) . At step2 ,SS may receive the set of loop back packets in the same slot or in multiple slots |
| Summary of change | TTCN is modified to handle set of loop back packets in the same slot or in multiple slots ;Also added pass verdicts based on the number of packets received associated to QFI2 and QFI5 |
| TTCN module | Ttcn\develop\NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted, conditional to approval of prose CR. |

Before Change:

function f_TC_8_1_5_4_1_NR5GC_TestBody (IntegerList_Type p_DRBList, integer p_PduSessionId) runs on NR_BASE_PTC { var template (value) DRB_CountMSB_InfoList v_CountList; var integer v_FirstDRB := p_DRBList[0]; var integer v_SecondDRB := p_DRBList[1]; var integer v_ThirdDRB := p_DRBList[2]; // To be used as not established DRB // @sic R5s200630 sic@ var octetstring v_IPDataQFI2 := f_SdapTests_CreateDatagram("3"); // Packet with remote address matching filter #3 @sic R5s200733 sic@ var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4 var template (value) QosFlow_Identification_Type v_QoS_Id2 := cs_IP_QosFlowId(p_PduSessionId, 2); // QFI2 is mapped to First DRB @sic R5s200733 sic@ var template (value) QosFlow_Identification_Type v_QoS_Id5 := cs_IP_QosFlowId(p_PduSessionId, 5); // QFI5 is mapped to Second DRB var integer i; var integer v_IPDataCount := 0; // @siclog "Step 1" siclog@ // The SS transmits 10 IP packets on each DRB (using QF1 for DRB1 and QFI5 for DRB2) for (i:=0; i < 10; i:=i+1) { DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id2, {v_IPDataQFI2})); // @sic R5s200630, R5s200733 sic@ } for (i:=0; i < 10; i:=i+1) { DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id5, {v_IPDataQFI5})); // @sic R5s200630 sic@ } // @siclog "Step 2" siclog@ // The UE loops back the IP packets received in step 1 do { DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, (v_QoS_Id2, v_QoS_Id5), ({v_IPDataQFI2}, {v_IPDataQFI5}))); // @sic R5s200630, R5s200733, R5s200978 sic@ v_IPDataCount := v_IPDataCount + 1; } while (v_IPDataCount < 20); // @siclog "Step 3" siclog@ // The SS transmits CounterCheck message with drb-Identity not included in the drb-CountMSB-InfoList v_CountList := {cs_38508_DRB_CountMSB_Info(v_FirstDRB, 1, 1), cs_38508_DRB_CountMSB_Info(v_ThirdDRB, 3, 3)}; SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList))); // @siclog "Step 4" siclog@ // Check: Does the UE send CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink // set to the value of TX_NEXT and RX_NEXT" 1 in drb-CountInfoList IE for each DRB established? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, {permutation (cr_DRB_CountInfo(v_FirstDRB, 9, 9), //@sic R5s200978 sic@ cr_DRB_CountInfo(v_SecondDRB, 9, 9), //@sic R5s200978 sic@ cr_DRB_CountInfo(v_ThirdDRB, 384, 384))}))); //@sic R5s200733 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); }// end of f_TC_8_1_5_4_1_NR5GC_TestBody

After Change:

function f_TC_8_1_5_4_1_NR5GC_TestBody (IntegerList_Type p_DRBList, integer p_PduSessionId) runs on NR_BASE_PTC { var template (value) DRB_CountMSB_InfoList v_CountList; var integer v_FirstDRB := p_DRBList[0]; var integer v_SecondDRB := p_DRBList[1]; var integer v_ThirdDRB := p_DRBList[2]; // To be used as not established DRB // @sic R5s200630 sic@ var octetstring v_IPDataQFI2 := f_SdapTests_CreateDatagram("3"); // Packet with remote address matching filter #3 @sic R5s200733 sic@ var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4 var template (value) QosFlow_Identification_Type v_QoS_Id2 := cs_IP_QosFlowId(p_PduSessionId, 2); // QFI2 is mapped to First DRB @sic R5s200733 sic@ var template (value) QosFlow_Identification_Type v_QoS_Id5 := cs_IP_QosFlowId(p_PduSessionId, 5); // QFI5 is mapped to Second DRB var integer i; var integer v_IPDataCount := 0; var integer v_IPDataCount_QFI2 := 0; var integer v_IPDataCount_QFI5 := 0; var NR_DRB_COMMON_IND v_ReceivedPdu1; var integer received_packets := 0; var octetstring v_IPDataQFIx := '00'O; // @siclog "Step 1" siclog@ // The SS transmits 10 IP packets on each DRB (using QF1 for DRB1 and QFI5 for DRB2) for (i:=0; i < 10; i:=i+1) { DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id2, {v_IPDataQFI2})); // @sic R5s200630, R5s200733 sic@ } for (i:=0; i < 10; i:=i+1) { DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QoS_Id5, {v_IPDataQFI5})); // @sic R5s200630 sic@ } // @siclog "Step 2" siclog@ // The UE loops back the IP packets received in step 1 do { DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, (v_QoS_Id2, v_QoS_Id5), ?)) -> value v_ReceivedPdu1 ;// @sic R5s200630, R5s200733, R5s200978 sic@ { received_packets := lengthof(v_ReceivedPdu1.U_Plane.SlotData.PduSduList.SdapSdu); for ( i:=0 ; i < received_packets ; i := i+1 ) { v_IPDataQFIx := v_ReceivedPdu1.U_Plane.SlotData.PduSduList.SdapSdu[i]; if ((v_IPDataQFIx == v_IPDataQFI2) and (v_IPDataCount_QFI2 < 10)) { v_IPDataCount_QFI2 := v_IPDataCount_QFI2 + 1 ; if (v_IPDataCount_QFI2 == 10) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2 - SS Received all 10 IP packets DRB using QF2 "); } } else if ((v_IPDataQFIx == v_IPDataQFI5) and (v_IPDataCount_QFI5 < 10)) { v_IPDataCount_QFI5 := v_IPDataCount_QFI5 + 1 ; if (v_IPDataCount_QFI5 == 10) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2 - SS Received all 10 IP packets using QF5 "); } } else { f_SetVerdict(fail,__FILE__,__LINE__,"Unexpected Packet is received at SS"); } } } v_IPDataCount := v_IPDataCount + received_packets; }while (v_IPDataCount < 20); // @siclog "Step 3" siclog@ // The SS transmits CounterCheck message with drb-Identity not included in the drb-CountMSB-InfoList v_CountList := {cs_38508_DRB_CountMSB_Info(v_FirstDRB, 1, 1), cs_38508_DRB_CountMSB_Info(v_ThirdDRB, 3, 3)}; SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList))); // @siclog "Step 4" siclog@ // Check: Does the UE send CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink // set to the value of TX_NEXT and RX_NEXT" 1 in drb-CountInfoList IE for each DRB established? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, {permutation (cr_DRB_CountInfo(v_FirstDRB, 9, 9), //@sic R5s200978 sic@ cr_DRB_CountInfo(v_SecondDRB, 9, 9), //@sic R5s200978 sic@ cr_DRB_CountInfo(v_ThirdDRB, 384, 384))}))); //@sic R5s200733 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); }// end of f_TC_8_1_5_4_1_NR5GC_TestBody