3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210873

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1878 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC RRC test case 8.1.6.1.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.6.1.4.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.6.1.4.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

3	Branches executed	9

4	Execution Log Files	9

4.1	HiSilicon Balong 5000	9

5	References	9

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.4.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.6.1.4.4.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8450+SDX65

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_8_1_6_1_4_4_NR5GC() |
| --- | --- |
| Reason for change | In order for the UE to send ULInformationTransfer with DEREGISTRATION REQUEST at switch off in postamble, the SRB2 needs to be resumed after the reestablishment procedure. If SRB2 is still suspended, the UE will not send UlInformationTransfer on SRB1. From TS 38.331, section 6.2.2: – ULInformationTransfer The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information. Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resume |
| Summary of change | Expect postamble signalling on SRB2 instead of SRB1 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_1_6_1_4_4_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment f_NR5GC_Init(NR_2); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); // Max Power level for FR2 is FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -80, -80); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_4_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); }

After Change

function f_TC_8_1_6_1_4_4_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment f_NR5GC_Init(NR_2); //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80); // Max Power level for FR2 is FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -80, -80); // Max Power level for FR2 is FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_6_1_4_4_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_CONNECTED_3A); }

  - Change 1

| Function name | f_TC_8_1_6_1_4_4_NR5GC() |
| --- | --- |
| Reason for change | - In order for the UE to send ULInformationTransfer with DEREGISTRATION REQUEST at switch off in postamble, the SRB2 needs to be resumed after the reestablishment procedure. If SRB2 is still suspended, the UE will not send UlInformationTransfer on SRB1. From TS 38.331, section 6.2.2: – ULInformationTransfer The ULInformationTransfer message is used for the uplink transfer of NAS or non-3GPP dedicated information. Signalling radio bearer: SRB2 or SRB1 (only if SRB2 not established yet). If SRB2 is suspended, the UE does not send this message until SRB2 is resume |
| Summary of change | Use function f_NR5GC_RRCReestablishment_SON_MDT() which includes f_NR5GC_RRCReestablishment_Steps8To13(), to re-establish SRB2 and DRB. Note : An associated prose CR on 38.523-1 will be raised at RAN5#92. |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle and implemented according to R5-215681. Please see MCC160 implementation below. |

Before Change

function fl_TC_8_1_6_1_4_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell2, -80, -80) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 3" siclog@ //The SS waits for 2sec (T300 expiry). f_Delay(2.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit, true_, omit)))); //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 parameters according to the row "T1"in table 8.1.6.1.4.4.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 13" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2. f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell2, otherFailure); //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. //Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_CEF; f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, nr_Cell2, -, -, valueof(v_RRCReestablishmentComplete_v1610_IEs)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); }

After Change

function fl_TC_8_1_6_1_4_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), // Power level for FR2 is FFS cs_NR_CellPower(nr_Cell2, -80, -80) // Power level for FR2 is FFS }; //@siclog "Step 1" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 3" siclog@ //The SS waits for 2sec (T300 expiry). f_Delay(2.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit, true_, omit)))); //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 parameters according to the row "T1"in table 8.1.6.1.4.4.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 13" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2. f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell2, otherFailure); //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, nr_Cell2, connEstFailInfoAvailable); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); }

MCC160 proposed implementation

function fl_TC_8_1_6_1_4_4_TestBody() runs on NR5GC_PTC

{

……

//@siclog "Step 13" siclog@

//The UE transmits an RRCReestablishmentRequest message on Cell 2.

f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell2, otherFailure);

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReestablishment message on Cell 2.

//Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2?

v_RRCReestablishmentComplete_v1610_IEs := {

ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(-, -, -, true_, true_), // @sic R5-215681 sic@

nonCriticalExtension := *

};

f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, nr_Cell2, -, -, valueof(v_RRCReestablishmentComplete_v1610_IEs));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Steps 16-17" siclog@

//@sic R5-215681, R5s210873 sic@

//The SS transmits an RRCReconfiguration message to establish SRB2 and DRB.

//The UE transmits an RRCReconfigurationtComplete message.

f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2);

}

- Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

- Execution Log Files

  - Qualcomm SM8450+SDX65

The Qualcomm SM8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_6_1_4_4_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_4_4_PIXIT.xml

The Qualcomm SM8450+SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_8_1_6_1_4_4_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_8_1_6_1_4_4_PICS.txt

- References

| [1] | R5s210874 : NR5GC FR1 : Supporting information for addition of NR5GC RRC test case 8.1.6.1.4.4 |
| --- | --- |