3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210879

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0739 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.15 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.15 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Qualcomm SM8450+SDX65	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.15 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.15.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM8450+SDX65

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes documented in R5s210877 are required and applied.

## Change 1

| Function name | f_IMS_MOCallSetup_Video_Step2() |
| --- | --- |
| Reason for change | This testcase is without preconditions at both originating UE and terminating UE, hence the SIP INVITE sent by the UE for the MO Video call will not preconditions tag in the Supported header, and as such, should not be checked in the TTCN. |
| Summary of change | Changed the expected value of preconditions tag in supported header based on a variable passed in the function. Note : An associated prose CR on 34.229-5 will be raised at RAN5#92 |
| TTCN module | IMS_Procedures_Video.ttcn |
| MCC160 Comment | Accepted according to R5-214386 |

Before Change

function f_IMS_MOCallSetup_Video_Step2(VideoCallTypeMO_Type p_VideoCallType, template SipUrl p_OtherCalleeUri := omit, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only, boolean p_WithPreconditions := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5w140112: return type changed to IMS_InviteRequestWithSdp_Type to carry decoded SDP offer */ /* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */ /* @sic R5-211433: p_AudioCodecList, p_VideoCodecList sic@ */ /* @sic R5-213070: p_WithPreconditions sic@ */ var template (present) SDP_Message v_SDP_Message_Step2; var INVITE_A_2_1_Context_Type v_Context; var IMS_DATA_REQ v_IMS_INVITE_REQ; var SDP_Message v_SDP_Message; var boolean v_A11 := true; /* @sic R5-176371: INVITE for creating a video call sic@ */ var boolean v_CheckForbiddenCodecParameters := f_IMS_PTC_RanTypeIsNR5GC(); var boolean v_PreconditionsSupported := true; /* @sic R5-213367: Supported header field shall indicate preconditions even for A.15.2; in all other cases p_WithPreconditions is true anyway sic@ */ var OptionTag_List v_AdditionalOptionTagsForSupported := f_IMS_AdditionalOptionTagsForSupported(v_PreconditionsSupported); var IMS_SDP_AudioCodec_Type v_AudioCodec; select (p_VideoCallType) { case (SetupVideoCall) { v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(p_AudioCodecList, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A4; } case (AddVideo) { v_AudioCodec := p_AudioCodecList[0]; /* @sic R5-213073 sic@ */ v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(v_AudioCodec, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A5; } } … … }

After Change

function f_IMS_MOCallSetup_Video_Step2(VideoCallTypeMO_Type p_VideoCallType, template SipUrl p_OtherCalleeUri := omit, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only, boolean p_WithPreconditions := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5w140112: return type changed to IMS_InviteRequestWithSdp_Type to carry decoded SDP offer */ /* @sic R5s150204: p_OtherCalleeUri charstring -> SipUrl sic@ */ /* @sic R5-211433: p_AudioCodecList, p_VideoCodecList sic@ */ /* @sic R5-213070: p_WithPreconditions sic@ */ var template (present) SDP_Message v_SDP_Message_Step2; var INVITE_A_2_1_Context_Type v_Context; var IMS_DATA_REQ v_IMS_INVITE_REQ; var SDP_Message v_SDP_Message; var boolean v_A11 := true; /* @sic R5-176371: INVITE for creating a video call sic@ */ var boolean v_CheckForbiddenCodecParameters := f_IMS_PTC_RanTypeIsNR5GC(); var boolean v_PreconditionsSupported := p_WithPreconditions; /* @sic R5-213367: Supported header field shall indicate preconditions even for A.15.2; in all other cases p_WithPreconditions is true anyway sic@ */ var OptionTag_List v_AdditionalOptionTagsForSupported := f_IMS_AdditionalOptionTagsForSupported(v_PreconditionsSupported); var IMS_SDP_AudioCodec_Type v_AudioCodec; select (p_VideoCallType) { case (SetupVideoCall) { v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(p_AudioCodecList, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A4; } case (AddVideo) { v_AudioCodec := p_AudioCodecList[0]; /* @sic R5-213073 sic@ */ v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(v_AudioCodec, p_VideoCodecList, p_WithPreconditions); v_Context := A_2_1_A5; } } … … }

# Branches executed

The test case was executed on NR band n41 using ciphering algorithm nea1 and integrity algorithm nea2

# Execution Log Files

## Qualcomm SM8450+SDX65

The Qualcomm SM8450+SDX65  UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_15_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_15_PIXIT.xml

# References

| [1] | R5s210880: Supporting information for addition of IMS 5GS call control test case 7.15 |
| --- | --- |