3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210949

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1892 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR capability functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If a UE is supporting both FR1 And FR2 bands and if sends “csi_RSRP_AndRSRQ_MeasWithSSB" IE in both fr1-Add-UE-NR-Capabilities and fr2-Add-UE-NR-Capabilities the capability cecking test cases will fail with the message “reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the TTCN checking to allow the UE to report csi_RSRP_AndRSRQ_MeasWithSSB for both FR1 and FR2 bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB(UE_NR_Capability p_ReceivedCapabilityMsg) |
| --- | --- |
| Reason for change | If a UE is supporting both FR1 And FR2 bands and if sends “csi_RSRP_AndRSRQ_MeasWithSSB" IE in both fr1-Add-UE-NR-Capabilities and fr2-Add-UE-NR-Capabilities the capability cecking test cases will fail with the message “reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent” |
| Summary of change | Correct the TTCN checking to allow the UE to report csi_RSRP_AndRSRQ_MeasWithSSB for both FR1 and FR2 bands. |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted in principle. See MCC160 Implementation below. Similar update for checks on Dual mode has been done in functions: fl_Check_intraAndInterF_MeasAndReport(p_ReceivedCapabilityMsg); // Note 1 fl_Check_eventA_MeasAndReport(p_ReceivedCapabilityMsg); // Note 2 fl_Check_skipUplinkTxDynamic(p_ReceivedCapabilityMsg); // Note 4 fl_Check_logicalChannelSR_DelayTimer(p_ReceivedCapabilityMsg); // Note 5 fl_Check_longDRX_Cycle(p_ReceivedCapabilityMsg); // Note 6 fl_Check_shortDRX_Cycle(p_ReceivedCapabilityMsg); // Note 7 |

Before Change

function fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5s190471 R5-197070 R5-206430 sic@ var boolean v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersFRX_Diff v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common; var MeasAndMobParametersFRX_Diff v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; var MeasAndMobParametersFRX_Diff v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; …………………………. if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB) or v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB or v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Single frequency range UE and reported IEs are inconsistent"); } else if (f_NR_FreqRangeFr1AndFR2() and ((v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB) and (v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB))) { // fail case on both frequency ranges supported f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common.csi_RSRP_AndRSRQ_MeasWithSSB) or ispresent(v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB) or ispresent(v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_csi_RSRP_AndRSRQ_MeasWithSSB to false is inconsistent with reported capabilities for Rel-15 and above UE"); } } ……………………. }

After Change

function fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5s190471 R5-197070 R5-206430 sic@ var boolean v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersFRX_Diff v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common; var MeasAndMobParametersFRX_Diff v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; var MeasAndMobParametersFRX_Diff v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; …………………. if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB) or v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB or v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Single frequency range UE and reported IEs are inconsistent"); } else if(f_NR_FreqRangeFr1AndFR2()) { if (v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB and v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB) { // fail case on both frequency ranges supported f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } else if ((v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB or v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB) and v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB ) { f_SetVerdict(fail, __FILE__, __LINE__, "Fr1/Fr2 frequency range UE and reported IEs are inconsistent"); } } } ………………… }

MCC160 implementation

function fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB

function fl_Check_csi_RSRP_AndRSRQ_MeasWithSSB(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5s190471 R5-197070 R5-206430 R5s210949 sic@ var boolean v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB := false; var boolean v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersFRX_Diff v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common; var MeasAndMobParametersFRX_Diff v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; var MeasAndMobParametersFRX_Diff v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB; if (ispresent(v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff)) { v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common := v_ReceivedCapabilityMsg.measAndMobParameters.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := v_ReceivedCapabilityMsg.fr1_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if (ispresent(v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) { v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := v_ReceivedCapabilityMsg.fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff; } if(pc_csi_RSRP_AndRSRQ_MeasWithSSB) { // check indication of MeasAndMobParametersFRX_Diff_Common if (ispresent(v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common.csi_RSRP_AndRSRQ_MeasWithSSB)) { v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB := true; } // check indication of FR1 IE if (ispresent(v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB)) { v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB := true; } // check indication of FR2 IE if (ispresent(v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB)) { v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB := true; } if ((not f_NR_FreqRangeFr1AndFR2()) and ((not v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB) or v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB or v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB)) { // fail cases on Single frequency range f_SetVerdict(fail, __FILE__, __LINE__, "Reported IEs are inconsistent with pc_csi_RSRP_AndRSRQ_MeasWithSSB on Single Mode"); } else if(f_NR_FreqRangeFr1AndFR2()) //FR1&FR2 case { if (not v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB) { if (v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB and v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB) // fail case: both FR1 and FR2 capabilities are reported and IE MeasAndMobParamFRX_Diff is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FR1/FR2_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB) and (not v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB)) // inconclusive case: UE does not report any IE for csi_RSRP_AndRSRQ_MeasWithSSB but pc_csi_RSRP_AndRSRQ_MeasWithSSB is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithSSB is set but csi_RSRP_AndRSRQ_MeasWithSSB IEs are not reported by the UE"); } } if (v_MeasAndMobParamFRX_Diff_csi_RSRP_AndRSRQ_MeasWithSSB and (v_FR1_Add_csi_RSRP_AndRSRQ_MeasWithSSB or v_FR2_Add_Cap_csi_RSRP_AndRSRQ_MeasWithSSB) ) // fail cases: if IE MeasAndMobParamFRX_Diff is reported but FR1 and/or FR2 capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "FR1/FR2 frequency range UE and reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FR1/FR2 IEs are reported by the UE if (ispresent(v_MeasAndMobParamFRX_csi_RSRP_AndRSRQ_MeasWithSSB_Common.csi_RSRP_AndRSRQ_MeasWithSSB) or ispresent(v_FR1_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB) or ispresent(v_FR2_Add_UE_NR_Cap_csi_RSRP_AndRSRQ_MeasWithSSB.csi_RSRP_AndRSRQ_MeasWithSSB)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_csi_RSRP_AndRSRQ_MeasWithSSB to false is inconsistent with reported capabilities for Rel-15 and above UE"); } } }

function fl_Check_intraAndInterF_MeasAndReport

function fl_Check_intraAndInterF_MeasAndReport(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5-197070 R5s191050 R5-206430 R5s210949 sic@ var boolean v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport := false; var boolean v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD := false; var boolean v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersXDD_Diff v_IntraAndInterF_MeasAndReport_Common; var MeasAndMobParametersXDD_Diff v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport; var MeasAndMobParametersXDD_Diff v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport; … // Error cases if the feature is set if ((not f_NR_DualModeCheck()) and (not v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport) or v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) { // fail cases on Single mode f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-CapabilityAddXDD IEs are inconsistent with pc_eventA_MeasAndReport on Single Mode"); } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport) { if (v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD and v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) // fail case: both FDD and TDD capabilities are reported and IE UE_NR_Cap_intraAndInterF_MeasAndReport_XDD is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) and (not v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD)) // inconclusive case: UE does not report any IE for intraAndInterF_MeasAndReport but pc_intraAndInterF_MeasAndReport is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_intraAndInterF_MeasAndReport is set but intraAndInterF_MeasAndReport IEs are not reported by the UE"); } } if (v_MeasAndMobParamXDD_Diff_intraAndInterF_MeasAndReport and (v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport_XDD) ) // fail cases: if IE intraAndInterF_MeasAndReport is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_IntraAndInterF_MeasAndReport_Common.intraAndInterF_MeasAndReport) or ispresent(v_FDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport.intraAndInterF_MeasAndReport) or ispresent(v_TDD_Add_UE_NR_Cap_intraAndInterF_MeasAndReport.intraAndInterF_MeasAndReport)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_intraAndInterF_MeasAndReport to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

function fl_Check_eventA_MeasAndReport

function fl_Check_eventA_MeasAndReport(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5s190471 R5s191050 R5-206430 R5s210949 sic@ var boolean v_MeasAndMobParamXDD_Diff_Common := false; var boolean v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD := false; var boolean v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MeasAndMobParametersXDD_Diff v_EventA_MeasAndReport_Common; var MeasAndMobParametersXDD_Diff v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport; var MeasAndMobParametersXDD_Diff v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport; … if ((not f_NR_DualModeCheck()) and (not v_MeasAndMobParamXDD_Diff_Common) or v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD) { // fail cases on Single mode f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-CapabilityAddXDD IEs are inconsistent with pc_eventA_MeasAndReport on Single Mode"); } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_MeasAndMobParamXDD_Diff_Common) { if (v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD and v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD) // fail case: both FDD and TDD capabilities are reported and IE in measAndMobParameters.measAndMobParametersXDD_Diff is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD) and (not v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD)) // inconclusive case: UE does not report any IE for eventA_MeasAndReport but pc_eventA_MeasAndReport is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_eventA_MeasAndReport is set but eventA_MeasAndReport IEs are not reported by the UE"); } } if (v_MeasAndMobParamXDD_Diff_Common and (v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD or v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport_XDD) ) // fail cases: if IE in measAndMobParameters.measAndMobParametersXDD_Diff is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_EventA_MeasAndReport_Common.eventA_MeasAndReport) or ispresent(v_FDD_Add_UE_NR_Cap_eventA_MeasAndReport.eventA_MeasAndReport) or ispresent(v_TDD_Add_UE_NR_Cap_eventA_MeasAndReport.eventA_MeasAndReport)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_eventA_MeasAndReport to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

function fl_Check_skipUplinkTxDynamic

function fl_Check_skipUplinkTxDynamic(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5-197070 R5-206430 R5s210949 sic@ var boolean v_Mac_ParametersXDD_Diff_skipUplinkTxDynamic := false; var boolean v_FDD_Add_UE_NR_Cap_skipUplinkTxDynamic := false; var boolean v_TDD_Add_UE_NR_Cap_skipUplinkTxDynamic := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MAC_ParametersXDD_Diff v_SkipUplinkTxDynamic_Common; var MAC_ParametersXDD_Diff v_FDD_Add_UE_NR_Cap; var MAC_ParametersXDD_Diff v_TDD_Add_UE_NR_Cap; … if ((not f_NR_DualModeCheck()) and ((not v_Mac_ParametersXDD_Diff_skipUplinkTxDynamic) or (v_FDD_Add_UE_NR_Cap_skipUplinkTxDynamic or v_TDD_Add_UE_NR_Cap_skipUplinkTxDynamic))) { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-Capabilities IEs are inconsistent with pc_skipUplinkTxDynamic on Single Mode"); // fail case single mode } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Mac_ParametersXDD_Diff_skipUplinkTxDynamic) { if (v_FDD_Add_UE_NR_Cap_skipUplinkTxDynamic and v_TDD_Add_UE_NR_Cap_skipUplinkTxDynamic) // fail case: both FDD and TDD capabilities are reported and IE in mac_Parameters.mac_ParametersXDD_Diff is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_skipUplinkTxDynamic) and (not v_TDD_Add_UE_NR_Cap_skipUplinkTxDynamic)) // inconclusive case: UE does not report any IE for skipUplinkTxDynamic but pc_skipUplinkTxDynamic is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_skipUplinkTxDynamic is set but skipUplinkTxDynamic IEs are not reported by the UE"); } } if (v_Mac_ParametersXDD_Diff_skipUplinkTxDynamic and (v_FDD_Add_UE_NR_Cap_skipUplinkTxDynamic or v_TDD_Add_UE_NR_Cap_skipUplinkTxDynamic) ) // fail cases: if IE in mac_Parameters.mac_ParametersXDD_Diff is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_SkipUplinkTxDynamic_Common.skipUplinkTxDynamic) or ispresent(v_FDD_Add_UE_NR_Cap.skipUplinkTxDynamic) or ispresent(v_TDD_Add_UE_NR_Cap.skipUplinkTxDynamic)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_skipUplinkTxDynamic to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

function fl_Check_logicalChannelSR_DelayTimer

function fl_Check_logicalChannelSR_DelayTimer(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5-197070 R5-206430 R5s210949 sic@ var boolean v_Mac_ParametersXDD_Diff_logicalChannelSR_DelayTimer := false; var boolean v_FDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer := false; var boolean v_TDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MAC_ParametersXDD_Diff v_LogicalChannelSR_DelayTimer_Common; var MAC_ParametersXDD_Diff v_FDD_Add_UE_NR_Cap; var MAC_ParametersXDD_Diff v_TDD_Add_UE_NR_Cap; … if ((not f_NR_DualModeCheck()) and (not v_Mac_ParametersXDD_Diff_logicalChannelSR_DelayTimer) or ((v_FDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer) or (v_TDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer))) { //@sic R5s200160 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-Capabilities IEs are inconsistent with pc_logicalChannelSR_DelayTimer Single Mode"); // Fail case on Single Mode } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Mac_ParametersXDD_Diff_logicalChannelSR_DelayTimer) { if (v_FDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer and v_TDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer) // fail case: both FDD and TDD capabilities are reported and IE in mac_Parameters.mac_ParametersXDD_Diff is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer) and (not v_TDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer)) // inconclusive case: UE does not report any IE for logicalChannelSR_DelayTimer but pc_logicalChannelSR_DelayTimer is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_logicalChannelSR_DelayTimer is set but logicalChannelSR_DelayTimer IEs are not reported by the UE"); } } if (v_Mac_ParametersXDD_Diff_logicalChannelSR_DelayTimer and (v_FDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer or v_TDD_Add_UE_NR_Cap_logicalChannelSR_DelayTimer) ) // fail cases: if IE in mac_Parameters.mac_ParametersXDD_Diff is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_LogicalChannelSR_DelayTimer_Common.logicalChannelSR_DelayTimer) or ispresent(v_FDD_Add_UE_NR_Cap.logicalChannelSR_DelayTimer) or ispresent(v_TDD_Add_UE_NR_Cap.logicalChannelSR_DelayTimer)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_logicalChannelSR_DelayTimer to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }

function fl_Check_longDRX_Cycle

function fl_Check_longDRX_Cycle(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5-197070 R5-206430 R5s210949 sic@ var boolean v_Mac_ParametersXDD_Diff_longDRX_Cycle := false; var boolean v_FDD_Add_UE_NR_Cap_longDRX_Cycle := false; var boolean v_TDD_Add_UE_NR_Cap_longDRX_Cycle := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MAC_ParametersXDD_Diff v_LongDRX_Cycle_Common; var MAC_ParametersXDD_Diff v_FDD_Add_UE_NR_Cap; var MAC_ParametersXDD_Diff v_TDD_Add_UE_NR_Cap; … if ((not f_NR_DualModeCheck()) and ((not v_Mac_ParametersXDD_Diff_longDRX_Cycle) or (v_FDD_Add_UE_NR_Cap_longDRX_Cycle or v_TDD_Add_UE_NR_Cap_longDRX_Cycle))) { //@sic R5s200160 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD UE-NR-Capabilities IEs are inconsistent with pc_longDRX_Cycle on Single Mode"); // Fail case on Single Mode } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Mac_ParametersXDD_Diff_longDRX_Cycle) { if (v_FDD_Add_UE_NR_Cap_longDRX_Cycle and v_TDD_Add_UE_NR_Cap_longDRX_Cycle) // fail case: both FDD and TDD capabilities are reported and IE mac_Parameters.mac_ParametersXDD_Diff is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_longDRX_Cycle) and (not v_TDD_Add_UE_NR_Cap_longDRX_Cycle)) // inconclusive case: UE does not report any IE for longDRX_Cycle but pc_longDRX_Cycle is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_longDRX_Cycle is set but longDRX_Cycle IEs are not reported by the UE"); } } if (v_Mac_ParametersXDD_Diff_longDRX_Cycle and (v_FDD_Add_UE_NR_Cap_longDRX_Cycle or v_TDD_Add_UE_NR_Cap_longDRX_Cycle) ) // fail cases: if IE mac_Parameters.mac_ParametersXDD_Diff is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_LongDRX_Cycle_Common.longDRX_Cycle) or ispresent(v_FDD_Add_UE_NR_Cap.longDRX_Cycle) or ispresent(v_TDD_Add_UE_NR_Cap.longDRX_Cycle)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_longDRX_Cycle to false is inconsistent with reported capabilities for Rel-15 and above UE"); } } }

function fl_Check_shortDRX_Cycle

function fl_Check_shortDRX_Cycle(UE_NR_Capability p_ReceivedCapabilityMsg) { //@sic R5-201761 R5-197070 R5-206430 R5s210949 sic@ var boolean v_Mac_ParametersXDD_Diff_shortDRX_Cycle := false; var boolean v_FDD_Add_UE_NR_Cap_shortDRX_Cycle := false; var boolean v_TDD_Add_UE_NR_Cap_shortDRX_Cycle := false; var UE_NR_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var MAC_ParametersXDD_Diff v_ShortDRX_Cycle_Common; var MAC_ParametersXDD_Diff v_FDD_Add_UE_NR_Cap; var MAC_ParametersXDD_Diff v_TDD_Add_UE_NR_Cap; … if ((not f_NR_DualModeCheck()) and ((not v_Mac_ParametersXDD_Diff_shortDRX_Cycle) or (v_FDD_Add_UE_NR_Cap_shortDRX_Cycle or v_TDD_Add_UE_NR_Cap_shortDRX_Cycle))) { //@sic R5s200160 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "Reporting of FDD/TDD UE-NR-Capabilities IEs are inconsistent with pc_shortDRX_Cycle on Single Mode"); // Fail case on Dual mode } else if(f_NR_DualModeCheck()) // case dual mode { if (not v_Mac_ParametersXDD_Diff_shortDRX_Cycle) { if (v_FDD_Add_UE_NR_Cap_shortDRX_Cycle and v_TDD_Add_UE_NR_Cap_shortDRX_Cycle) // fail case: both FDD and TDD capabilities are reported and IE mac_ParametersXDD_Diff.shortDRX_Cycle is not { f_SetVerdict(fail, __FILE__, __LINE__, "reporting of FDD/TDD_Add_UE_NR_Capabilities IEs are inconsistent"); } if ((not v_FDD_Add_UE_NR_Cap_shortDRX_Cycle) and (not v_TDD_Add_UE_NR_Cap_shortDRX_Cycle)) // inconclusive case: UE does not report any IE for longDRX_Cycle but pc_shortDRX_Cycle is set { f_SetVerdict(inconc, __FILE__, __LINE__, "pc_shortDRX_Cycle is set but longDRX_Cycle IEs are not reported by the UE"); } } if (v_Mac_ParametersXDD_Diff_shortDRX_Cycle and (v_FDD_Add_UE_NR_Cap_shortDRX_Cycle or v_TDD_Add_UE_NR_Cap_shortDRX_Cycle) ) // fail cases: if IE mac_ParametersXDD_Diff.shortDRX_Cycle is reported but FDD and/or TDD capabilities are also reported { f_SetVerdict(fail, __FILE__, __LINE__, "reported IEs are inconsistent"); } } } else { // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE if (ispresent(v_ShortDRX_Cycle_Common.shortDRX_Cycle) or ispresent(v_FDD_Add_UE_NR_Cap.shortDRX_Cycle) or ispresent(v_TDD_Add_UE_NR_Cap.shortDRX_Cycle)) { f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_shortDRX_Cycle to false is inconsistent with reported capabilities for a Rel-15 and above UE"); } } }