3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211168

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2000 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcases 7.1.3.2.x. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, 1) In the function : fl_EUTRA_PDCP_Integrity, preamble is called with TESTModeA_ON , but as per the prose For EN_DC , preamble should be configured without message condition UE TEST LOOP MODE A 2) In the function : f_NR_RRC_PSCellChange_SRB_NR_PDCP , within the case ‘NR_PDCP_SRB1_2_SRB3’ , SRB3 is configured with Sdap as None , but as per 38.523-3 Sdap should be set to Omit in case of ENDC 3) In the function : f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP Default value of v_DrbConfigAtSS is assigned only with DRB1 , which will be released and reconfigured during the Intra Lte HO Procedure , but DRB8 which is 2nd Pdn DRB at EUTRA PTC is not taken into the account. Hence within the case ‘NR_PDCP_SRB1_2_SRB3/ NR_PDCP_SRB1_2’ both Drb1 and Drb8 are considered |  |  |  |  |  |  |  |  |
| Summary of change: |  | - f_EUTRA38_ENDC_Preamble is called with ‘TESTMode_OFF’ instead of TESTModeA_ON to be in align with Prose - In the function f_NR_RRC_PSCellChange_SRB_NR_PDCP , within the case ‘NR_PDCP_SRB1_2_SRB3’ , Sdap is set to Omit - In the function , f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP within the case ‘NR_PDCP_SRB1_2_SRB3/ NR_PDCP_SRB1_2 ‘ Both DRB1 and DRB8 are filled for v_DrbConfigAtSS , so that both these DRB entries will be released and reconfigured during Intra Lte HO procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.1.ENDC, 7.1.3.2.2.ENDC, 7.1.3.2.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_EUTRA_PDCP_Integrity	3

1.2	Correction to function f_NR_RRC_PSCellChange_SRB_NR_PDCP	6

1.3	Correction to function f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP	9

1.4	Correction to module EUTRA38_Handover	12

# Corrections required

## Correction to function fl_EUTRA_PDCP_Integrity

| Function name | fl_EUTRA_PDCP_Integrity |
| --- | --- |
| Reason for change | In the current TTCN implementation, 1) In the function : fl_EUTRA_PDCP_Integrity, preamble is called with TESTModeA_ON , but as per the prose For EN_DC , preamble should be configured without message condition UE TEST LOOP MODE A |
| Summary of change | - f_EUTRA38_ENDC_Preamble is called with ‘TESTMode_OFF’ instead of TESTModeA_ON to be in align with Prose |
| TTCN module | ttcn-3\ttcn\develop\ENDC\7_1_3\PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_EUTRA_PDCP_Integrity(IntegrityProtAlgorithm_Type p_AS_IntegrityProtAlgorithm) runs on EUTRA_5GS_PTC {//@sic R5-202614 sic@ var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2; if (pc_srb3){ v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3; } f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set integrity algorithm f_EUTRA_AS_IntegrityAlgorithm_Set(p_AS_IntegrityProtAlgorithm);//@sic R5-202614 sic@ //Preamble to enter UE in state 2A . NR PDCP is configured on SRB1 and SRB2 and SRB 3 //@sic R5-195339 sic@ f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeA_ON, MCG_SCG, RRC_CONNECTED);//@sic R5s190476 sic@ ch 4.8 f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (eutra_Cell1, v_SecondaryConfig,1); //@sic R5s190476 sic@ ch 4.9 f_EUTRA_TestBody_Set(true); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 1a1-1a2" siclog@ //@sic R5-212370 sic@ //IF Connectivity is EN-DC or NGEN-DC, the SS sends EUTRA RRC UECapabilityEnquiry including RAT-Type eutra message to the UE. //Check: Does the UE send a EUTRA RRC UECapabilityInformation message? //@sic R5s201545 sic@ //@sic R5-206327 sic@ f_EUTRA_CheckUeCapabilityEnquiry(eutra_Cell1); if (pc_srb3){ //@siclog "Step 2a4" siclog@ //Does the UE send SCGFailureInformationNR with failureType as srb3-IntegrityFailure on SRB1? SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(srb3_IntegrityFailure)))); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); } f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204475 sic@ // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

function fl_EUTRA_PDCP_Integrity(IntegrityProtAlgorithm_Type p_AS_IntegrityProtAlgorithm) runs on EUTRA_5GS_PTC {//@sic R5-202614 sic@ var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2; if (pc_srb3){ v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3; } f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set integrity algorithm f_EUTRA_AS_IntegrityAlgorithm_Set(p_AS_IntegrityProtAlgorithm);//@sic R5-202614 sic@ //Preamble to enter UE in state 2A . NR PDCP is configured on SRB1 and SRB2 and SRB 3 //@sic R5-195339 sic@ f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_SCG, RRC_CONNECTED);//@sic R5s190476 sic@ ch 4.8 //Test mode to be Off for ENDC f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (eutra_Cell1, v_SecondaryConfig,1); //@sic R5s190476 sic@ ch 4.9 f_EUTRA_TestBody_Set(true); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 1a1-1a2" siclog@ //@sic R5-212370 sic@ //IF Connectivity is EN-DC or NGEN-DC, the SS sends EUTRA RRC UECapabilityEnquiry including RAT-Type eutra message to the UE. //Check: Does the UE send a EUTRA RRC UECapabilityInformation message? //@sic R5s201545 sic@ //@sic R5-206327 sic@ f_EUTRA_CheckUeCapabilityEnquiry(eutra_Cell1); if (pc_srb3){ //@siclog "Step 2a4" siclog@ //Does the UE send SCGFailureInformationNR with failureType as srb3-IntegrityFailure on SRB1? SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(srb3_IntegrityFailure)))); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); } f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204475 sic@ // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Correction to function f_NR_RRC_PSCellChange_SRB_NR_PDCP

| Function name | f_NR_RRC_PSCellChange_SRB_NR_PDCP |
| --- | --- |
| Reason for change | In the current TTCN implementation, 1) In the function : f_NR_RRC_PSCellChange_SRB_NR_PDCP , within the case ‘NR_PDCP_SRB1_2_SRB3’ , SRB3 is configured with having Sdap as None , but as per 38.523-3 Sdap should be set to Omit in case of ENDC |
| Summary of change | 1) In the function f_NR_RRC_PSCellChange_SRB_NR_PDCP , within the case ‘NR_PDCP_SRB1_2_SRB3’ , Sdap is set to Omit |
| TTCN module | ttcn\develop\ENDC\Common_NR\NR_Handover_ENDC.ttcn |
| MCC160 Comment | Accepted, implemented as shown below |

Before Change:

function f_NR_RRC_PSCellChange_SRB_NR_PDCP(NR_CellId_Type p_NR_CellId, ENDC_SRB_DRB_Configuration_Type p_RBConfig ) runs on ENDC_NR_PTC { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig:= f_NR_GetRACHProcedureConfigPSCell( p_NR_CellId, -); var octetstring v_OctetData; var integer v_MCG_Id; var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RadioBearerList_Type v_SS_RadioBearerList; var template (value) NR_RadioBearerList_Type v_SS_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; var template (value) SRB_ToAddModList v_SRB_ToAddModList_SRB1SRB2 := { cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1), cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2) }; var template (value) NR_RadioBearer_Type v_RadioBearerDRB2 := f_NR_GetRadioBearerConfig_FullAM_DRB2(p_NR_CellId); select (p_RBConfig ) { case(NR_PDCP_SRB1_2){ v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, false); //DRB2 v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList_SRB1SRB2, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_SCGConfig := f_NR_CellGroupConfig_PSCellChange( p_NR_CellId, -, false); v_SS_RadioBearerList := {v_RadioBearerDRB2}; } case(NR_PDCP_SRB1_2_SRB3){ v_SCGConfig := f_NR_CellGroupConfig_PSCellChange(p_NR_CellId, -, true); v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, true); //DRB2 + SRB3 v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList_SRB1SRB2, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_SCGConfig := f_NR_CellGroupConfig_PSCellChange(p_NR_CellId, -, true); // Configure SRB3 in SS v_SS_RadioBearerList[0] := cs_NR_SS_SRB3_Config; // @sic R5s201350 sic@ v_SS_RadioBearerList[1] := f_NR_GetRadioBearerConfig_FullAM_DRB2(p_NR_CellId); } << SKIPPED CODE >> }

After Change:

function f_NR_RRC_PSCellChange_SRB_NR_PDCP(NR_CellId_Type p_NR_CellId, ENDC_SRB_DRB_Configuration_Type p_RBConfig ) runs on ENDC_NR_PTC { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig:= f_NR_GetRACHProcedureConfigPSCell( p_NR_CellId, -); var octetstring v_OctetData; var integer v_MCG_Id; var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RadioBearerList_Type v_SS_RadioBearerList; var template (value) NR_RadioBearerList_Type v_SS_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; var template (value) SRB_ToAddModList v_SRB_ToAddModList_SRB1SRB2 := { cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1), cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2) }; var template (value) NR_RadioBearer_Type v_RadioBearerDRB2 := f_NR_GetRadioBearerConfig_FullAM_DRB2(p_NR_CellId); select (p_RBConfig ) { case(NR_PDCP_SRB1_2){ v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, false); //DRB2 v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList_SRB1SRB2, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_SCGConfig := f_NR_CellGroupConfig_PSCellChange( p_NR_CellId, -, false); v_SS_RadioBearerList := {v_RadioBearerDRB2}; } case(NR_PDCP_SRB1_2_SRB3){ v_SCGConfig := f_NR_CellGroupConfig_PSCellChange(p_NR_CellId, -, true); v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, true); //DRB2 + SRB3 v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList_SRB1SRB2, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_SCGConfig := f_NR_CellGroupConfig_PSCellChange(p_NR_CellId, -, true); // Configure SRB3 in SS //v_SS_RadioBearerList[0] := cs_NR_SS_SRB3_Config; // @sic R5s201350 sic@ //WA#WI=878123 //SDAP has to be Omit for ENDC v_SS_RadioBearerList[0] := cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority), omit); //WA#WI=878123 //SDAP has to be Omit for ENDC v_SS_RadioBearerList[1] := f_NR_GetRadioBearerConfig_FullAM_DRB2(p_NR_CellId); } <<< SKIPPED CODE >>> }

MCC160 proposed implementation

template (value) NR_RadioBearer_Type cs_NR_SS_SRB3_Config(template (omit)  SDAP_Configuration_Type p_Sdap := omit) :=

/* @status    APPROVED (ENDC) */

//@sic R5s211168: added p_Sdap sic@

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3,

cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB),

cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority),

p_Sdap

);

## Correction to function f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP

| Function name | f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP |
| --- | --- |
| Reason for change | In the current TTCN implementation, In the function : f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP Default value of v_DrbConfigAtSS is passed only with DRB1 , which will be released and reconfigured during the Intra Lte HO Procedure , but DRB8 which is 2nd Pdn DRB at EUTRA PTC is not taken into the account. Hence within the case ‘NR_PDCP_SRB1_2_SRB3/ NR_PDCP_SRB1_2’ both Drb1 and Drb8 are considered |
| Summary of change | In the function , f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP within the case ‘NR_PDCP_SRB1_2_SRB3/ NR_PDCP_SRB1_2 ‘ Both DRB1 and DRB8 are filled for v_DrbConfigAtSS , so that both these DRB entries will be released and reconfigured during Intra Lte HO procedure |
| TTCN module | ttcn\develop\ENDC\Common_EUTRA\EUTRA38_Handover.ttcn |
| MCC160 Comment | Accepted in principle – see proposed implementation below |

Before Change:

function f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (EUTRA_CellId_Type p_CellId, ENDC_SRB_DRB_Configuration_Type p_SecondaryConfig, template (omit) integer p_SK_Counter := 0 ) runs on EUTRA_5GS_PTC { //@sic R5s190476 change 4.15: added p_SK_Counter sic@ var template (value) RadioBearerList_Type v_DrbConfigAtSS := cs_DrbConfigList_DRB1; //@sic R5s191169 R5s200051 change 1 sic@ var integer v_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG(); var template (value) octetstring v_NR_SCGConfig; var template (value) octetstring v_NR_RBConfig; var template (value) octetstring v_NR_RBConfig2; var octetstring v_OctetData; var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs; var SubFrameTiming_Type v_Timing; var template (value) RadioBearerList_Type v_SrbDrbList; //@sic R5s190476 change 4.2 sic@ var template (omit) DRB_ToAddModList v_Drb_ToAddModList := omit; //@sic R5s200570 change 2.3 sic@ var template (omit) DRB_ToReleaseList v_Drb_ToReleaseList := omit; //@sic R5s200570 change 2.3 sic@ select (p_SecondaryConfig) { case (NR_PDCP_MCG_SRB1_2) {// LTE config to configure NR PDCP on EUTRA PTC NR.send (cms_EUTRA_NR_OctetData (int2oct(v_DRB_Id, 1), "DRB ID")); // if changing MCG DRB need to tell NR the Id } } select (p_SecondaryConfig) { // if changing SRB 1/2 need to also update the ciphering/integrity algorithms to NR values case (NR_PDCP_SRB1_2, NR_PDCP_MCG_SRB1_2, NR_PDCP_SRB1_2_SRB3) { v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg"); f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3))); v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg"); f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3))); } } //Receive SCG, RBConfig and RBConfig2 from NR PTC v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB2"); v_SrbDrbList := { //@sic R5s190476 change 4.2 sic@ cds_SRB1_Config_NRPDCP, cds_SRB2_Config_NRPDCP }; select (p_SecondaryConfig) { case (NR_PDCP_SRB1_2, NR_PDCP_SRB1_2_SRB3) {// LTE config to configure NR PDCP on EUTRA PTC //@sic R5s190476 change 4.16 sic@ // do nothing as SRBs changed elsewhere } case (NR_PDCP_MCG_SRB1_2) {// LTE config to configure NR PDCP on EUTRA PTC EUTRA38_DRB! v_DrbConfigAtSS := { cs_RadioBearer_Reconfig_DRBwithNRPDCPonEUTRA(tsc_DRB8, cs_RLC_Configuration_AM, cs_MAC_Configuration_AM) }; //@sic R5s190554 change 4.22 sic@ //@sic R5s200051 change 1 sic@ /* @sic R5s200570: replaced p_AddRelDrb8 with p_Drb_ToReleaseList and p_DRB_ToAddModList sic@ */ v_Drb_ToAddModList := { cds_508_DRB_ToAddMod_Common(tsc_DRB8, omit, cs_508_RLC_Config_DRB_AM,cs_508_LogicalChannelConfig_DRB_AM)}; v_Drb_ToReleaseList := {tsc_DRB8}; } } << SKIPPED CODE >> }

After Change:

function f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (EUTRA_CellId_Type p_CellId, ENDC_SRB_DRB_Configuration_Type p_SecondaryConfig, template (omit) integer p_SK_Counter := 0 ) runs on EUTRA_5GS_PTC { //@sic R5s190476 change 4.15: added p_SK_Counter sic@ var template (value) RadioBearerList_Type v_DrbConfigAtSS := cs_DrbConfigList_DRB1; //@sic R5s191169 R5s200051 change 1 sic@ var integer v_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG(); var template (value) octetstring v_NR_SCGConfig; var template (value) octetstring v_NR_RBConfig; var template (value) octetstring v_NR_RBConfig2; var octetstring v_OctetData; var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs; var SubFrameTiming_Type v_Timing; var template (value) RadioBearerList_Type v_SrbDrbList; //@sic R5s190476 change 4.2 sic@ var template (omit) DRB_ToAddModList v_Drb_ToAddModList := omit; //@sic R5s200570 change 2.3 sic@ var template (omit) DRB_ToReleaseList v_Drb_ToReleaseList := omit; //@sic R5s200570 change 2.3 sic@ select (p_SecondaryConfig) { case (NR_PDCP_MCG_SRB1_2) {// LTE config to configure NR PDCP on EUTRA PTC NR.send (cms_EUTRA_NR_OctetData (int2oct(v_DRB_Id, 1), "DRB ID")); // if changing MCG DRB need to tell NR the Id } } select (p_SecondaryConfig) { // if changing SRB 1/2 need to also update the ciphering/integrity algorithms to NR values case (NR_PDCP_SRB1_2, NR_PDCP_MCG_SRB1_2, NR_PDCP_SRB1_2_SRB3) { v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg"); f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3))); v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg"); f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3))); } } //Receive SCG, RBConfig and RBConfig2 from NR PTC v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB2"); v_SrbDrbList := { //@sic R5s190476 change 4.2 sic@ cds_SRB1_Config_NRPDCP, cds_SRB2_Config_NRPDCP }; select (p_SecondaryConfig) { case (NR_PDCP_SRB1_2, NR_PDCP_SRB1_2_SRB3) {// LTE config to configure NR PDCP on EUTRA PTC //@sic R5s190476 change 4.16 sic@ // do nothing as SRBs changed elsewhere v_DrbConfigAtSS[0] := cs_OneDRB_ConfigAM(tsc_DRB1); v_DrbConfigAtSS[1] := cs_OneDRB_ConfigAM(f_EUTRA_NB_EpsBearerAssociatedDRB(tsc_EpsDefaultBearerId2ndPDN)); } case (NR_PDCP_MCG_SRB1_2) {// LTE config to configure NR PDCP on EUTRA PTC EUTRA38_DRB! v_DrbConfigAtSS := { cs_RadioBearer_Reconfig_DRBwithNRPDCPonEUTRA(tsc_DRB8, cs_RLC_Configuration_AM, cs_MAC_Configuration_AM) }; //@sic R5s190554 change 4.22 sic@ //@sic R5s200051 change 1 sic@ /* @sic R5s200570: replaced p_AddRelDrb8 with p_Drb_ToReleaseList and p_DRB_ToAddModList sic@ */ v_Drb_ToAddModList := { cds_508_DRB_ToAddMod_Common(tsc_DRB8, omit, cs_508_RLC_Config_DRB_AM,cs_508_LogicalChannelConfig_DRB_AM)}; v_Drb_ToReleaseList := {tsc_DRB8}; } } << SKIPPED CODE >> }

MCC160 proposed implementation

function f_EUTRA38_IntraLTE_HO_IntraPSCell_NR_PDCP (EUTRA_CellId_Type p_CellId,

ENDC_SRB_DRB_Configuration_Type p_SecondaryConfig,

template (omit) integer p_SK_Counter := 0

) runs on EUTRA_5GS_PTC

{ //@sic R5s190476 change 4.15: added p_SK_Counter sic@

var template (value) RadioBearerList_Type v_DrbConfigAtSS := { cs_OneDRB_ConfigAM(tsc_DRB1), cs_OneDRB_ConfigAM(f_EUTRA_NB_EpsBearerAssociatedDRB(tsc_EpsDefaultBearerId2ndPDN)) }; //@sic R5s191169 R5s200051 change 1 R5s211128 sic@

var integer v_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) octetstring v_NR_SCGConfig;

var template (value) octetstring v_NR_RBConfig;

var template (value) octetstring v_NR_RBConfig2;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var SubFrameTiming_Type v_Timing;

var template (value) RadioBearerList_Type v_SrbDrbList;   //@sic R5s190476 change 4.2 sic@

var template (omit) DRB_ToAddModList v_Drb_ToAddModList := omit;    //@sic R5s200570 change 2.3 sic@

var template (omit) DRB_ToReleaseList v_Drb_ToReleaseList := omit;  //@sic R5s200570 change 2.3 sic@

select (p_SecondaryConfig) {

case (NR_PDCP_MCG_SRB1_2) {// LTE config to configure NR PDCP on EUTRA PTC

NR.send (cms_EUTRA_NR_OctetData (int2oct(v_DRB_Id, 1), "DRB ID")); // if changing MCG DRB need to tell NR the Id

}

## 1.4 Correction to module EUTRA38_Handover

| Module name | EUTRA38_Handover |
| --- | --- |
| Reason for change | For implementing change 1.3 , need to make use of tsc_DRB1 and tsc_EpsDefaultBearerId2ndPDN , so need to import the below modules |
| Summary of change | For the compilation purpose , imported EPS_NAS_Constants and EUTRA_NB_CommonDefs |
| TTCN module | ttcn\develop\ENDC\Common_EUTRA\EUTRA38_Handover.ttcn |
| MCC160 Comment | Accepted |

Before Change:

import from Common4G5G all; import from CommonDefs all; import from CommonAspDefs all; import from EUTRA_NR_CoordMsg all; import from EUTRA_CommonDefs all; import from CommonDefs all; import from CommonAspDefs all; import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from EUTRA_Component_5GS all; import from EUTRA_NR_CoordMsg all; import from EUTRA_CommonDefs all; import from EUTRA_ASP_TypeDefs all; import from Common4G5G_Templates all; import from EUTRA_SRB_Templates all; import from NR_PDCP_Templates all; import from EUTRA_DRB_Templates all; import from EUTRA_CellCfg_Templates all; import from EUTRA_NB_ASP_Templates all; import from EUTRA38_CommonProcedures all; import from EUTRA38_ENDC_CommonProcedures all; import from EUTRA_Timing all; import from EUTRA_RRCSteps all; import from EUTRA_SecuritySteps all; import from EUTRA_ConfigurationSteps all;

After Change:

import from Common4G5G all; import from CommonDefs all; import from CommonAspDefs all; import from EUTRA_NR_CoordMsg all; import from EUTRA_CommonDefs all; import from CommonDefs all; import from CommonAspDefs all; import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from EUTRA_Component_5GS all; import from EUTRA_NR_CoordMsg all; import from EUTRA_CommonDefs all; import from EUTRA_ASP_TypeDefs all; import from Common4G5G_Templates all; import from EUTRA_SRB_Templates all; import from NR_PDCP_Templates all; import from EUTRA_DRB_Templates all; import from EUTRA_CellCfg_Templates all; import from EUTRA_NB_ASP_Templates all; import from EUTRA38_CommonProcedures all; import from EUTRA38_ENDC_CommonProcedures all; import from EUTRA_Timing all; import from EUTRA_RRCSteps all; import from EUTRA_SecuritySteps all; import from EUTRA_ConfigurationSteps all; import from EPS_NAS_Constants all; import from EUTRA_NB_CommonDefs all;