3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211189

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0741 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS supplementary services test case 8.34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS supplementary services test case 8.34 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS supplementary services test case 8.34 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	6

4	Execution Log Files	6

4.1	HiSilicon Balong 5000	6

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.34 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.34.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_34_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Calling f_IMS_BuildCOnferenceUri before IMS PTC has been initialised will give a run-time error. |
| Summary of change | Move URI creation to after call to f_IMS_CC_Preamble() |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Accepted in principle (implementation has been changed since 21wk24 due to implementation of other test cases) |

Before change

function f_TC_8_34_NR5GC_IMS1() runs on IMS_PTC

{ /* sister test case of 15.21a */

var charstring v_ConferenceFactoryUri   := f_IMS_BuildConferenceUri(conferenceFactoryUri);

var charstring v_TemporaryConferenceUri := f_IMS_BuildConferenceUri(temporaryConferenceUri);

var charstring v_FinalConferenceUri     := f_IMS_BuildConferenceUri(finalConferenceUri);

var InviteUserToConference_StateInfo_Type v_Dialog1_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri,  px_IMS_CalleeContactUri));

var InviteUserToConference_StateInfo_Type v_Dialog2_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri2, px_IMS_CalleeContactUri));

var boolean v_RadioBearersEstablished := false;

var template (omit) SUBSCRIBE_Request v_SubscribeRequest;      // optional SUBSCRIBE for conference event package

var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog1;

var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog2;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp_Dialog3;

var SipUrl v_ContactUrl_UE;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

//--------------------------------------------

// DIALOG 1

//--------------------------------------------

// Setup MO speech call

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, v_Dialog1_StateInfo.CalleeUri);

…..

}

After change

function f_TC_8_34_NR5GC_IMS1() runs on IMS_PTC

{ /* sister test case of 15.21a */

//

var charstring v_ConferenceFactoryUri ;

var charstring v_TemporaryConferenceUri;

var charstring v_FinalConferenceUri  ;

var InviteUserToConference_StateInfo_Type v_Dialog1_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri,  px_IMS_CalleeContactUri));

var InviteUserToConference_StateInfo_Type v_Dialog2_StateInfo := valueof(cs_InviteUserToConference_StateInfo_Init(px_IMS_CalleeUri2, px_IMS_CalleeContactUri));

var boolean v_RadioBearersEstablished := false;

var template (omit) SUBSCRIBE_Request v_SubscribeRequest;      // optional SUBSCRIBE for conference event package

var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog1;

var ThreeWaySession_SessionInfo_Type v_SessionInfo_Dialog2;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp_Dialog3;

var SipUrl v_ContactUrl_UE;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

v_ConferenceFactoryUri   := f_IMS_BuildConferenceUri(conferenceFactoryUri);

v_TemporaryConferenceUri := f_IMS_BuildConferenceUri(temporaryConferenceUri);

v_FinalConferenceUri     := f_IMS_BuildConferenceUri(finalConferenceUri);

//--------------------------------------------

// DIALOG 1

//--------------------------------------------

// Setup MO speech call

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, v_Dialog1_StateInfo.CalleeUri);

…

}

## Change 2

| Function name | fl_IMS_ThreeWaySession_CreateSession_NR5GC_A_19 |
| --- | --- |
| Reason for change | At Step 23-25: Steps 1-13 of A.19 are used to create a conference which invoke steps 1-7 of A.4.1. This sequence mandates the use of SDP precondition attributes. |
| Summary of change | Pass tsc_SDP_WithPreconditions param into the call to f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5() |
| TTCN module | IMS_NR5GC_SupplementaryServiceTestcases.ttcn |
| MCC160 Comment | Accepted in principle (implementation has been changed since 21wk24 due to implementation of other test cases) |

Before change

function fl_IMS_ThreeWaySession_CreateSession_NR5GC_A_19(charstring p_ConferenceFactoryUri,

charstring p_TemporaryConferenceUri,

charstring p_FinalConferenceUri,

boolean p_RadioBearersEstablished) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type

{

var template (value) SemicolonParam_List v_AdditionalContactParams := {cs_GenericParam("isfocus")};

var IMS_CallType_Type v_CallType := CONFERENCE_CALL;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1_OtherCalleeUri(p_ConferenceFactoryUri);                                                       // A.4.1 Step 1        INVITE

f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp);                                                                                                    // A.4.1 Step 2        100 Trying

f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5(v_InviteRequestWithSdp, p_TemporaryConferenceUri, v_AdditionalContactParams, -, v_CallType);                          // A.4.1 Step 3 - 5    183 Session Progress, PRACK, 200 OK

if (not p_RadioBearersEstablished) {  // !!!! In case that the radio bearer is established already most likely the UE will send the updated offer in the PRACK and there shall be no UPDATE !!!!

f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall);

}

f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7(v_InviteRequestWithSdp, p_TemporaryConferenceUri, -, v_CallType);                                                     // A.4.1 Step 6 - 7     UPDATE, 200 OK

f_IMS_Conferencing_Send200OK_ReceiveACK(v_InviteRequestWithSdp, p_FinalConferenceUri, v_AdditionalContactParams);                                           // A.19  Step 8 - 9     200 OK, ACK

return v_InviteRequestWithSdp;

}

After change

function fl_IMS_ThreeWaySession_CreateSession_NR5GC_A_19(charstring p_ConferenceFactoryUri,

charstring p_TemporaryConferenceUri,

charstring p_FinalConferenceUri,

boolean p_RadioBearersEstablished) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type

{

var template (value) SemicolonParam_List v_AdditionalContactParams := {cs_GenericParam("isfocus")};

var IMS_CallType_Type v_CallType := CONFERENCE_CALL;

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1_OtherCalleeUri(p_ConferenceFactoryUri);                                                       // A.4.1 Step 1        INVITE

f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp);

// A.4.1 Step 2        100 Trying

f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5(v_InviteRequestWithSdp, p_TemporaryConferenceUri, v_AdditionalContactParams, -, v_CallType, -,-,tsc_SDP_WithPreconditions ); // A.4.1 Step 3 - 5    183 Session Progress, PRACK, 200 OK

if (not p_RadioBearersEstablished) {  // !!!! In case that the radio bearer is established already most likely the UE will send the updated offer in the PRACK and there shall be no UPDATE !!!!

f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall);

}

f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7(v_InviteRequestWithSdp, p_TemporaryConferenceUri, -, v_CallType);                                                     // A.4.1 Step 6 - 7     UPDATE, 200 OK

f_IMS_Conferencing_Send200OK_ReceiveACK(v_InviteRequestWithSdp, p_FinalConferenceUri, v_AdditionalContactParams);                                           // A.19  Step 8 - 9     200 OK, ACK

return v_InviteRequestWithSdp;

}

# Branches executed

The test case was executed on NR band n41 using ciphering algorithm nea1 and integrity algorithm nea2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_34_LOG.html

- PICS/PIXIT parameter file:

TC_8_34_PIXIT.xml

# References

| [1] | R5s211190: Supporting information for addition of IMS 5GS supplementary services test case 8.34 |
| --- | --- |