3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211263

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2027 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 9.1.5.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of f_TC_9_1_5_1_15_NR5GC, for postamble the function f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); is called. This is correct because before calling the postamble, there is no SRB2 established. However, in the call f_NR_Postamble f_NR_UE_DeRegisterOnSwitchOff, the p_SRB is not passed on. Also, within function f_NR_UE_DeRegisterOnSwitchOff for case STATE_CONNECTED_3A, the UL message is always received on SRB2 (tsc_NR_RbId_SRB2 is hardcoded). This causes testcase failure for 9.1.5.1.15 where SRB2 was never established, but the TTCN expects UL message on SRB2 This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an argument for passing SRB information to function f_NR_UE_DeRegisterOnSwitchOff, and passed parameter p_SRB to this function when being called from f_NR_Postamble. Also passed the p_SRB within f_NR_UE_DeRegisterOnSwitchOff for the case STATE_CONNECTED_3A. Pkease see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function f_NR_Postamble	3

1.2	Correction to function f_NR_UE_DeRegisterOnSwitchOff	7

# Corrections required

## Correction to function f_NR_Postamble

| Function name | f_NR_Postamble |
| --- | --- |
| Reason for change | In the current TTCN implementation of f_TC_9_1_5_1_15_NR5GC, for postamble the function f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); is called. This is correct because before calling the postamble, there is no SRB2 established. However, in the call f_NR_Postamble f_NR_UE_DeRegisterOnSwitchOff, the p_SRB is not passed on. Also, within function f_NR_UE_DeRegisterOnSwitchOff for case STATE_CONNECTED_3A, the UL message is always received on SRB2 (tsc_NR_RbId_SRB2 is hardcoded). This causes testcase failure for 9.1.5.1.15 where SRB2 was never established. This needs to be addressed. |
| Summary of change | Added an argument for passing SRB information to function f_NR_UE_DeRegisterOnSwitchOff, and passed parameter p_SRB to this function when being called from f_NR_Postamble. Also passed the p_SRB within f_NR_UE_DeRegisterOnSwitchOff for the case STATE_CONNECTED_3A. Pkease see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); // @sic R5-195241 sic@ } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

After Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function //WA#9_1_5_1_15 COMMENTED OUT f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); // @sic R5-195241 sic@ f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, -, -, p_SRB); //WA#9_1_5_1_15 } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

## Correction to function f_NR_UE_DeRegisterOnSwitchOff

| Function name | f_NR_UE_DeRegisterOnSwitchOff |
| --- | --- |
| Reason for change | Change needed for Change 1.1 above |
| Summary of change | Please see change 1.1 |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_UE_DeRegisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, boolean p_USIM_Rmvd := false, B2_Type p_AccessType := '01'B) runs on NR5GC_PTC { <<SKIPPED CODE>> case (STATE_CONNECTED_3A){ // @sic R5-201202, R5-202562 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s201132 sic@ // DRB already configured, so the UE won't send any NAS msg until it's completed IMS Deregistration SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } <<SKIPPED CODE>>

After Change:

function f_NR_UE_DeRegisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, boolean p_USIM_Rmvd := false, B2_Type p_AccessType := '01'B, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2/*WA#9_1_5_1_15*/) runs on NR5GC_PTC { <<SKIPPED CODE>> case (STATE_CONNECTED_3A){ // @sic R5-201202, R5-202562 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s201132 sic@ // DRB already configured, so the UE won't send any NAS msg until it's completed IMS Deregistration SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB /*WA#9_1_5_1_15 tsc_NR_RbId_SRB2*/, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB /*WA#9_1_5_1_15 tsc_NR_RbId_SRB2*/, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } <<SKIPPED CODE>>