3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211308

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2068 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, In the function : f_TC_8_1_2_1_1_NR5GC_TestBody , At step 6 , when sending RRC Connection Reconfiguration to establish DRB1 and SRB2 , mac logical channel config is filled as Omit which is not in align with 38.508-1 [4], table 4.6.3-66 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_2_1_1_NR5GC_TestBody , mac Logical Channel Config is filled while sending Reconfiguration at step 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may reject this reconfiguration message without Logical channel configuration for SRB and DRB |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_8_1_2_1_1_NR5GC_TestBody	2

1.2	Correction to module RRC_Reconfiguration_NR5GC	6

# Corrections required

## Correction to function f_TC_8_1_2_1_1_NR5GC_TestBody

| Function name | f_TC_8_1_2_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, In the function : f_TC_8_1_2_1_1_NR5GC_TestBody , At step 6 , when sending RRC Connection Reconfiguration to establish DRB1 and SRB2 , mac logical channel config is filled as Omit which is not in align with 38.508-1 [4], table 4.6.3-66 |
| Summary of change | In the function f_TC_8_1_2_1_1_NR5GC_TestBody , mac Logical Channel Config is filled while sending Reconfiguration at step 6 |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#93 |

Before Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_CellGroupConfig.rlc_BearerToAddModList[0].mac_LogicalChannelConfig := omit; v_CellGroupConfig.rlc_BearerToAddModList[1].mac_LogicalChannelConfig := omit; v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); << SKIPPED CODE >>

After Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_NoOfDRBs := f_NR5GC_MobileInfo_GetNoOfDRBs(); var template (omit) NR_DRBInfo_Type v_DRBInfo; var integer v_DRBId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var NR_RadioBearerList_Type v_SS_Config; // @siclog "Step 1" siclog@ // SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // @siclog "Step 2-4" siclog@ // UE transmits an RRCSetupRequest message // SS transmits an RRCSetup message // The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs configured"); } //Get DRB Info for DRB associated with 1st Internet PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession(); // @sic R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); if (not (isvalue(v_DRBInfo))) { // @sic R5s200571 Ch. 4 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No Non-IMS DRBs configured"); } // @siclog "Step 5" siclog@ // SS transmits a SecurityModeCommand message to activate AS security. v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedMsg.SecurityProtection.NasCount); f_NR_Security_Set(v_SecurityParams); // Read PDCP SQN for SRB1 and calculate Ciphering Activation times v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); // send SecurityModeCommand on SRB1. Follow on flag is set to TRUE to ensure the RRCConnectionReconfiguration message is sent on the same TTI and therefore SRB.send(cas_NR_SRB1_RrcPdu_REQ_FollowOnFlag(nr_Cell1, -, // cs_TimingInfo_Now is needed for FollowOn = true to work cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm)))); // Sent before it sends SecurityModeComplete message. // @siclog "Step 6" siclog@ // Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_CellGroupConfig.rlc_BearerToAddModList[0].mac_LogicalChannelConfig := cs_38508_LogicalChannelConfig_SRB(tsc_SRB2); v_CellGroupConfig.rlc_BearerToAddModList[1].mac_LogicalChannelConfig := cs_38508_LogicalChannelConfig_DRB; v_CellGroupConfig.mac_CellGroupConfig := omit; v_CellGroupConfig.physicalCellGroupConfig := omit; v_CellGroupConfig.spCellConfig := omit; f_ReconfigureDRBs( nr_Cell1, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg())}, // @sic Baseline Moving sic@, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, -, false, -, -, v_SecurityParams); << SKIPPED CODE >>

## Correction to module RRC_Reconfiguration_NR5GC

| Module name | RRC_Reconfiguration_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation, In the function : f_TC_8_1_2_1_1_NR5GC_TestBody , At step 6 , when sending RRC Connection Reconfiguration to establish DRB1 and SRB2 , mac logical channel config is filled as Omit which is not in align with 38.508-1 [4], table 4.6.3-66 |
| Summary of change | In the function f_TC_8_1_2_1_1_NR5GC_TestBody , mac Logical Channel Config is filled while sending Reconfiguration at step 6. And to make this change compilable , below module has to be imported |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment | Linked to change 1.1 |

Before Change:

module RRC_Reconfiguration_NR5GC { import from Common4G5G_Templates all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NG_NAS_MsgContainers all; import from NR_CommonDefs all; import from NR_ASP_SrbDefs all; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NG_NAS_Common all; import from Common4G5G_Templates all; import from NR_RRC_Templates all; import from NR_ASP_Templates_RB_Config all; import from NR_ASP_Templates_CommonPart all; import from NR_ASP_Templates_SRB all; import from NR5GC_RRC_Templates all; import from NR5GC_RRC_Templates all; import from NR_CommonFunctions all; import from NR_ConfigurationSteps all; import from NR_CellInfo all; import from NR_SecuritySteps all; import from NR_SecurityFunctions all; import from NR_SecurityFunctions all; import from NR_SecuritySteps all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR5GC_RRCSteps all; import from NR5GC_SysinfoPaging all; import from NR5GC_CommonFunctions all; import from NR5GC_LoopBack all; import from NG_NAS_MsgContainers all; import from NG_NASTemplateFunctions all; import from NG_NAS_Templates all; import from NAS_CommonTemplates all; import from NAS_5GC_Parameters all; import from EUTRA_NR_Parameters all; import from UpperTesterFunctions all; import from NR_ASP_TypeDefs all; import from NR_CellCfg_Templates all;

After Change:

module RRC_Reconfiguration_NR5GC { import from Common4G5G_Templates all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NG_NAS_MsgContainers all; import from NR_CommonDefs all; import from NR_ASP_SrbDefs all; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NG_NAS_Common all; import from Common4G5G_Templates all; import from NR_RRC_Templates all; import from NR_ASP_Templates_RB_Config all; import from NR_ASP_Templates_CommonPart all; import from NR_ASP_Templates_SRB all; import from NR5GC_RRC_Templates all; import from NR5GC_RRC_Templates all; import from NR_CommonFunctions all; import from NR_ConfigurationSteps all; import from NR_CellInfo all; import from NR_SecuritySteps all; import from NR_SecurityFunctions all; import from NR_SecurityFunctions all; import from NR_SecuritySteps all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR5GC_RRCSteps all; import from NR5GC_SysinfoPaging all; import from NR5GC_CommonFunctions all; import from NR5GC_LoopBack all; import from NG_NAS_MsgContainers all; import from NG_NASTemplateFunctions all; import from NG_NAS_Templates all; import from NAS_CommonTemplates all; import from NAS_5GC_Parameters all; import from EUTRA_NR_Parameters all; import from UpperTesterFunctions all; import from NR_ASP_TypeDefs all; import from NR_CellCfg_Templates all; import from CommonAspDefs all;