3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211326

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2079 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR idle mode test case 6.3.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When UE released cell 13 at step 17Aa2, if pc_noOf_PDUsNewConnection > 0 and UE doesn’t start PDU session establish automatically, UE will select cell 12 rather than cell 11 (as expected at step 20) |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 17Aa3a1, refer to cell 12 instead of cell 11 for checking access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_6_3_1_5_TestBody |
| --- | --- |
| Reason for change | When UE released cell 13 at step 17Aa2, if pc_noOf_PDUsNewConnection > 0 and UE doesn’t start PDU session establish automatically, UE will select cell 12 rather than cell 11 (as expected at step 20) |
| Summary of change | At step 17Aa3a1, refer to cell 12 instead of cell 11 for checking access. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_6_3_1_5_TestBody() runs on NR5GC_PTC { ………………… //@siclog "Step 17Aa2" siclog@ //@sic R5-216162 sic@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell13); //EXCEPTION: Step 17Aa3a1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically //@sic R5-216162 sic@ //@siclog "Step 17Aa3a1" siclog@ //@sic R5-216162 sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 ){ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; f_NR5GC_ExIdlePDUsession(nr_Cell13); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; } [] t_Wait.timeout{} } } ………………… }

After Change

function fl_TC_6_3_1_5_TestBody() runs on NR5GC_PTC { …………….. //@siclog "Step 17Aa2" siclog@ //@sic R5-216162 sic@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell13); //EXCEPTION: Step 17Aa3a1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically //@sic R5-216162 sic@ //@siclog "Step 17Aa3a1" siclog@ //@sic R5-216162 sic@ //The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. if (pc_noOf_PDUsNewConnection > 0 ){ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; f_NR5GC_ExIdlePDUsession(nr_Cell13); } [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Wait.stop; } [] t_Wait.timeout{} } } ………………….. }