3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211335

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2088 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR_RRCReconfig_AddSCG_DRBs_NRDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In this function f_NR_RRCReconfig_AddSCG_DRBs_NRDC, p_PDCP_Config is passed as a parameter and evaluated with isvalue to assign v_DRBToAddMod_SCG. In test cases the parameter is not passed and therefore v_DRBToAddMod_SCG results in uninitialized variable. The DRB configuration is required to setup NR-DC SCG and therefore in order avoit the uninitialized variable, it is proposed to change the default to cs_38508_PDCP_Config and where it is not required it has to be handled with omit as a parameter when calling the function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Assigned p_PDCP_Config default value as cs_38508_PDCP_Config |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execution will result in failure due to SCG configuration being incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NR-DC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for NR-DC	2

1.1	Correction to f_NR_RRCReconfig_AddSCG_DRBs_NRDC	2

# Corrections required for NR-DC

## Correction to f_NR_RRCReconfig_AddSCG_DRBs_NRDC

| Function name | f_NR_RRCReconfig_AddSCG_DRBs_NRDC |
| --- | --- |
| Reason for change | In this function f_NR_RRCReconfig_AddSCG_DRBs_NRDC, p_PDCP_Config is passed as a parameter and evaluated with isvalue to assign v_DRBToAddMod_SCG. In test cases the parameter is not passed and therefore v_DRBToAddMod_SCG results in uninitialized variable. The DRB configuration is required to setup NR-DC SCG and therefore in order avoid the uninitialized variable, it is proposed to change the default to cs_38508_PDCP_Config and where it is not required it has to be handled with omit as a parameter when calling the function. |
| Summary of change | Assigned p_PDCP_Config default value as cs_38508_PDCP_Config |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted, in addition the parameter is set to omit in TC 8.2.2.5.2 step 1 f_NR_RRCReconfig_AddSCG_DRBs_NRDC(nr_Cell1,nr_Cell10,MCG_Split, omit); |

Before Change:

function f_NR_RRCReconfig_AddSCG_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, NRDC_DRB_Configuration_Type p_NRDC_DRB_Type, template(omit) PDCP_Config p_PDCP_Config := omit, template(omit) NR_RadioBearerList_Type p_SS_ConfigMCG := omit, template(omit) NR_RadioBearerList_Type p_SS_ConfigSCG := omit ) runs on NR5GC_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (omit) RadioBearerConfig v_RadioBearerConfig1 := omit; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (omit) CellGroupConfig v_CellGroupConfigMCG := omit; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (omit) NR_RadioBearerList_Type v_SS_ConfigMCG := omit; var template (omit) NR_RadioBearerList_Type v_SS_ConfigSCG := omit; var template (value) SDAP_Config v_SDAP_ConfigSCG; var integer v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(p_SCG_CellId); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var NRDC_SecurityActivation_Type v_NRDC_SecurityActivation; var RNTI_Value v_RNTI_ValueSecondary := tsc_C_RNTI_Value4; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var CellGroupId v_CellGroupIdSCG := f_NR_CellInfo_Get_CellGroupId(p_SCG_CellId); if (isvalue(v_GSM_MobilityInfo)) { v_QoSFlow := cs_QoS_Flow5; v_QoSFlow.qfi := '000101'B; // QFI = 5 v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow})); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } select(p_NRDC_DRB_Type) { case(MCG_SCG){ // Configure SCG DRB //@sic R5s210791 R5s210888 sic@ if (isvalue(p_PDCP_Config)) { v_DRBToAddMod_SCG := cs_38508_NRDRB_ToAddMod(v_SDAP_ConfigSCG, v_SCG_DRB_Id, p_PDCP_Config); } v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()), secondary)); if (isvalue(p_SS_ConfigSCG)) { v_SS_ConfigSCG := p_SS_ConfigSCG; } else { v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; } v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_SCG_CellId, v_SCG_DRB_Id, v_RNTI_ValueSecondary); v_NRDC_SecurityActivation := MCG_SCG_SecondaryKeys; }

After Change:

function f_NR_RRCReconfig_AddSCG_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, NRDC_DRB_Configuration_Type p_NRDC_DRB_Type, template(omit) PDCP_Config p_PDCP_Config := cs_38508_PDCP_Config, template(omit) NR_RadioBearerList_Type p_SS_ConfigMCG := omit, template(omit) NR_RadioBearerList_Type p_SS_ConfigSCG := omit ) runs on NR5GC_PTC { var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (omit) RadioBearerConfig v_RadioBearerConfig1 := omit; var template (omit) RadioBearerConfig v_RadioBearerConfig2 := omit; var template (omit) CellGroupConfig v_CellGroupConfigMCG := omit; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (omit) NR_RadioBearerList_Type v_SS_ConfigMCG := omit; var template (omit) NR_RadioBearerList_Type v_SS_ConfigSCG := omit; var template (value) SDAP_Config v_SDAP_ConfigSCG; var integer v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(p_SCG_CellId); var template (value) DRB_ToAddMod v_DRBToAddMod_SCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var NRDC_SecurityActivation_Type v_NRDC_SecurityActivation; var RNTI_Value v_RNTI_ValueSecondary := tsc_C_RNTI_Value4; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var CellGroupId v_CellGroupIdSCG := f_NR_CellInfo_Get_CellGroupId(p_SCG_CellId); if (isvalue(v_GSM_MobilityInfo)) { v_QoSFlow := cs_QoS_Flow5; v_QoSFlow.qfi := '000101'B; // QFI = 5 v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow})); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } select(p_NRDC_DRB_Type) { case(MCG_SCG){ // Configure SCG DRB //@sic R5s210791 R5s210888 sic@ if (isvalue(p_PDCP_Config)) { v_DRBToAddMod_SCG := cs_38508_NRDRB_ToAddMod(v_SDAP_ConfigSCG, v_SCG_DRB_Id, p_PDCP_Config); } v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, {v_DRBToAddMod_SCG}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()), secondary)); if (isvalue(p_SS_ConfigSCG)) { v_SS_ConfigSCG := p_SS_ConfigSCG; } else { v_SS_ConfigSCG := { cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG) }; } v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_SCG_CellId, v_SCG_DRB_Id, v_RNTI_ValueSecondary); v_NRDC_SecurityActivation := MCG_SCG_SecondaryKeys; }