3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211341

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2091 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR_SendRRCReconfiguration_NRDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SRB2 is configured therefore the NAS message will be received on SRB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed SRB.receive(car_NR_SRB_NasPdu_IND ) to receive NAS message on SRB2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NR-DC test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for NR-DC test cases	2

1.1	Correction to f_NR_SendRRCReconfiguration_NRDC	2

# Corrections required for NR-DC test cases

## Correction to f_NR_SendRRCReconfiguration_NRDC

| Function name | f_NR_SendRRCReconfiguration_NRDC |
| --- | --- |
| Reason for change | SRB2 is configured therefore the NAS message will be received on SRB2 |
| Summary of change | Changed SRB.receive(car_NR_SRB_NasPdu_IND ) to receive NAS message on SRB2 |
| TTCN module | NR5GC_RRCSteps_NRDC |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_SendRRCReconfiguration_NRDC (NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) RadioBearerConfig p_RadioBearerConfig1 := omit, template (omit) RRCReconfiguration_v1530_IEs p_V1530Ext := omit, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg := omit, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_PDUSessionModificationComplete := false ) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //Reconfigure PSCell to enable PRACH indication - UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell. if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); } // Send RRCReconfigurationmessage to UE - Don't wait for RRCReconfigurationComplete as it may come in parallel to the PRACH indication on PSCell f_NR_SendRRCReconfiguration (p_MCG_CellId, p_SRB, p_RadioBearerConfig1, p_V1530Ext, p_NASMsg, -, p_TimingInfo, false); if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ if (p_PDUSessionModificationComplete) { // @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } else {

After Change:

function f_NR_SendRRCReconfiguration_NRDC (NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) RadioBearerConfig p_RadioBearerConfig1 := omit, template (omit) RRCReconfiguration_v1530_IEs p_V1530Ext := omit, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg := omit, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_PDUSessionModificationComplete := false ) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //Reconfigure PSCell to enable PRACH indication - UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell. if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); } // Send RRCReconfigurationmessage to UE - Don't wait for RRCReconfigurationComplete as it may come in parallel to the PRACH indication on PSCell f_NR_SendRRCReconfiguration (p_MCG_CellId, p_SRB, p_RadioBearerConfig1, p_V1530Ext, p_NASMsg, -, p_TimingInfo, false); if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ if (p_PDUSessionModificationComplete) { // @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } else {