3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211342

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2092 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The current implementation to check the specific message contents of REGISTRATION REQUEST from GMM mobility info after the call to f_NR5GC_PreambleReject () always gives a run-time error, because the stored information is cleared during the function f_NR_UE_DeRegisterOnSwitchOff () - Upon updating the security from the mapped context received from EUTRA, the DL NAS count need to be reset in order to successfully calculate the MAC-I for the Security Mode Command message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the call to f_NR5GC_PreambleReject () and bring the preamble signalling into the test case so that the specific message contents of REGISTRATION REQUEST can be checked before sending REGISTRATION REJECT - Reset NAS counts before sending SECURITY MODE COMMAND. Note : This is the original proposed change 1 from R5s210446. The solution implemented in 21wk37, to set both NAS counts to 0 in f_NR5GC_MappedContextFromS1_InitNAS() does not work because the next DL message is then sent with count value 1. If we reset the NAS count (send NG_NAS_SecurityStart_Req with NasCountReset = true), the NG NAS EMU resets the count to the maximum value, so that it will send the following DL message with count 0 (see the comment in f_NasCountInit()) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_9_3_1_2_NR5GC |
| --- | --- |
| Reason for change | The current implementation to check the specific message contents of REGISTRATION REQUEST from GMM mobility info after the call to f_NR5GC_PreambleReject () always gives a run-time error, because the stored information is cleared during the function f_NR_UE_DeRegisterOnSwitchOff () |
| Summary of change | Remove the call to f_NR5GC_PreambleReject () and bring the preamble signalling into the test case so that the specific message contents of REGISTRATION REQUEST can be checked before sending REGISTRATION REJECT |
| TTCN module | Mobility_Registration_NR5GC_NR |
| MCC160 Comment | Not needed - after implementation of R5s211358, the GMM mobility info is (re-)stored at the end of f_NR5GC_PreambleReject |

Before Change

function f_TC_9_3_1_2_NR5GC() runs on NR5GC_PTC { //Mobility Registration Update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template bitstring v_PatterncIoT_V2X := (? length (2) & '1'B & ? length (5)); f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); // @sic R5-205142 sic@ f_NR5GC_PreambleReject (v_NGC_NASCellA, tsc_EmmCause_IllegalUe); // @sic R5-216185 sic@ // Now check specific message contents v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); if (match(v_GMM_MobilityInfo.GMMCap.s1Cap, '1'B) and match(v_GMM_MobilityInfo.S1NwkCap.cIoT_V2X, v_PatterncIoT_V2X)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } // Turn power off f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // Now tell EUTRA to attach f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); // First pass capability info to EUTRA f_NR5GC_IRAT_UECapabilityTransfer(v_NGC_NASCellA); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); f_NR_TestBody_Set(true); v_SRB := fl_TC_9_3_1_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, v_SRB); // @sic R5s200379, R5s201005 sic@ sic@ // Now tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function f_TC_9_3_1_2_NR5GC() runs on NR5GC_PTC { //Mobility Registration Update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template bitstring v_PatterncIoT_V2X := (? length (2) & '1'B & ? length (5)); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4(v_NGC_NASCellA); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, Initial_NoSecurity); // @sic R5-216185 sic@ // Now check specific message contents if (match(v_GMM_MobilityInfo.GMMCap.s1Cap, '1'B) and match(v_GMM_MobilityInfo.S1NwkCap.cIoT_V2X, v_PatterncIoT_V2X)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "EUTRA and NR Capability, Preamble"); } SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_EmmCause_IllegalUe))))); f_NR_RRCRelease (v_NGC_NASCellA); f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_DEREGISTERED); // Turn power off f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // Now tell EUTRA to attach f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); // First pass capability info to EUTRA f_NR5GC_IRAT_UECapabilityTransfer(v_NGC_NASCellA); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); f_NR_TestBody_Set(true); v_SRB := fl_TC_9_3_1_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, v_SRB); // @sic R5s200379, R5s201005 sic@ sic@ // Now tell EUTRA test is finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 2

| Function name | fl_TC_9_3_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | Upon updating the security from the mapped context received from EUTRA, the DL NAS count need to be reset in order to successfully calculate the MAC-I for the Security Mode Command message. |
| Summary of change | Reset NAS counts before sending SECURITY MODE COMMAND. Note : This is the original proposed change 1 from R5s210446. The solution implemented in 21wk37, to set both NAS counts to 0 in f_NR5GC_MappedContextFromS1_InitNAS() does not work because the next DL message is then sent with count value 1. If we reset the NAS count (send NG_NAS_SecurityStart_Req with NasCountReset = true), the NG NAS EMU resets the count to the maximum value, so that it will send the following DL message with count 0 (see the comment in f_NasCountInit()) |
| TTCN module | Mobility_Registration_NR5GC_NR.ttcn |
| MCC160 Comment | Accepted in principle, but added the change to common function f_NR5GC_MappedContextFromS1_InitNAS instead |

Before Change

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_RadioBearerId_Type v_SRB;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_UL_Pdu_Type v_NASMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;

var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var boolean v_Result;

var NasCount_Type v_NasCountUL;

//@siclog "Step 1" siclog@

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 2" siclog@

// Steps 1 - 3

// RRC Connection Setup

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security(EUTRA);        // @sic R5-205142 sic@

v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@

v_NASMsg := v_ReceivedMsg.Pdu;  // @sic R5s210359 sic@  Store original Registration Request

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo,

v_ReceivedMsg.Pdu,

Mobility,

f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@

'111'B,

'?'B,

omit,

omit,

?,

omit,

omit,

omit,

omit,

omit,

omit,

cr_UEStatus_S1,

-,

omit,

omit,

omit,

?,

omit,

omit,

omit,

omit,

omit,

omit,

omit,

omit)) {  // @sic R5-206290, R5-211404 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

// Send TAU Req for checking in EUTRA

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq"));

// derive 5G parameters from values received from EUTRA

f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, false);  // @sic R5s210078 sic@

// Do NAS Security Mode procedure

v_SecurityParams := f_NR5GC_Security_Get();

v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams,

Initial_NoSecurity,   // @sic R5s210078 sic@

v_GMM_MobilityInfo);

// Mapped security context with Kasme @sic R5-205142 sic@

v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext);

// send SECURITY MODE COMMAND

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend)));

// receive SECURITY MODE COMPLETE

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp;

v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity);  // @sic R5s210078 sic@

v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount;

if (not v_Result) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed");

}

if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg)) { // @sic R5s210359 sic@

// Check updated Registration Request and update v_GMM_MobilityInfo

v_NASMsg.PiggybackedPduList[0].Msg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg;

v_NASMsg.PiggybackedPduList[0].PiggybackedPduList := omit;

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Mobility)) {  // @sic R5s210446 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed");

}

}

f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo);

f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL);

f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def);

// @sic R5-205142 sic@ Now finish the procedure

v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @sic R5s201005 sic@

return v_SRB;

}

After Change

…

import from NG_NasEmu_CtrlFunctions all;

…

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_RadioBearerId_Type v_SRB;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_UL_Pdu_Type v_NASMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;

var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var boolean v_Result;

var NasCount_Type v_NasCountUL;

//@siclog "Step 1" siclog@

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 2" siclog@

// Steps 1 - 3

// RRC Connection Setup

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security(EUTRA);        // @sic R5-205142 sic@

v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@

v_NASMsg := v_ReceivedMsg.Pdu;  // @sic R5s210359 sic@  Store original Registration Request

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo,

v_ReceivedMsg.Pdu,

Mobility,

f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@

'111'B,

'?'B,

omit,

omit,

?,

omit,

omit,

omit,

omit,

omit,

omit,

cr_UEStatus_S1,

-,

omit,

omit,

omit,

?,

omit,

omit,

omit,

omit,

omit,

omit,

omit,

omit)) {  // @sic R5-206290, R5-211404 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

// Send TAU Req for checking in EUTRA

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq"));

// derive 5G parameters from values received from EUTRA

f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, false);  // @sic R5s210078 sic@

// Do NAS Security Mode procedure

v_SecurityParams := f_NR5GC_Security_Get();

f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* download security keys and reset NAS COUNT */ //ADDED

v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams,

Initial_NoSecurity,   // @sic R5s210078 sic@

v_GMM_MobilityInfo);

// Mapped security context with Kasme @sic R5-205142 sic@

v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext);

// send SECURITY MODE COMMAND

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend)));

// receive SECURITY MODE COMPLETE

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp;

v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity);  // @sic R5s210078 sic@

v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount;

if (not v_Result) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed");

}

if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg)) { // @sic R5s210359 sic@

// Check updated Registration Request and update v_GMM_MobilityInfo

v_NASMsg.PiggybackedPduList[0].Msg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg;

v_NASMsg.PiggybackedPduList[0].PiggybackedPduList := omit;

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Mobility)) {  // @sic R5s210446 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed");

}

}

f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo);

f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL);

f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def);

// @sic R5-205142 sic@ Now finish the procedure

v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @sic R5s201005 sic@

return v_SRB;

}

MCC160 Implementation:

function f_NR5GC_MappedContextFromS1_InitNAS(EUTRA_NR_CoOrd_Security_Type p_EUTRA_Auth_Params,

boolean p_Handover )  runs on NR5GC_PTC

{

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get();

if (p_Handover) {

v_SecurityParams.KAMF := f_NG_Authentication_A15_HO(p_EUTRA_Auth_Params.NH,   // @sic R5s201078 sic@

p_EUTRA_Auth_Params.Key,

v_SecurityParams.KDF);

} else {

v_SecurityParams.KAMF := f_NG_Authentication_A15_IdleMode(p_EUTRA_Auth_Params.NASCountUL,

p_EUTRA_Auth_Params.Key,

v_SecurityParams.KDF);

}

// Derives KNASenc key

v_SecurityParams.NAS_Ciphering.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Enc_Alg,v_SecurityParams.NAS_Ciphering.Algorithm,

v_SecurityParams.KAMF,

v_SecurityParams.KDF);

// Derives KNASint key

v_SecurityParams.NAS_Integrity.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Int_Alg,v_SecurityParams.NAS_Integrity.Algorithm,

v_SecurityParams.KAMF,

v_SecurityParams.KDF);

// Now NAS parameters updated, store updated common values in RRC global variable @sic R5s201078 sic@

v_RRC_SecurityParams.KAMF := v_SecurityParams.KAMF;

v_RRC_SecurityParams.KSIamf := v_SecurityParams.KSIamf;

// Derive KgNB  @sic R5s201078 sic@

v_RRC_SecurityParams.KGNB := f_NR_Authentication_A9(v_RRC_SecurityParams.KDF, v_RRC_SecurityParams.KAMF, 'FFFFFFFF'O);

// set NH and NCC as per 33.501 cl.8.4.2 point 3  @sic R5s201078 sic@

v_RRC_SecurityParams.NH:= v_RRC_SecurityParams.KGNB;

v_RRC_SecurityParams.NCC := 0;

// Now update the SS

f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true);

// @sic R5s201467 sic@

f_NR_Security_Set(v_RRC_SecurityParams);

f_NR5GC_Security_Set (v_SecurityParams);

}