3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211344

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2094 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC multilayer test case 11.4.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Following implementation of agreed prose CR R5-214620, thee new postamble state EMERGENCY_RELEASE_IDLE should be used for this test case. - For state STATE_EMERGENCY_RELEASE_IDLE, the current implementation of f_NR_Postamble() will check for RRCSetupRequest to come at the SRB port but does not handle the message. In the subsequent call to f_NR_UE_DeRegisterOnSwitchOff() the processing will break during IP configuration (during f_IP_ChangeNrCell()) and the RRCSetupRequest will then be handled by the component default handler, causing the test case to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use postamble state STATE_EMERGENCY_RELEASE_IDLE for test case 11.4.6 - Move the handling for STATE_EMERGENCY_RELEASE_IDLE from f_NR_Postamble to f_NR_UE_DeRegisterOnSwitchOff() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | f_TC_11_4_6_NR5GC |
| --- | --- |
| Reason for change | Following implementation of agreed prose CR R5-214620, thee new postamble state EMERGENCY_RELEASE_IDLE should be used for this test case. |
| Summary of change | Use postamble state STATE_EMERGENCY_RELEASE_IDLE |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_11_4_6_NR5GC() runs on NR5GC_PTC { var NAS_PlmnId v_PLMN_cell1; var NAS_PlmnId v_PLMN_cell11; var TrackingAreaCode v_TAC_cell1; var TrackingAreaCode v_TAC_cell11; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) ServiceAreaIdList v_ServiceAreaIdList; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; // System information combination NR-1 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells f_NR5GC_Init(NR_2); // @sic R5s210635 sic@ //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); v_PLMN_cell1 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); v_PLMN_cell11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); v_TAC_cell1 := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAC_cell11 := f_NR_CellInfo_GetTAC (nr_Cell11); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN_cell1, {bit2oct(v_TAC_cell1)}); v_ServiceAreaIdList := cs_NG_ServiceAreaIdList(v_PLMN_cell11, {bit2oct(v_TAC_cell11)}); // Preamble v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13(nr_Cell1); // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), v_TAIList, -, -, -, -, -, -, -, -, -, -, -, v_ServiceAreaIdList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); f_NR_TestBody_Set(true); fl_TC_11_4_6_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_DEREGISTERED); //@sic R5s210635, R5s210770 sic@ }

After Change

function f_TC_11_4_6_NR5GC() runs on NR5GC_PTC { var NAS_PlmnId v_PLMN_cell1; var NAS_PlmnId v_PLMN_cell11; var TrackingAreaCode v_TAC_cell1; var TrackingAreaCode v_TAC_cell11; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) ServiceAreaIdList v_ServiceAreaIdList; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; // System information combination NR-1 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells f_NR5GC_Init(NR_2); // @sic R5s210635 sic@ //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); v_PLMN_cell1 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)); v_PLMN_cell11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); v_TAC_cell1 := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAC_cell11 := f_NR_CellInfo_GetTAC (nr_Cell11); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN_cell1, {bit2oct(v_TAC_cell1)}); v_ServiceAreaIdList := cs_NG_ServiceAreaIdList(v_PLMN_cell11, {bit2oct(v_TAC_cell11)}); // Preamble v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13(nr_Cell1); // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), v_TAIList, -, -, -, -, -, -, -, -, -, -, -, v_ServiceAreaIdList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20 (nr_Cell1, TESTMode_OFF); f_NR_TestBody_Set(true); fl_TC_11_4_6_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_EMERGENCY_RELEASE_IDLE); //@sic R5s210635, R5s210770 sic@ //KS R5sxxx change_11.4.6 }

  - Change 2

| Function name | f_NR_Postamble() |
| --- | --- |
| Reason for change | For state STATE_EMERGENCY_RELEASE_IDLE, the current implementation of this function wll check for RRCSetupRequest to come at the SRB port but does not handle the message. In the subsequent call to f_NR_UE_DeRegisterOnSwitchOff() the processing will break during IP configuration (during f_IP_ChangeNrCell()) and the RRCSetupRequest will then be handled by the component default handler, causing the test case to fail. |
| Summary of change | Move the handling for STATE_EMERGENCY_RELEASE_IDLE from this function to f_NR_UE_DeRegisterOnSwitchOff() |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment | Alternative solution implemented: moved all of the handling for STATE_EMERGENCY_RELEASE_IDLE to this function |

Before Change

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, -, -, p_SRB); //KS R5sxxxx change_common // @sic R5-195241 sic@ } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

After Change

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } /* REMOVE else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } */ else { // All other options already handled in Deregister function f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, -, -, p_SRB); //KS R5sxxxx change_common // @sic R5-195241 sic@ } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

  - Change 3

| Function name | f_NR_UE_DeRegisterOnSwitchOff |
| --- | --- |
| Reason for change | For state STATE_EMERGENCY_RELEASE_IDLE, the current implementation of f_NR_Postamble() will check for RRCSetupRequest to come at the SRB port but does not handle the message. In the subsequent call to f_NR_UE_DeRegisterOnSwitchOff() the processing will break during IP configuration (during f_IP_ChangeNrCell()) and the RRCSetupRequest will then be handled by the component default handler, causing the test case to fail. |
| Summary of change | Move the handling for STATE_EMERGENCY_RELEASE_IDLE from f_NR_Postamble to f_NR_UE_DeRegisterOnSwitchOff() |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment | Alternative solution implemented: removed case statement for STATE_EMERGENCY_RELEASE_IDLE as this is now done in f_NR_Postamble |

Before Change

function f_NR_UE_DeRegisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, boolean p_USIM_Rmvd := false, B2_Type p_AccessType := '01'B, //KS R5sxxxx change_common NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2 ) runs on NR5GC_PTC { var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_MSG_Indication_Type v_DetachReq; var boolean v_RxdMsg := false; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var SubFrameTiming_Type v_TimingOfLocalRelease; timer t_T3540 := f_NR_SetTimerToleranceMax (p_NR_CellId, nasTimer, 10.0); if (p_UE_State == STATE_CONNECTED_3A_T3540){ // @sic R5-201202, R5-201333 sic@ t_T3540.start; // release local configuration f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); t_T3540.timeout; // Now continue with RRC_IDLE procedure } if (pc_SwitchOnOff) { // The UE can actually be switched off f_IP_ChangeNrCell(IP, p_NR_CellId); // @sic R5s210603 sic@ select (p_UE_State) { // @sic R5-195325 sic@ case (STATE_IDLE_1A, STATE_CONNECTED_3A_T3540) { // Already switched off in calling function f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, v_ReceivedMsg); // @sic R5-204389 sic@ v_RxdMsg := true; // @sic R5s190930 sic@ f_NR_RRCRelease(p_NR_CellId); } case (STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ v_DetachReq := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } case (STATE_INACTIVE_2A){ // @sic R5-195325 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ v_ReceivedMsgTemplate := f_NR_RRC_Resume_DefWithNas(p_NR_CellId, tsc_NR_RRC_TI_Def, ?, ?); if (not isvalue(v_ReceivedMsgTemplate)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else { v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, valueof(v_ReceivedMsgTemplate)); // @sic R5-204389 sic@ v_RxdMsg := true; // @sic R5s190930 sic@ f_NR_RRCRelease(p_NR_CellId); } } case (STATE_CONNECTED_3A){ // @sic R5-201202, R5-202562 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s201132 sic@ // DRB already configured, so the UE won't send any NAS msg until it's completed IMS Deregistration //KS R5sxxxx change_common SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, //KS R5sxxxx change_common cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } case(STATE_DEREGISTERED) { // @sic R5-201202 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ // don't need to release the local configuration as RRC Release is always done before this function is called @sic R5s201228 sic@ v_TimingOfLocalRelease := f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); //@sic R5s190984 sic@ f_Delay(0.1*int2float(f_SubFrameTiming_Duration(f_NR_GetCurrentTiming(p_NR_CellId),v_TimingOfLocalRelease))); //@sic R5s210839 sic@ //KS R5sxxxx change_common } } if (v_RxdMsg) { // @sic R5s190930 sic@ if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, p_AccessType))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } } } else { f_UT_PowerOffUE(UT); } // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init(); }

After Change

function f_NR_UE_DeRegisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, boolean p_USIM_Rmvd := false, B2_Type p_AccessType := '01'B, //KS R5sxxxx change_common NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2 ) runs on NR5GC_PTC { var template (omit) NG_NAS_MSG_Indication_Type v_ReceivedMsgTemplate; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_MSG_Indication_Type v_DetachReq; var boolean v_RxdMsg := false; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var SubFrameTiming_Type v_TimingOfLocalRelease; timer t_T3540 := f_NR_SetTimerToleranceMax (p_NR_CellId, nasTimer, 10.0); timer t_Wait := 5.0; if (p_UE_State == STATE_CONNECTED_3A_T3540){ // @sic R5-201202, R5-201333 sic@ t_T3540.start; // release local configuration f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); t_T3540.timeout; // Now continue with RRC_IDLE procedure } if (pc_SwitchOnOff) { // The UE can actually be switched off f_IP_ChangeNrCell(IP, p_NR_CellId); // @sic R5s210603 sic@ select (p_UE_State) { // @sic R5-195325 sic@ case (STATE_IDLE_1A, STATE_CONNECTED_3A_T3540) { // Already switched off in calling function f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, v_ReceivedMsg); // @sic R5-204389 sic@ v_RxdMsg := true; // @sic R5s190930 sic@ f_NR_RRCRelease(p_NR_CellId); } case (STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); t_Wait.start; v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait, ?) if(v_RxdMsg) { v_RxdMsg := false; // 1a3Aa3 send RRC Connection Setup f_NR_RRCSetup_Def (p_NR_CellId); //1a3Aa4 receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, v_ReceivedMsg); v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } } case (STATE_INACTIVE_2A){ // @sic R5-195325 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ v_ReceivedMsgTemplate := f_NR_RRC_Resume_DefWithNas(p_NR_CellId, tsc_NR_RRC_TI_Def, ?, ?); if (not isvalue(v_ReceivedMsgTemplate)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else { v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(p_NR_CellId, valueof(v_ReceivedMsgTemplate)); // @sic R5-204389 sic@ v_RxdMsg := true; // @sic R5s190930 sic@ f_NR_RRCRelease(p_NR_CellId); } } case (STATE_CONNECTED_3A){ // @sic R5-201202, R5-202562 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s201132 sic@ // DRB already configured, so the UE won't send any NAS msg until it's completed IMS Deregistration //KS R5sxxxx change_common SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, //KS R5sxxxx change_common cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } v_RxdMsg := true; f_NR_RRCRelease(p_NR_CellId); } case(STATE_DEREGISTERED) { // @sic R5-201202 sic@ f_UT_SwitchOffUE(UT, p_USIM_Rmvd); // @sic R5s190930 sic@ // don't need to release the local configuration as RRC Release is always done before this function is called @sic R5s201228 sic@ v_TimingOfLocalRelease := f_NR_RRCRelease_Local(p_NR_CellId, f_NR_GetNextSendOccasion_DL(p_NR_CellId)); //@sic R5s190984 sic@ f_Delay(0.1*int2float(f_SubFrameTiming_Duration(f_NR_GetCurrentTiming(p_NR_CellId),v_TimingOfLocalRelease))); //@sic R5s210839 sic@ //KS R5sxxxx change_common } } if (v_RxdMsg) { // @sic R5s190930 sic@ if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, p_AccessType))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } } } else { f_UT_PowerOffUE(UT); } // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init(); }

MCC160 Implementation

function f_NR_Postamble(NR_CellId_Type p_NR_CellId,

NG_ALL_STATE_Type p_UE_State,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC

{

timer t_Wait := 5.0;

var NG_NAS_MSG_Indication_Type v_DetachReq;

var boolean v_RxdMsg := false;

if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@

//The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required".

SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit))));

t_Wait.start;

alt {

// Does the UE transmit a DEREGISTRATION ACCEPT message?

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

p_SRB,

cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered),

cr_NG_DEREGISTRATION_ACCEPT))){

t_Wait.stop;

f_NR_RRCRelease(p_NR_CellId);

}

[] t_Wait.timeout{}

}

f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED);

}

else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@

if (pc_SwitchOnOff) { // The UE can actually be switched off

f_UT_SwitchOffUE(UT, false, CNF_REQUIRED);

t_Wait.start;

v_RxdMsg := f_NR_RRC_SetupRequest_Common(p_NR_CellId, t_Wait);

if (v_RxdMsg) {

f_NR_RRCSetup_Def (p_NR_CellId);

v_DetachReq := f_NR_RRCSetupComplete_Def(p_NR_CellId, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def));

if (not f_Check_NG_DeregistrationReqMOMsg (v_DetachReq.Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

f_NR_RRCRelease(p_NR_CellId);

}

} else {

f_UT_PowerOffUE(UT);

}

} else {

// All other options already handled in Deregister function

f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, p_SRB); // @sic R5-195241, R5s21163 sic@

}

//And now remove the cell(s)

f_NR_ReleaseAllCells();

}