3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211355

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2104 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of 5GMM Rel-16 test case 9.1.9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add 5GMM Rel-16 RACS test case 9.1.9.2 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GMM Rel-16 RACS test case 9.1.9.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

3	Branches executed	10

4	Execution Log Files	10

4.1	SM-G996B	10

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.9.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	9.1.9.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	SM-G996B

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes 2.3, 2.4 and 2.5 of R5s211170 are required and applied.

## Change 1

| Function name | function f_TC_9_1_9_2_NR5GC() |
| --- | --- |
| Reason for change | - As 5GMM capability is non-cleartext IE and hence not included in the initial NAS message without security It should be included in the Registration request piggyback with Security Mode complete message - NW-assigned UE radio capability ID consists of TF + Version ID + RCI.So, if TE set Version ID as 00, UE radio capability ID in the digit expression is as follows: TF + Version ID + RCI = 1 00 00000000001 #draft proseCR would be raised for RAN5#93-e meeting |
| Summary of change | - Removed GMM cap checking from IntialRegistration request - Correct encoded Radio Capability ID to “100000000005” |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | - Alternative solution proposed (see below for implementation): The contents of the Registration Request message is checked after step 9, instead of at step 3. Implementation postponed until outcome of prose CR at RAN5#93-e - Accepted conditional to prose CR agreement at RAN5#93-e. |

Before Change

function f_TC_9_1_9_2_NR5GC() runs on NR5GC_PTC { //RACS / UE configuration update / UE radio capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; f_NR5GC_Init_NAS(NR_2); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); // turn on NR cells f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '100000000005'O) ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF, noRrcConnectionRelease); f_NR_TestBody_Set(true); fl_TC_9_1_9_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); }

After Change

function f_TC_9_1_9_2_NR5GC() runs on NR5GC_PTC { //RACS / UE configuration update / UE radio capability ID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; f_NR5GC_Init_NAS(NR_2); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); // turn on NR cells f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000050'O) ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF, noRrcConnectionRelease); f_NR_TestBody_Set(true); fl_TC_9_1_9_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A); }

## Change 2

| Function name | function fl_TC_9_1_9_2_Body() |
| --- | --- |
| Reason for change | NW-assigned UE radio capability ID consists of TF + Version ID + RCI.So, if TE set Version ID as 00, UE radio capability ID in the digit expression is as follows: TF + Version ID + RCI = 1 00 000000000070 |
| Summary of change | Correct encoded Radio Capability ID |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#93-e. |

Before Change

function fl_TC_9_1_9_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The SS transmits CONFIGURATION UPDATE COMMAND including a new UE Radio Capability ID SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '100000000007'O))))); // @siclog "Step 2" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1"); // @siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); // @siclog "Step 4" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 5" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '100000000007'O))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); f_NR5GC_MobilityRegistration_Steps4_6 (v_NGC_NASCellB, v_GMM_MobilityInfo); } //End of fl_TC_9_1_9_2_Body

After Change

function fl_TC_9_1_9_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 1" siclog@ //The SS transmits CONFIGURATION UPDATE COMMAND including a new UE Radio Capability ID SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, ' 01000000000070 'O))))); // @siclog "Step 2" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1"); // @siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); // @siclog "Step 4" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 5" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, ' 01000000000070 'O))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); f_NR5GC_MobilityRegistration_Steps4_6 (v_NGC_NASCellB, v_GMM_MobilityInfo); } //End of fl_TC_9_1_9_2_Body

## Additional Change Required:

| Function name | function f_TC_9_1_9_2_NR5GC() |
| --- | --- |
| Reason for change | The prose specifies the preamble state is 3N-A |
| Summary of change | Complete the procedure defined in 38.508-1 table 4.5.4.2-3 |
| TTCN module | RACS_NR5GC.ttcn |

function f_TC_9_1_9_2_NR5GC() runs on NR5GC_PTC

{

//RACS / UE configuration update / UE radio capability ID

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN);

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (value) NG_MobileIdentity v_GUTIToSend;

var NG_NAS_GutiParameters_Type v_GutiParams;

var NAS_PlmnId v_PLMN;

var TrackingAreaCode v_TAC;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var template (value) NG_TrackingAreaIdList v_TAIList;

f_NR5GC_Init_NAS(NR_2);

// Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

f_NR_CellConfig_Def(v_NGC_NASCellB);

v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA);

if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS))  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA));

v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA);

v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams);

v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA);

v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)});

// Registration Accept

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -,

-, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000050'O)

);

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF, rrcConnectionRelease);

f_NR5GC_RRC_ConnectedState3N_Def(v_NGC_NASCellA);

f_NR_TestBody_Set(true);

fl_TC_9_1_9_2_Body();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(v_NGC_NASCellB, STATE_IDLE_1A);

}

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## SM-G996B

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_1_9_2_LOG.html

- PICS/PIXIT parameter file:

TC_9_1_9_2_PIXIT.xml

# References

| [1] | R5s211356 : NR5GC FR1 : Supporting infromation for adition of 5GMM Rel-16 test case 9.1.9.2 |
| --- | --- |