3GPP TSG-RAN5 Meeting #2021-TTCN email	 R5s211359

# Online, 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0046 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 6.2.1 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MCPTT TC 6.2.1 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s211360 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Nemergent Solutions Client v7.0	3

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.2.1 which is part of the ATS iwd-TTCN3-B2020-09_D21wk37 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fidel Liberal
	fidel.liberal@ehu.eus

## Verification Test Summary

Test Case: 	6.2.1

ATS Version:	ATS  iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

None.

## Additional Change from MCC160

| Parameter in re-INVITE | Parameter in re-INVITE not checked in TTCN |
| --- | --- |
| Reason for change | In case of re-INVITE, Answer-Mode is not checked correctly: per 36.579-1 Table 5.5.2.5.1-1 it shall be not present |
| Summary of change | Corrections in f_MCPTT_ReInviteMO and f_MCX_InviteRequest_MessageHeaderRX |
| TTCN module | MCX_SIP_CallControl |

- Change:

- function f_MCPTT_ReInviteMO(MCX_CallType_Type p_CallType, - MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, - MCX_MediaControlCondition_Type p_FloorControlConditionSdpOffer := floorControlWithoutImplictGrantRequest, - MCX_MediaControlConditionSdpAnswer_Type p_FloorControlConditionSdpAnswer := implicitFloorRequestNoted, - template (present) MCX_MIME_XmlBodies_Type p_XmlBodies) - runs on MCX_IMS_PTC - { - var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; - var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; - var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX := f_MCPTT_SdpInfo_RX(p_CallType, SDP_OFFER, p_FloorControlConditionSdpOffer); var MCX_CommencementMode_Type v_CommencementMode := NONE; - v_InviteRequestWithSdpAndXml := f_MCPTT_InviteMO_ReceiveINVITE(p_CallType, p_PriorityCallType, v_CommencementMode, -, v_SdpInfoRX, p_XmlBodies, tsc_MCX_ReInvite); - v_InviteRequestWithSdp := v_InviteRequestWithSdpAndXml.InviteRequestWithSdp; - f_IMS_PreliminaryPass(__FILE__, __LINE__, "re-INVITE"); - f_MCX_InviteMO_Send100Trying(v_InviteRequestWithSdp); - f_MCPTT_CallSetupMO_Send200OK_ReceiveACK(v_InviteRequestWithSdp, p_FloorControlConditionSdpAnswer); - f_IMS_PreliminaryPass(__FILE__, __LINE__, "ACK"); - }

function f_MCX_InviteRequest_MessageHeaderRX(MCX_ServiceType_Type p_ServiceType, MCX_SIP_EstabMod_Type p_EstabModType := CALL_ESTAB_MOD, MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, MCX_CommencementMode_Type p_CommencementMode := NONE, template (present) SipUrl p_ExpectedCalleeUri, boolean p_IsReInvite := false) runs on IMS_PTC return template (present) MessageHeader { var template (present) SemicolonParam_List v_MandatoryContactParams := f_MCX_ContactFeatureParams_RX(p_ServiceType, p_EstabModType); var template (present) MessageHeader v_MessageHeader := f_MCX_InviteReferCommon_MessageHeaderRX(p_ServiceType, "INVITE", p_PriorityCallType, p_ExpectedCalleeUri, v_MandatoryContactParams, p_IsReInvite); var template (present) AcceptBody_List v_AcceptBodyList; v_AcceptBodyList[0] := cr_AcceptBody(c_sdpApplication); select (p_EstabModType) { case (CALL_ESTAB_MOD) { v_AcceptBodyList[1] := cr_AcceptBody(f_MCX_ServiceSpecificApplicationType(p_ServiceType)); v_MessageHeader.contentType := cr_ContentTypeMIME; } case (PRE_EST_SESSION_ESTAB_MOD) { // 36.579-1 Table 5.3.3.4-1: Step 8 of Generic Test Procedure for MCPTT pre-established session establishment CO v_MessageHeader.contentType := cr_ContentTypeSDP; } } v_MessageHeader.supported := cr_Supported(tsc_OptionTagList_timer); v_MessageHeader.sessionExpires := cr_SessionExpires; v_MessageHeader.require := cr_RequireSecAgree; v_MessageHeader.accept := cr_Accept(superset(all from v_AcceptBodyList)); // !!!! PROSE ISSUE: clarification to be added to 36.579-1 Table 5.3.3.4-1 and Table 5.5.2.5.1-1 !!!! v_MessageHeader.acceptContact := cr_AcceptContact_Common(f_MCX_AcceptContact_AcValues_RX(p_ServiceType, p_EstabModType)); v_MessageHeader := f_MCX_InviteRefer_MessageHeaderRX_SetAnswerMode(v_MessageHeader, p_CommencementMode); return v_MessageHeader; - }

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SSLog_Coversheet.txt

- Signalling-TC6.2.1.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

# References

| [1] | R5s211360: Supporting information for the addition of MCPTT test case 6.2.1 to the MCX ATS |
| --- | --- |