3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211470

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2109 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-DC test case 8.2.2.9.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In scenarios where a PDCP entity for a radio bearer is configured with more than one associated logical channel (split DRB config), pdcp_Config.moreThanOneRLC is mandatory present as per TS38.331. However, current TTCN configures pdcp_Config with the IE moreThanOneRLC omitted by default even in Split Bearer cases. - The function f_ReconfigureDRBs_NRDC() does not have a way to ONLY send RRCReconfiguration message and NOT wait for a RRCReconfigurationCpomplete reply. Its sub-function f_NR_SendRRCReconfiguration_NRDC() always waits for a RRCReconfigurationComplete. Furthermore, in this function, incoming RRCReconfigurationComplete must have a v1560 extension IE. According to TS 38.523-1 Table 8.2.2.9.2.3.2-1 Steps 2 and 5, a plain RRCReconfigurationComplete is expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Assigned “omit” to the input parameter p_PDCP_ConfigSCG_DRB of the function f_NR5GC_RRC_ConnectedState3N_NRDC when used from the TestBody. This results in the pdcp_Config.moreThanOneRLC IE getting populated when the added bearer is of split-type - (a) For Steps 2 and 5, noRACH_check parameter value is used when calling the function f_ReconfigureDRBs_NRDC, as no RACH procedure is expected when changing only the uplink default path; (b) f_NR_SendRRCReconfiguration_NRDC() modified so that when noRACH_check is required, contents of incoming RRCReconfigurationComplete is not checked; (c) do not wait for RRCReconfigurationComplete at the TestBody. Alternatively, a new parameter may be added to f_NR_SendRRCReconfiguration_NRDC() and f_ReconfigureDRBs_NRDC() as to whether wait for RRCReconfigurationComplete message inside these functions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | f_TC_8_2_2_9_2_NR5GC() |
| --- | --- |
| Reason for change | In scenarios where a PDCP entity for a radio bearer is configured with more than one associated logical channel (split DRB config), pdcp_Config.moreThanOneRLC is mandatory present as per TS38.331. However, current TTCN configures pdcp_Config with the IE moreThanOneRLC omitted by default even in Split Bearer cases. |
| Summary of change | Assigned “omit” to the input parameter p_PDCP_ConfigSCG_DRB of the function f_NR5GC_RRC_ConnectedState3N_NRDC when used from the TestBody. This results in the pdcp_Config.moreThanOneRLC IE getting populated when the added bearer is of split-type |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_2_2_9_2_NR5GC() runs on NR5GC_PTC { // Bearer Modification / Uplink data path / Split DRB Reconfiguration / NR-DC f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); //@sic R5s210772, R5-214751 sic@ // Create and configure NR cell1 and NR cell 10 f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //@sic R5s210891 sic@ f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, PING_Or_TESTModeB_ON); //@sic R5s210772, R5-214751 sic@ f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_Split, PING_Or_TEST_LOOPModeB_ON); f_NR_TestBody_Set(true); fl_TC_8_2_2_9_2_TestBody(); f_NR_TestBody_Set(false); if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_2_2_9_2_NR5GC() runs on NR5GC_PTC { // Bearer Modification / Uplink data path / Split DRB Reconfiguration / NR-DC f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); //@sic R5s210772, R5-214751 sic@ // Create and configure NR cell1 and NR cell 10 f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //@sic R5s210891 sic@ f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, PING_Or_TESTModeB_ON); //@sic R5s210772, R5-214751 sic@ f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_Split, PING_Or_TEST_LOOPModeB_ON, -, -, omit); f_NR_TestBody_Set(true); fl_TC_8_2_2_9_2_TestBody(); f_NR_TestBody_Set(false); if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

  - Change 2

| Function name | fl_TC_8_2_2_9_2_TestBody() |
| --- | --- |
| Reason for change | The RRC Reconfiguration message sent in Steps 1 and 4 will not cause the UE to RACH. The function f_ReconfigureDRBs_NRDC() does not have a way to ONLY send RRCReconfiguration message and NOT wait for a RRCReconfigurationCpomplete reply. Its sub-function f_NR_SendRRCReconfiguration_NRDC() always waits for a RRCReconfigurationComplete. |
| Summary of change | noRACH_Check is used when calling the function f_ReconfigureDRBs_NRDC(). Removed part of the code where RRCReconfigurationComplete is received at the TestBody level |
| TTCN module | RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | Accepted – see change 3 for the proposed implementation |

Before Change:

function fl_TC_8_2_2_9_2_TestBody() runs on NR5GC_PTC { //@sic R5s210772, R5-214751 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) PDCP_Config v_PDCP_Config; var DRB_Identity v_DrbId; var template (value) RadioBearerConfig v_RadioBearerConfig2; var PDUSessionInfoList_Type v_PDUSessionInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG; // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to MCG radio path from SCG radio path v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_DrbId := f_NR_GetDefaultDRB_SCG(); v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_MCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC( nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_MasterKeys, -, -, -, false); // @siclog "Step 2" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using MCG radio path in the uplink? v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 3", -, -, nr_Cell1); // @siclog "Step 4" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to SCG radio path from MCG radio path v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_SCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC(nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_SecondaryKeys, -, -, -, false); // @siclog "Step 5" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using SCG radio path in the uplink? f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 6", -, -, nr_Cell10); }

After Change:

function fl_TC_8_2_2_9_2_TestBody() runs on NR5GC_PTC { //@sic R5s210772, R5-214751 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) PDCP_Config v_PDCP_Config; var DRB_Identity v_DrbId; var template (value) RadioBearerConfig v_RadioBearerConfig2; var PDUSessionInfoList_Type v_PDUSessionInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) SDAP_Config v_SDAP_ConfigSCG; // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to MCG radio path from SCG radio path v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_DrbId := f_NR_GetDefaultDRB_SCG(); v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_MCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC( nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_MasterKeys, -, -, noRACH_Check, false); // @siclog "Step 2" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? // SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using MCG radio path in the uplink? v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 3", -, -, nr_Cell1); // @siclog "Step 4" siclog@ // The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to SCG radio path from MCG radio path v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_SCG); v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config); v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList); f_ReconfigureDRBs_NRDC(nr_Cell1, nr_Cell10, omit, -, omit, v_RadioBearerConfig2, omit, omit, omit, omit, MCG_SCG_SecondaryKeys, -, -, noRACH_Check, false); // @siclog "Step 5" siclog@ // Check: Does the UE transmits an RRCReconfigurationComplete message? //SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); // @siclog "Step 6" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using SCG radio path in the uplink? f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 6", -, -, nr_Cell10); }

  - Change 3

| Function name | f_NR_SendRRCReconfiguration_NRDC |
| --- | --- |
| Reason for change | There must be a way for this function to wait for RRCReconfigurationComplete with no SCG-Response IE included. It is assumed that when no RACH process is expected during the Reconfiguration procedure, the incoming RRCReconfigurationComplete will not have any SCG-response IE included. |
| Summary of change | When noRACH_Check is enabled, wait for RRCReconfigurationComplete with no IE Check. |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted but implemented differently – see below |

Before Change:

function f_NR_SendRRCReconfiguration_NRDC (NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) RadioBearerConfig p_RadioBearerConfig1 := omit, template (omit) RRCReconfiguration_v1530_IEs p_V1530Ext := omit, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg := omit, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_PDUSessionModificationComplete := false ) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //Reconfigure PSCell to enable PRACH indication - UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell. if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); } // Send RRCReconfigurationmessage to UE - Don't wait for RRCReconfigurationComplete as it may come in parallel to the PRACH indication on PSCell f_NR_SendRRCReconfiguration (p_MCG_CellId, p_SRB, p_RadioBearerConfig1, p_V1530Ext, p_NASMsg, -, p_TimingInfo, false); if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ if (p_PDUSessionModificationComplete) { // @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB2, //Anritsu TTCN CR R5s211341 cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } else { // No NAS message expected @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); } else { // no RACH check, so just wait for RRC message @sic R5s210716 sic@ //@sic R5-216300 sic@ SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))); } if (p_PDUSessionModificationComplete) { // @sic R5s210716 sic@ if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } }

After Change:

function f_NR_SendRRCReconfiguration_NRDC (NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (omit) RadioBearerConfig p_RadioBearerConfig1 := omit, template (omit) RRCReconfiguration_v1530_IEs p_V1530Ext := omit, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg := omit, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH, boolean p_PDUSessionModificationComplete := false ) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //Reconfigure PSCell to enable PRACH indication - UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell. if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); } // Send RRCReconfigurationmessage to UE - Don't wait for RRCReconfigurationComplete as it may come in parallel to the PRACH indication on PSCell f_NR_SendRRCReconfiguration (p_MCG_CellId, p_SRB, p_RadioBearerConfig1, p_V1530Ext, p_NASMsg, -, p_TimingInfo, false); if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@ if (p_PDUSessionModificationComplete) { // @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SRB.receive(car_NR_SRB_NasPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB2, //Anritsu TTCN CR R5s211341 cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } else { // No NAS message expected @sic R5-215678 sic@ interleave { //@sic R5-206300 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))) {} [] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {} } } f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); } else { // no RACH check, so just wait for RRC message @sic R5s210716 sic@ //@sic R5-216300 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); } if (p_PDUSessionModificationComplete) { // @sic R5s210716 sic@ if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } }

MCC160 proposed implementation

function f_ReconfigureDRBs_NRDC( NR_CellId_Type p_MCG_CellId,

NR_CellId_Type p_SCG_CellId,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) RadioBearerConfig p_RadioBearerConfig1,

template (omit) RadioBearerConfig p_RadioBearerConfig2,

template (omit) CellGroupConfig p_CellGroupConfigMCG,

template (omit) CellGroupConfig p_CellGroupConfigSCG,

template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG,

template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG,

NRDC_SecurityActivation_Type  p_NRDC_SecurityActivation,

RNTI_Value p_RNTI_Value :=  tsc_C_RNTI_Value4,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1,

ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH,

boolean p_AddingSCG := false,

template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?)))

) runs on NR5GC_PTC

{

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template(omit) SK_Counter v_SK_Counter := omit;

var boolean v_PDUSessionModificationComplete := false;

if (p_AddingSCG) {  // @sic R5s210716 sic@

v_PDUSessionModificationComplete := true;

// This field is always included upon initial configuration of an NR SCG

v_SK_Counter := 0;

}

v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( p_CellGroupConfigSCG, p_CellGroupConfigMCG, p_RadioBearerConfig2, -, v_SK_Counter ); // @sic R5s210716 sic@

//Reconfigure SS

f_ReconfigureSS_DRBs_NRDC( p_MCG_CellId, p_SCG_CellId, p_TimingInfo, p_SS_ConfigMCG, p_SS_ConfigSCG, p_NRDC_SecurityActivation, p_RNTI_Value);

//Send RRCReconfiguration Receive RRCReconfiurationComplete and checl PRACH premable in the SCG cell

f_NR_SendRRCReconfiguration_NRDC(p_MCG_CellId, p_SCG_CellId,

p_RadioBearerConfig1, v_V1530Ext, p_NASMsg, p_TimingInfo,

p_SRB, p_WaitForRachPreamble, v_PDUSessionModificationComplete,   // @sic R5s210716 sic@

p_RRCConnReconfigComplete

);

}

//====================================================================================

function f_NR_SendRRCReconfiguration_NRDC (NR_CellId_Type p_MCG_CellId,

NR_CellId_Type p_SCG_CellId,

template (omit) RadioBearerConfig p_RadioBearerConfig1 := omit,

template (omit) RRCReconfiguration_v1530_IEs p_V1530Ext := omit,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg := omit,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1,

ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH,

boolean p_PDUSessionModificationComplete := false,

template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?)))

) runs on NR5GC_PTC

{ /* @sic R5s211470: added p_RRCConnReconfigComplete sic@ */

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN);

//Reconfigure PSCell to enable PRACH indication - UE performs a C-RNTI based Contention Based Random Access (CBRA) procedure on the PSCell.

if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) {  // @sic R5s210716 sic@

f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable));

}

// Send RRCReconfigurationmessage to UE - Don't wait for RRCReconfigurationComplete as it may come in parallel to the PRACH indication on PSCell

f_NR_SendRRCReconfiguration (p_MCG_CellId, p_SRB, p_RadioBearerConfig1, p_V1530Ext, p_NASMsg, -, p_TimingInfo, false);

if (p_WaitForRachPreamble == reconfigurationWithSync_CheckRACH) { // @sic R5s210716 sic@

if (p_PDUSessionModificationComplete) { // @sic R5-215678 sic@

interleave {

//@sic R5-206300 sic@

[] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, p_RRCConnReconfigComplete)) {}

//@sic R5s2111341 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{}

[] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {}

}

} else { // No NAS message expected @sic R5-215678 sic@

interleave {

//@sic R5-206300 sic@

[] SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, p_RRCConnReconfigComplete)) {}

[] SYSIND.receive(car_NR_PRACH_Preamble_IND(p_SCG_CellId, cr_TimingInfo_Any, ?)) {}

}

}

f_NR_SS_SystemIndCtrlConfig(p_SCG_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable));

} else { // no RACH check, so just wait for RRC message @sic R5s210716 sic@

//@sic R5-216300 sic@

SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_CellId, tsc_NR_RbId_SRB1, p_RRCConnReconfigComplete));

}

if (p_PDUSessionModificationComplete) { // @sic R5s210716 sic@

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed");

}

}

}

//====================================================================================

function fl_TC_8_2_2_9_2_TestBody() runs on NR5GC_PTC

{ //@sic R5s210772, R5-214751 sic@

var template (value) DRB_ToAddModList v_DRBToAddModList;

var template (value) PDCP_Config v_PDCP_Config;

var DRB_Identity v_DrbId;

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var PDUSessionInfoList_Type v_PDUSessionInfo;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN);

var template (value) SDAP_Config v_SDAP_ConfigSCG;

// @siclog "Step 1" siclog@

// The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to MCG radio path from SCG radio path

v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5});

v_DrbId := f_NR_GetDefaultDRB_SCG();

v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_MCG);

v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config);

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList);

f_ReconfigureDRBs_NRDC(nr_Cell1,

nr_Cell10,

omit,

-,

omit,

v_RadioBearerConfig2,

omit,

omit,

omit,

omit,

MCG_SCG_MasterKeys,

-,

-,

-,

false,

cr_38508_RRCReconfigurationComplete);

// @siclog "Step 2" siclog@

// Check: Does the UE transmits an RRCReconfigurationComplete message?

//    SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

// @siclog "Step 3" siclog@

// Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using MCG radio path in the uplink?

v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 3", -, -, nr_Cell1);

// @siclog "Step 4" siclog@

// The SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to change the primaryPath to SCG radio path from MCG radio path

v_PDCP_Config := cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DrbId), tsc_NR_CellGroupId_SCG);

v_DRBToAddModList[0] := cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_DrbId, true_, true_, v_PDCP_Config);

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(omit, v_DRBToAddModList);

f_ReconfigureDRBs_NRDC(nr_Cell1,

nr_Cell10,

omit,

-,

omit,

v_RadioBearerConfig2,

omit,

omit,

omit,

omit,

MCG_SCG_SecondaryKeys,

-,

-,

-,

false,

cr_38508_RRCReconfigurationComplete);

// @siclog "Step 5" siclog@

// Check: Does the UE transmits an RRCReconfigurationComplete message?

//    SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 6" siclog@

// Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on Split DRB#n using SCG radio path in the uplink?

f_NR5GC_CheckDataPath(nr_Cell10, f_NR5GC_GetPdnIndex(v_PDUSessionInfo[0]), dedicatedBearer, v_DrbId, "Step 6", -, -, nr_Cell10);

}