3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211471

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2110 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to TS 38.523-1 Section 7.1.3.0: ++++++++++++++++ Preamble: - The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.3.0-2 and using the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU. ++++++++++++++++ - At Step 5, TTCN constructs the Reconfiguration message using default bearer tsc_NR_DRB2 which is the default bearer used for ENDC cases. This results in an incorrect LCID in the resulting PDCP_Config IE in the Reconfiguration message. - During SCG addition, the RRC Reconfiguration message ends up using the ciphering algorithm intended for MCG because the incorrect function is used when extracting the current SCG ciphering algorithm. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test Loop Mode A is configured during the Preamble. - Used tsc_NR_DRB3 when constructing the Reconfiguration message at step 5, when the test is executed as NR5GC. - Used the function f_NR_AS_CipheringAlgorithm_Get_SCG() in the case where the DRB is MCG_Split |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_5_2_NR5GC() |
| --- | --- |
| Reason for change | - According to TS 38.523-1 Section 7.1.3.0: ++++++++++++++++ Preamble: - The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.3.0-2 and using the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU. ++++++++++++++++ |
| Summary of change | - Test Loop Mode A is configured during the Preamble. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var integer v_SCG_DRB_Id; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18); //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); //run the preamble as normal and bring back UE to IDLE f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100,- , - ,tsc_NR_DRB3)); //@sic R5s210888 sic@ => Will bring UE to connected state, The SCG DRB is configured in normal mode v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18, v_SCG_DRB_Id); //@sic R5s210888 sic@ //Reconfigure SCG DRB PDCP in Test Mode in SS v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, nr_Cell10, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var integer v_SCG_DRB_Id; //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18); //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC( nr_Cell1, nr_Cell10, MCG_Split, TEST_LOOPModeA_ON,-,-,cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b100,- , - ,tsc_NR_DRB3)); //@sic R5s210888 sic@ => Will bring UE to connected state, The SCG DRB is configured in normal mode v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); f_NR_PDCP_Init (v_NR_PDCP_State,PDCP_SNLength18, v_SCG_DRB_Id); //@sic R5s210888 sic@ //Reconfigure SCG DRB PDCP in Test Mode in SS v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell10, v_SCG_DRB_Id, cs_RlcBearerRouting_NR(nr_Cell1), cs_NR_PDCP_RbConfig_TransparentMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell10, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); f_TC_7_1_3_5_2_NR_TestBody(v_NR_PDCP_State, nr_Cell10, v_SCG_DRB_Id); f_NR_TestBody_Set(false); f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_ReleaseAllCells(); }

## Change 2

| Function name | f_TC_7_1_3_5_2_NR_TestBody |
| --- | --- |
| Reason for change | At Step 5, TTCN constructs the Reconfiguration message using default bearer tsc_NR_DRB2 which is the default bearer used for ENDC cases. This results in an incorrect LCID in the resulting PDCP_Config IE in the Reconfiguration message. |
| Summary of change | Used tsc_NR_DRB3 when constructing the Reconfiguration message at step 5, when the test is executed as NR5GC. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_ScellId, integer p_DRB_Id) runs on NR_BASE_PTC { … //@sic R5-204426 sic@ //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes (Note 1). if (v_IsENDC){ //@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b0,- ,-, tsc_NR_DRB3))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); } else {//@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,b0))}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ } …

After Change:

function f_TC_7_1_3_5_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_ScellId, integer p_DRB_Id) runs on NR_BASE_PTC { … //@sic R5-204426 sic@ //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes (Note 1). if (v_IsENDC){ //@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,-,b0))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); } else {//@sic R5-205575 sic@ v_DRB_ToAddModList := {cs_NR_DRB_ToAddMod(omit, p_DRB_Id, -, -, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits, tsc_NR_CellGroupId_MCG,b0,-,-, tsc_NR_DRB3))}; v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(omit, omit, v_RadioBearerConfig2); //SS transmits an RRCReconfiguration message containing NR RadioBearerConfig to modify PDCP discardTimer value of SCG DRB. f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext);//@sic R5s210888 sic@ } …

## Change 3

| Function name | f_NR_RRCReconfig_AddSCG_DRBs_NRDC |
| --- | --- |
| Reason for change | During SCG addition, the RRC Reconfiguration message ends up using the ciphering algorithm intended for MCG because the incorrect function is used when extracting the current SCG ciphering algorithm. |
| Summary of change | Used the function f_NR_AS_CipheringAlgorithm_Get_SCG() in the case where the DRB is MCG_Split |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_RRCReconfig_AddSCG_DRBs_NRDC(…) runs on NR5GC_PTC { … case(MCG_Split) { //MCG DRB reconfigured with Split - New SCG DRB is split SN terminated //@sic R5s210888 sic@ if (isvalue(p_PDCP_Config)) { v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_SCG_DRB_Id, p_PDCP_Config, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); } else { v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, v_CellGroupIdSCG), secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); } … }

After Change:

function f_NR_RRCReconfig_AddSCG_DRBs_NRDC(…) runs on NR5GC_PTC { … case(MCG_Split) { //MCG DRB reconfigured with Split - New SCG DRB is split SN terminated //@sic R5s210888 sic@ if (isvalue(p_PDCP_Config)) { v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_SCG_DRB_Id, p_PDCP_Config, secondary, f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()); } else { v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(cs_NR_DRB_SDAP(v_SDAP_ConfigSCG), v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, v_CellGroupIdSCG), secondary, f_NR_AS_CipheringAlgorithm_Get_SCG(), f_NR_AS_IntegrityAlgorithm_Get_SCG()); } … }