3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211486

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2121 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC multilayer test case 11.4.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - During the MO speech call steps, the IMS PTC will wait for a trigger from IPCAN when voice bearer is established. There is a potential race condition if UPDATE is received from the UE before this trigger. - During the execution of 11.4.8 multiple consecutive IMS MO and emergency calls are executed. For a UE supporting S1 mode, the TTCN will call the function f_GetNextEPSBearerId()) to allocate 2 new EPS bearer IDs to associate with the QoS flows for each call (once during f_NR5GC_PDUSessionEstablishment_InnerLoop() and once during f_NR5GC_ModifyPDUSession_AddVoice()). There is currently no method to re-use allocated EPS bearer Ids, so after a few IMS calls the allocated EPS bearer ID will exceed the valid range (1-15) and a run-time error occurs during f_Get_NG_PDUSessionEstablishmentAccept() |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove trigger IPCAN->IMS during MO call setup - Fix EPS bearer IDs associated with consecutive IMS calls |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_IMS_NR5GC_MOSpeechCall_Step2ToEnd() |
| --- | --- |
| Reason for change | During the MO speech call steps, the IMS PTC will wait for a trigger from IPCAN when voice bearer is established. There is a potential race condition if UPDATE is received from the UE before this trigger. |
| Summary of change | Remove wait for trigger so that IMS PTC is ready to recieve UPDATE (see also change 2). |
| TTCN module | IMS_NR5GC_Procedures.ttcn |
| MCC160 Comment | Accepted in principle, but keep waiting for the trigger if not pc_Preconditions |

Before Change

function f_IMS_NR5GC_MOSpeechCall_Step2ToEnd(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := p_InviteRequestWithSdp; if (pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_5(v_InviteRequestWithSdp); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_5(v_InviteRequestWithSdp); } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); if (pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_12(v_InviteRequestWithSdp); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step6_8(v_InviteRequestWithSdp); } // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function f_IMS_NR5GC_MOSpeechCall_Step2ToEnd(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := p_InviteRequestWithSdp; if (pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_5(v_InviteRequestWithSdp); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_5(v_InviteRequestWithSdp); } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //REMOVED f_IMS_IPCAN_WaitForTrigger(IPCAN); if (pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_12(v_InviteRequestWithSdp); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step6_8(v_InviteRequestWithSdp); } // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

## Change 2

| Function name | f_NR5GC_SpeechCall() |
| --- | --- |
| Reason for change | During the MO speech call steps, the IMS PTC will wait for a trigger from IPCAN when voice bearer is established. There is a potential race condition if UPDATE is received from the UE before this trigger. |
| Summary of change | Remove the sending of trigger after PDU session modification (see also change 1). Note : This can be removed because the function is currently only used for MO call setup. If the function is also used for MT calls then the trigger will need to be sent for MT case only. |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment | Accepted in principle, but still send the trigger if not pc_Preconditions |

Before Change

function f_NR5GC_SpeechCall(NR_CellId_Type p_NR_CellId, charstring p_NumToDial) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_UT_RequestIMSCall(UT, p_NumToDial); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Establishment: Step 6"); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

After Change

function f_NR5GC_SpeechCall(NR_CellId_Type p_NR_CellId, charstring p_NumToDial) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_UT_RequestIMSCall(UT, p_NumToDial); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Establishment: Step 6"); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); //REMOVED f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

## Change 3

| Function name | f_NR5GC_ModifyPDUSession_AddVoice () |
| --- | --- |
| Reason for change | During the execution of 11.4.8 multiple consecutive IMS MO and emergency calls are executed. For a UE supporting S1 mode, the TTCN will call the function f_GetNextEPSBearerId()) to allocate 2 new EPS bearer IDs to associate with the QoS flows (once during f_NR5GC_PDUSessionEstablishment_InnerLoop() and once during f_NR5GC_ModifyPDUSession_AddVoice()). There is currently no method to re-use allocated EPS bearer Ids, so after a few IMS calls the allocated EPS bearer ID will exceed the valid range (1-15) and a run-time error occurs during f_Get_NG_PDUSessionEstablishmentAccept() |
| Summary of change | Remove the call to f_GetNextEPSBearerId and hardcode the EPS bearer ID associated with voice QoS flow to 6. This aligns with ensures each consecutive voice call uses the same EPS beare ID value which aligns with 38.508-1 table 4.8.3.2-5. |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment | Alternative solution implemented: passed additional parameter IPCAN_SpeechCall to f_GetNextEPSBearerId |

Before Change

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var boolean v_InterworkWithEPS := false; var O1_Type v_EPS_BearerId := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPS_BearerId := hex2oct(int2hex(valueof(f_GetNextEPSBearerId (p_GSM_MobilityInfo, v_GMM_MobilityInfo)), 1) & '0'H); // @sic R5-216161 sic@ } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS, v_EPS_BearerId)})); // @sic R5-216161 sic@ v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, ms80, omit))}); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

After Change

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var boolean v_InterworkWithEPS := false; var O1_Type v_EPS_BearerId := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var integer v_NewDRBId; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) QosFlow_Identification_Type v_QosFlowId; if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPS_BearerId := ‘60’H; } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules("7_Voice", '7A'O), // @sic R5s210073 sic@ cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS, v_EPS_BearerId)})); // @sic R5-216161 sic@ v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false); // @sic R5-211171 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5 v_NewDRBId, -, -, cs_38508_PDCP_Config (-, -, -, ms80, omit))}); // UM as per 38.508-1 Table 4.6.3-99: PDCP-Config @sic R5-213445 sic@ v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7}))))}; // @sic R5-211371, R5s210330 sic@ v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_38508_LogicalChannelConfig_DRB)}; v_QosFlowId := cs_IP_QosFlowId(oct2int(p_GSM_MobilityInfo.SessionId), 7); // @sic R5s210363 sic@ f_IP_RtpRtcpLoopback(IP, p_GSM_MobilityInfo.PdnIndex, tsc_IMS_MediaPort_M1, cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s210363 sic@ // then send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment"); } }

## Change 4

| Function name | f_GetNextEPSBearerId() |
| --- | --- |
| Reason for change | During the execution of 11.4.8 multiple consecutive IMS MO and emergency calls are executed. For a UE supporting S1 mode, the TTCN will call the function f_GetNextEPSBearerId()) to allocate 2 new EPS bearer IDs to associate with the QoS flows (once during f_NR5GC_PDUSessionEstablishment_InnerLoop() and once during f_NR5GC_ModifyPDUSession_AddVoice()). There is currently no method to re-use allocated EPS bearer Ids, so after a few IMS calls the allocated EPS bearer ID will exceed the valid range (1-15) and a run-time error occurs during f_Get_NG_PDUSessionEstablishmentAccept() |
| Summary of change | Extend the check for IMS_DNN to include EMERGENCY_DNN. This means EPS bearer ID 5 will be allocated for QoS flow 2 for consecutive IMS emergency calls, aligned with 38.508-1 4.8.2.3-2. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted in principle, but extended this to hardcode values for all media types too |

Before Change

function f_GetNextEPSBearerId (GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo) runs on NR5GC_PTC return template (omit) integer { var template (omit) integer v_BearerId := omit; var IntegerList_Type v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); var integer v_NoOfIds := lengthof(v_ExistingIds); var integer i; if (isvalue(p_GMM_MobilityInfo.GMMCap)) { if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_BearerId := 5; for (i:= 0; i < v_NoOfIds; i :=i+1) { if (match (v_ExistingIds[i], 5)) { // IMS Bearer id = 5 v_BearerId := v_NoOfIds + (6-1); // allocate the next one, from 6 onwards break; } } } else { // Not IMS v_BearerId := v_NoOfIds + 6; } // Add the new id to the end of the list and store it v_ExistingIds[v_NoOfIds] := valueof(v_BearerId); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } return v_BearerId; }

After Change

function f_GetNextEPSBearerId (GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo) runs on NR5GC_PTC return template (omit) integer { var template (omit) integer v_BearerId := omit; var IntegerList_Type v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); var integer v_NoOfIds := lengthof(v_ExistingIds); var integer i; if (isvalue(p_GMM_MobilityInfo.GMMCap)) { if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { if (p_GSM_MobilityInfo.PDUType == IMS_DNN or p_GSM_MobilityInfo.PDUType == Emergency_PDN)) { // @sic R5-205939 sic@ v_BearerId := 5; for (i:= 0; i < v_NoOfIds; i :=i+1) { if (match (v_ExistingIds[i], 5)) { // IMS Bearer id = 5 v_BearerId := v_NoOfIds + (6-1); // allocate the next one, from 6 onwards break; } } } else { // Not IMS v_BearerId := v_NoOfIds + 6; } // Add the new id to the end of the list and store it v_ExistingIds[v_NoOfIds] := valueof(v_BearerId); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } return v_BearerId; }

## MCC160 Implementation

function f_GetNextEPSBearerId (GSM_MobilityInfo_Type p_GSM_MobilityInfo, GMM_MobilityInfo_Type p_GMM_MobilityInfo, template (omit) IMS_TestConfiguration_Type p_MediaType := omit) runs on NR5GC_PTC return template (omit) integer { var template (omit) integer v_BearerId := omit; var IntegerList_Type v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); var integer v_NoOfIds := lengthof(v_ExistingIds); var integer i; if (isvalue(p_GMM_MobilityInfo.GMMCap)) { if (p_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { if (isvalue(p_MediaType)) { // check to see if we can hardcode the dedicated bearer first select (valueof(p_MediaType)) { case (IPCAN_SpeechCall) { v_BearerId := 7; } case (IPCAN_VideoCall) { v_BearerId := 8; } case (IPCAN_EmergencyCall) { v_BearerId := 9; } } } else // now allocate id according to default bearer type if (p_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-205939 sic@ v_BearerId := 5; for (i:= 0; i < v_NoOfIds; i :=i+1) { if (match (v_ExistingIds[i], 5)) { // IMS Bearer id = 5 v_BearerId := v_NoOfIds + (6-1); // allocate the next one, from 6 onwards break; } } } else if (p_GSM_MobilityInfo.PDUType == Emergency_PDN) v_BearerId := 6; } else { // Not IMS or emergency v_BearerId := v_NoOfIds + 10; } } // Add the new id to the end of the list and store it v_ExistingIds[v_NoOfIds] := valueof(v_BearerId); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } return v_BearerId; }

## Change 5

| Function name | fl_TC_11_4_8_NR5GC_TestBody() |
| --- | --- |
| Reason for change | During the execution of 11.4.8 multiple consecutive IMS MO and emergency calls are executed. For a UE supporting S1 mode, the TTCN will call the function f_GetNextEPSBearerId()) to allocate 2 new EPS bearer IDs to associate with the QoS flows (once during f_NR5GC_PDUSessionEstablishment_InnerLoop() and once during f_NR5GC_ModifyPDUSession_AddVoice()). There is currently no method to re-use allocated EPS bearer Ids, so after a few IMS calls the allocated EPS bearer ID will exceed the valid range (1-15) and a run-time error occurs during f_Get_NG_PDUSessionEstablishmentAccept() Note : Even after the changes 3 and 4, the function f_GetNextEPSBearerId() (called from f_NR5GC_PDUSessionEstablishment_InnerLoop()) will still allocate a new EPS bearer ID (starting from 5) for each emergency PDU session which still leads to the value exceeding its range after a few consecutive emergency calls. The below proposal resolves the issue in this test case itself, another solution may be to remove the incrementing of EPS bearer IDs altogether and use hardcoded IDs for signalling, voice and video QoS flows (to allow for multiple consecutive IMS calls). |
| Summary of change | Remove an EPS bearer ID from the global list after each emergency call. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment | Not needed |

Before Change

function fl_TC_11_4_8_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_12 := f_NR_CellInfo_GetPLMN (nr_Cell12); // @sic R5s210810 sic@ var TrackingAreaCode v_TAC_12 := f_NR_CellInfo_GetTAC (nr_Cell12); // @sic R5s210810 sic@ var NAS_PlmnId v_PLMN_12 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_12); // @sic R5s210810 sic@ var template (value) NG_MobileIdentity v_GUTI_12 := f_NG_GutiParameters2MobileIdentity (v_PLMN_12, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12)); // @sic R5s210810 sic@ var template (value) NG_TrackingAreaIdList v_TAIList_12 := cs_NG_TAIListNonConsecutive(v_PLMN_12, {bit2oct(v_TAC_12)}); // @sic R5s210810 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var EmergencyNumList v_LocalNumList := {'120'H, '121'H, '122'H, '123'H, '124'H, '125'H, '126'H, '127'H, '128'H, '129'H}; var EmergencyNumList v_ExtdNumList := {'130'H, '131'H, '132'H, '133'H, '134'H, '135'H, '136'H, '137'H, '138'H, '139'H, '140'H, '141'H, '142'H, '143'H, '144'H, '145'H, '146'H, '147'H, '148'H, '149'H}; // @sic R5-216203 sic@ var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 var integer i; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210810 sic@ //@siclog "Step 2" siclog@ f_NR5GC_RRC_IdleState1N_Def (nr_Cell1); //@siclog "Steps 3 - 7" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 4"); //@siclog "Steps 8 - 12" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 9"); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 13" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (10, v_LocalNumList); v_XtdEmergNumList := f_Build_ExtdEmergNumList (20, v_ExtdNumList, '0'B); // @sic R5s210810 sic@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, // @sic R5s210810 sic@ v_GMM_MobilityInfo, v_GUTI_12, v_TAIList_12, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell12, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 14 - 18" siclog@ for (i := 0; i < 10; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_LocalNumList[i]), "Step 15, i = " & int2str(i)); } //@siclog "Steps 19 - 23" siclog@ for (i := 0; i < 20; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_ExtdNumList[i]), "Step 20, i = " & int2str(i)); }

After Change

function fl_TC_11_4_8_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN_12 := f_NR_CellInfo_GetPLMN (nr_Cell12); // @sic R5s210810 sic@ var TrackingAreaCode v_TAC_12 := f_NR_CellInfo_GetTAC (nr_Cell12); // @sic R5s210810 sic@ var NAS_PlmnId v_PLMN_12 := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN_12); // @sic R5s210810 sic@ var template (value) NG_MobileIdentity v_GUTI_12 := f_NG_GutiParameters2MobileIdentity (v_PLMN_12, f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12)); // @sic R5s210810 sic@ var template (value) NG_TrackingAreaIdList v_TAIList_12 := cs_NG_TAIListNonConsecutive(v_PLMN_12, {bit2oct(v_TAC_12)}); // @sic R5s210810 sic@ var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) EmergNumList v_EmergNumList; // emergency numbers in format as per 24.008 var EmergencyNumList v_LocalNumList := {'120'H, '121'H, '122'H, '123'H, '124'H, '125'H, '126'H, '127'H, '128'H, '129'H}; var EmergencyNumList v_ExtdNumList := {'130'H, '131'H, '132'H, '133'H, '134'H, '135'H, '136'H, '137'H, '138'H, '139'H, '140'H, '141'H, '142'H, '143'H, '144'H, '145'H, '146'H, '147'H, '148'H, '149'H}; // @sic R5-216203 sic@ var template (value) ExtdEmergNumList v_XtdEmergNumList; // emergency numbers in format as per 24.301 var integer i; var IntegerList_Type v_ExistingIds; var integer v_NoOfIds; var IntegerList_Type v_empty; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210810 sic@ //@siclog "Step 2" siclog@ f_NR5GC_RRC_IdleState1N_Def (nr_Cell1); //@siclog "Steps 3 - 7" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "120", "Step 4"); //@siclog "Steps 8 - 12" siclog@ fl_MOSpeechCallAndRelease(nr_Cell1, "10", "Step 9"); f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 13" siclog@ // Steps 1 - 3 // RRC Registration Req v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13 Registration Request Message Failed"); } v_EmergNumList := f_Build_EmergNumList (10, v_LocalNumList); v_XtdEmergNumList := f_Build_ExtdEmergNumList (20, v_ExtdNumList, '0'B); // @sic R5s210810 sic@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, // @sic R5s210810 sic@ v_GMM_MobilityInfo, v_GUTI_12, v_TAIList_12, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_EmergNumList, v_XtdEmergNumList); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(nr_Cell12, tsc_NR_RbId_SRB1); //Step 6 f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 14 - 18" siclog@ for (i := 0; i < 10; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_LocalNumList[i]), "Step 15, i = " & int2str(i)); //Remove the EPS Bearer ID from the global list v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); if(lengthof(v_ExistingIds) > 0) { v_NoOfIds := lengthof(v_ExistingIds) - 1; v_empty := {}; v_ExistingIds := replace(v_ExistingIds, v_NoOfIds, 1, v_empty); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } } //@siclog "Steps 19 - 23" siclog@ for (i := 0; i < 20; i := i+1) { fl_EmergencyCallAndRelease(nr_Cell12, hex2str(v_ExtdNumList[i]), "Step 20, i = " & int2str(i)); //Remove the EPS Bearer ID from the global list v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); if(lengthof(v_ExistingIds) > 0) { v_NoOfIds := lengthof(v_ExistingIds) - 1; v_empty := {}; v_ExistingIds := replace(v_ExistingIds, v_NoOfIds, 1, v_empty); f_NR5GC_MobileInfo_SetEPS_BearerIdList(v_ExistingIds); } }