3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211495

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0747 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for IMS 5GS registration test case 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following the UE-initiated de-registration for IMS services at steps 23-29 of test case 6.4, a few optional UE behaviours are possible. The UE may consider that the PDU session established for IMS is no longer needed so may transmit PDU SESSION RELEASE REQUEST. Alternatively a UE which does not have any active non-IMS PDU sessions may trigger deregistration from 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handle signalling for UE-triggered PDU session release or 5GS de-registration. This needs to be handled in IPCAN main loop so that IPCAN can still continue to handle requests coming from IMS PTC. For UE initiated de-registration, a variable is added to track the 5GS regsistration state so that the correct state can be used on IPCAN release. Note : An associated prose CR on 34.229-5 will be raised at RAN5#93 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	6

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_IPCAN_MainLoop() |
| --- | --- |
| Reason for change | Following the UE-initiated de-registration for IMS services at steps 23-29 of test case 6.4, a few optional UE behaviours are possible. The UE may consider that the PDU session established for IMS is no longer needed so may transmit PDU SESSION RELEASE REQUEST. Alternatively a UE which does not have any active non-IMS PDU sessions may trigger deregistration from 5GS. |
| Summary of change | Handle signalling for UE-triggered PDU session release or 5GS de-registration. This needs to be handled in IPCAN main loop so that IPCAN can still continue to handle requests coming from IMS PTC. For UE initiated de-registration, a variable is added to track the 5GS regsistration state so that the correct state can be used on IPCAN release. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_IPCAN_MainLoop() runs on NR5GC_PTC { var NR_CellId_Type v_CellId := tsc_IMS_NR5GC_CellId; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG; var IMS_IPCAN_Command_Type v_Command; var IMS_TestConfiguration_Type v_TestConfiguration; var TriggerResult_Type v_StartProcedureResult; var integer v_PdnIndex; var boolean v_ConnectionEstablished := false; var boolean v_Done := false; while (not v_Done) { alt { [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS1; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS2; } } v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); v_TestConfiguration := v_Command.TestConfiguration; } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_ConnectionEstablished); v_ConnectionEstablished := true; } case (IPCAN_ENDPROCEDURE) { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_TestConfiguration, v_Command.TestProcedure, v_StartProcedureResult); /* => RRC connection is released */ v_ConnectionEstablished := false; } case (IPCAN_RELEASE) { f_NR5GC_IPCAN_Release(IMS[v_PdnIndex], v_CellId); v_Done := true; } case (IPCAN_QUERY) { /* @sic R5s130133 additional changes: handling of queries from the IMS PTC sic@ */ IMS[v_PdnIndex].send(cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); } case else { FatalError(__FILE__, __LINE__, "invalid command"); } } } } }

After Change:

function f_NR5GC_IPCAN_MainLoop() runs on NR5GC_PTC { var NR_CellId_Type v_CellId := tsc_IMS_NR5GC_CellId; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG; var IMS_IPCAN_Command_Type v_Command; var IMS_TestConfiguration_Type v_TestConfiguration; var TriggerResult_Type v_StartProcedureResult; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_PdnIndex; var boolean v_ConnectionEstablished := false; var boolean v_Done := false; var boolean v_IPCAN_Registered := false; while (not v_Done) { alt { [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS1; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS2; } [v_ConnectionEstablished]SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp { if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList)) { if (f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR5GC_PDUSessionRelease(v_CellId, v_GSM_MobilityInfo.SessionId); } } else if (f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { v_IPCAN_Registered := false; } repeat; } } v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); v_TestConfiguration := v_Command.TestConfiguration; } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_ConnectionEstablished); v_ConnectionEstablished := true; if ( v_Command.TestProcedure == IPCAN_InitialRegistration) { v_IPCAN_Registered := true; } } case (IPCAN_ENDPROCEDURE) { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_TestConfiguration, v_Command.TestProcedure, v_StartProcedureResult); /* => RRC connection is released */ v_ConnectionEstablished := false; } case (IPCAN_RELEASE) { f_NR5GC_IPCAN_Release(IMS[v_PdnIndex], v_CellId, v_IPCAN_Registered); v_Done := true; } case (IPCAN_QUERY) { /* @sic R5s130133 additional changes: handling of queries from the IMS PTC sic@ */ IMS[v_PdnIndex].send(cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); } case else { FatalError(__FILE__, __LINE__, "invalid command"); } } } } }

## 2.2 Change 2

| Function name | f_NR5GC_IPCAN_MainLoop() |
| --- | --- |
| Reason for change | Following the UE-initiated de-registration for IMS services at steps 23-29 of test case 6.4, a few optional UE behaviours are possible. The UE may consider that the PDU session established for IMS is no longer needed so may transmit PDU SESSION RELEASE REQUEST. Alternatively a UE which does not have any active non-IMS PDU sessions may trigger deregistration from 5GS. |
| Summary of change | Update the function to allow for the case that the UE is already de-registered from 5GS before IPCAN_RELEASE is processed. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted. See below for additional changes – the UE has already deregistered for IMS, so will not do so again |

Before Change:

function f_NR5GC_IPCAN_Release(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId ) runs on NR5GC_PTC { f_NR_Postamble(p_NR_CellId, STATE_IDLE_1A); f_IMS_IPCAN_SendCoOrdMsg(p_Port); }

After Change:

function f_NR5GC_IPCAN_Release(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId, boolean p_IPCAN_Registered ) runs on NR5GC_PTC { if (p_IPCAN_Registered) { f_NR_Postamble(p_NR_CellId, STATE_IDLE_1A); } else { f_NR_Postamble(p_NR_CellId, STATE_DEREGISTERED); } f_IMS_IPCAN_SendCoOrdMsg(p_Port); }

## MCC160 Implementation:

type enumerated IMS_TestProcedure_Type {      /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */

IPCAN_InitialRegistration,                  /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.2.3 without RRC Connection Release at the end of the procedure

* NOTE: As working assumption the UE does IMS REGISTRATION automatically after RRC/NAS registration */

IPCAN_EmergencyCall_NormalService,          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3 */

IPCAN_EmergencyCall_NoRegistration,         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3, but with Call Release according to 34.229-1 C.32a */

IPCAN_EmergencyCall_LimitedService,         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.5.3 */

IPCAN_MO_SpeechCall,                        /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.6.3 */

IPCAN_MT_SpeechCall,                        /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.7.3 */

IPCAN_MO_VideoCall,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.8.3 */

IPCAN_MT_VideoCall,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.9.3 */

IPCAN_MO_AddVideo,                          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.11.3 */

IPCAN_MT_AddVideo,                          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.12.3 */

IPCAN_ReleaseVideo,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.15.3 */

IPCAN_MO_IMS_Signalling,                    /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 with m = n = 0; used e.g. for MT SMS test case 18.2 */

IPCAN_MT_IMS_Signalling,                    /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 Steps 3 to 9 with m = n = 0; used e.g. for MT SMS test case 18.1 */

IPCAN_XCAP_Signalling,                       /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.14 */

IPCAN_AdditionalIPAddress,                   /* CR 213400 */

IPCAN_Deregistration

};

function f_IMS_CC_Postamble(template (omit) IMS_TestProcedure_Type p_TestProcedure := omit,

template (omit) IMS_TCP_CloseHandling_Type p_TcpCloseHandling := omit,

float p_DelayForDL_SIP := 2.0) runs on IMS_PTC

{ /* @sic R5s140801: new parameter p_TcpCloseHandling, R5s170028: removed parameter p_WaitForDeRegistration sic@ */

/* @sic R5s190785: new parameter p_DelayForDL_SIP sic@ */

var boolean v_AllowRetransmission := fl_IMS_PTC_CheckRanType(UTRAN_FDD) or fl_IMS_PTC_CheckRanType(UTRAN_TDD);

var float v_DelayForDL_SIP := p_DelayForDL_SIP;

if (isvalue(p_TestProcedure)) {

f_IMS_CC_ReleaseConnection(valueof(p_TestProcedure), p_TcpCloseHandling, v_DelayForDL_SIP); /* @sic R5s140801: p_TcpCloseHandling, R5s190785 ch. 2.2 sic@ */

IPCAN.send(cms_IMS_IPCAN_ReleaseReq);

if (valueof(p_TestProcedure) != IPCAN_Deregistration) { // @sic R5s211495 sic@

f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission);  /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */

}

} else {

IPCAN.send(cms_IMS_IPCAN_ReleaseReq);

f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission);  /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */

}

function f_TC_6_4_NR5GC_IMS1() runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var SipUrl v_ContactUrl;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

// @siclog "Step 2 - 9" siclog@

v_IMS_REGISTER_REQ := f_IMS_Registration();

f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration);

// @siclog "Step 10 - 12" Void siclog@

// @siclog "Step 13 - 14"

f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling);

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl();

f_IMS_Register_Notify(v_ContactUrl, f_IMS_XmlMessageForNotify_Terminated(v_IMS_REGISTER_REQ.Request.Register), cs_SubscriptionState_Terminated);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14");

f_IMS_GlobalRelease();   /* @sic R5s210666: no change of public user ids according to R5-212127 sic@ */

// @siclog "Step 15 - 22" siclog@

f_IMS_Registration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15 - 22");

f_IMS_CC_ReleaseConnection(IPCAN_MT_IMS_Signalling);

// @siclog "Step 23" siclog@

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

f_UT_IMS_Deregister(MMI);

// @siclog "Step 24 - 29" siclog@

f_IMS_Deregistration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 24 - 29");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble( IPCAN_Deregistration);

}