3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211509

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2138 | rev | - | Current version: | 17.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 3GPP TS 38.331: "5.3.1.1 RRC connection control RRC connection establishment involves theestablishment of SRB1. The network completes RRC connection establishment priorto completing the establishment of the NG connection, i.e. prior to receivingthe UE context information from the 5GC. Consequently, AS security is notactivated during the initial phase of the RRC connection. During this initialphase of the RRC connection, the network may configure the UE to performmeasurement reporting, but the UE only sends the corresponding measurementreports after successful AS security activation. However, the UE only acceptsare-configuration with sync message when AS security has been activated. Upon receiving the UE contextfrom the 5GC,the RAN activates AS security (both ciphering and integrityprotection) using the initial AS security activation procedure. The RRCmessages to activate AS security (command and successful response) are integrityprotected, while ciphering is started only after completion of the procedure.That is, the response to the message used to activate AS security is notciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs)are both integrity protected and ciphered. After having initiated the initialAS security activation procedure, the network may initiate the establishment ofSRB2 and DRBs, i.e.the network may do this prior to receiving the confirmationof the initial AS security activation from the UE. In any case, the networkwill apply both ciphering and integrity protection for the RRC reconfigurationmessages used to establish SRB2 and DRBs. The network should release the RRCconnection if the initial AS security activation and/ or the radio bearerestablishment fails. Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported. This means that the only existing DRB cannot be released without releasing the RRCConnection in step1 sending the PDUSessionReleaseCommand. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configure DRB2 and DRB3 for UE and release DRB2 at step1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections requireds	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk37related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections requireds

## Change 1

| Function name | f_TC_10_1_2_2_NR5GC |
| --- | --- |
| Reason for change | According to 3GPP TS 38.331: "5.3.1.1 RRC connection control RRC connection establishment involves theestablishment of SRB1. The network completes RRC connection establishment priorto completing the establishment of the NG connection, i.e. prior to receivingthe UE context information from the 5GC. Consequently, AS security is notactivated during the initial phase of the RRC connection. During this initialphase of the RRC connection, the network may configure the UE to performmeasurement reporting, but the UE only sends the corresponding measurementreports after successful AS security activation. However, the UE only acceptsare-configuration with sync message when AS security has been activated. Upon receiving the UE contextfrom the 5GC,the RAN activates AS security (both ciphering and integrityprotection) using the initial AS security activation procedure. The RRCmessages to activate AS security (command and successful response) are integrityprotected, while ciphering is started only after completion of the procedure.That is, the response to the message used to activate AS security is notciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs)are both integrity protected and ciphered. After having initiated the initialAS security activation procedure, the network may initiate the establishment ofSRB2 and DRBs, i.e.the network may do this prior to receiving the confirmationof the initial AS security activation from the UE. In any case, the networkwill apply both ciphering and integrity protection for the RRC reconfigurationmessages used to establish SRB2 and DRBs. The network should release the RRCconnection if the initial AS security activation and/ or the radio bearerestablishment fails. Aconfiguration with SRB2 without DRB or with DRB without SRB2 is not supported(i.e., SRB2 and at least one DRB must be configured in the same RRCReconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 withoutDRB is supported. This means that the only existing DRB cannot be released without releasing the RRCConnection in step1 sending the PDUSessionReleaseCommand. |
| Summary of change | Configure DRB2 and DRB3 for UE and release DRB2 at step1. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment | Postponed. R5-217733 is raised for similar issue, so awaiting the outcome of that Prose CR. |

Before change

function f_TC_10_1_2_2_NR5GC() runs on NR5GC_PTC

{

//Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE / Invalid PDU session identity

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

f_NR5GC_Init_NAS(NR_1);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

//Preamble

//The UE is in 5GS state 3N-A

f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A);

//Main test body

fl_TC_10_1_2_2_Body();

// Finish

f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A);

}

After change

//Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE / Invalid PDU session identity

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var DRB_Identity v_DRBId1;

var DRB_Identity v_DRBId2;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (omit)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) DRB_ToAddModList v_DRBToAddModList;

var template (value) SDAP_Config v_SDAP_Config_DRB;

var template (value) SDAP_Config v_SDAP_Config_DRB2;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

f_NR5GC_Init_NAS(NR_1);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

//Preamble

//The UE is in 5GS state 3N-A

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A);

v_DRBId1 := 2;

v_DRBId2 := 3;

//@sic r5s190783 sic@

v_SS_Drb_ConfigList  := {

f_NR_GetRadioBearerConfig_FullAM(v_NGC_NASCellA, v_DRBId1),

f_NR_GetRadioBearerConfig_FullAM(v_NGC_NASCellA, v_DRBId2)

};

v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1,

cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(v_NGC_NASCellA)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(v_NGC_NASCellA))),

cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100));

v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2,

cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(v_NGC_NASCellA)), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(v_NGC_NASCellA))),

cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100));

//@sic R5s190347 r5-1905696 r5s191054 sic@

v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession());

// v_SDAP_Config_DRB.defaultDRB := false;

v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := {1};

v_SDAP_Config_DRB2 := v_SDAP_Config_DRB;

v_SDAP_Config_DRB2.defaultDRB := false;

v_SDAP_Config_DRB2.mappedQoS_FlowsToAdd := {2};

v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB,

v_DRBId1,

cs_38508_PDCP_Config(ms1500,-,-,omit,omit)),

cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB2,

v_DRBId2,

cs_38508_PDCP_Config(ms1500,-,-,omit,omit)) };

// Step 1 Page UE

f_NR_Paging_Def(nr_Cell1, RRC_IDLE);

v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1);

// Steps 5-6

// RRC Security Mode procedure

f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); // Use NasCount returned from Service Req

// Steps 7-8

f_NRL2_ReconfigExistingAndAddNewDRBs( nr_Cell1,

{cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))},  // @sic R5s201387 Baseline Moving sic@

v_RLC_BearerConfigList,

v_DRBToAddModList,

v_SS_Drb_ConfigList);

//Main test body

fl_TC_10_1_2_2_Body();

// Finish

f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A);

}