3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211515

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0148 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_OTDOA_CheckEutraBand |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, the function f_POS_OTDOA_CheckEutraBand checks for the value of: v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 v_OtdoaProvideCapabilities.numberOfRXantennas_r14 without checking first that such fields are actually present. The lack of boolean pics parameters to evaluate the presence of those fields leads to fail test case verdicts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the following Boolean pics parameters: pc_OTDOA_maxSupportedPrsBandwidth pc_OTDOA_maxSupportedPrsConfigs pc_OTDOA_numberOfRXantennas And implement a presence check before checking the value of the fields aforementioned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case if not reporting any of the fields mentioned in the reason for change |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1 Adding OTDOA pics parameters to POS_Parameters.ttcn	2

1.2 Correction to f_POS_OTDOA_CheckEutraBand	3

## 1.1 Adding OTDOA pics parameters to POS_Parameters.ttcn

| Function name | POS_Parameters.ttcn |
| --- | --- |
| Reason for change | In the current TTCN implementation, there are no pics included to check the presence of maxSupportedPrsBandwidth, maxSupportedPrsConfigs and numberOfRXantennas fields. |
| Summary of change | Introduce new pics parameters: pc_OTDOA_maxSupportedPrsBandwidth pc_OTDOA_maxSupportedPrsConfigs pc_OTDOA_numberOfRXantennas A Prose CR will be raised for this |
| TTCN module | POS_Parameters.ttcn |
| MCC160 Comment | Alternative solution proposed below. See change 1.2. |

Before

<<SKIPPED CODE>>

modulepar boolean                     pc_OTDOA_additional_prs_config := false;        // @desc  Support of additional-prs-config

// Reference: 37.571-3 Table A.4.3-3A/5

modulepar boolean                     pc_OTDOA_densePrsConfig     := false;           // @desc  Support of densePrsConfig

// Reference: 37.571-3 Table A.4.3-3A/8

modulepar boolean                     pc_OTDOA_idleStateForMeasurements := false;     // @desc  Support of idleStateForMeasurements

// Reference: 37.571-3 Table A.4.3-3A/15

modulepar boolean                     pc_OTDOA_interFreqRSTDmeasurement := false;     // @desc  Support of interFreqRSTDmeasurement

// Reference: 37.571-3 Table A.4.3-3A/1

modulepar boolean                     pc_OTDOA_interRAT_RSTDmeasurement := false;     // @desc  Support of interRAT-RSTDmeasurement

// Reference: 37.571-3 Table A.4.3-3A/18

modulepar boolean                     pc_OTDOA_motionMeasurements := false;           // @desc  Support of motionMeasurements

// Reference: 37.571-3 Table A.4.3-3A/17

modulepar boolean                     pc_OTDOA_multiPrbNprs       := false;           // @desc  Support of multiPrbNprs

// Reference: 37.571-3 Table A.4.3-3A/14

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

modulepar boolean                     pc_OTDOA_additional_prs_config := false;        // @desc  Support of additional-prs-config

// Reference: 37.571-3 Table A.4.3-3A/5

modulepar boolean                     pc_OTDOA_densePrsConfig     := false;           // @desc  Support of densePrsConfig

// Reference: 37.571-3 Table A.4.3-3A/8

modulepar boolean                     pc_OTDOA_idleStateForMeasurements := false;     // @desc  Support of idleStateForMeasurements

// Reference: 37.571-3 Table A.4.3-3A/15

modulepar boolean                     pc_OTDOA_interFreqRSTDmeasurement := false;     // @desc  Support of interFreqRSTDmeasurement

// Reference: 37.571-3 Table A.4.3-3A/1

modulepar boolean                     pc_OTDOA_interRAT_RSTDmeasurement := false;     // @desc  Support of interRAT-RSTDmeasurement

// Reference: 37.571-3 Table A.4.3-3A/18

modulepar boolean                     pc_OTDOA_maxSupportedPrsBandwidth := false;     // @desc  Support of maxSupportedPrsBandwidth

// Reference: 37.571-3 Table A.4.3-3A/9

modulepar boolean                     pc_OTDOA_maxSupportedPrsConfigs := false;       // @desc  Support of maxSupportedPrsConfigs

// Reference: 37.571-3 Table A.4.3-3A/12

modulepar boolean                     pc_OTDOA_motionMeasurements := false;           // @desc  Support of motionMeasurements

// Reference: 37.571-3 Table A.4.3-3A/17

modulepar boolean                     pc_OTDOA_multiPrbNprs       := false;           // @desc  Support of multiPrbNprs

// Reference: 37.571-3 Table A.4.3-3A/14

modulepar boolean                     pc_OTDOA_numberOfRXantennas := false;           // @desc  Support of numberOfRXantennas

// Reference: 37.571-3 Table A.4.3-3A/16

<<SKIPPED CODE>>

## 1.2 Correction to f_POS_OTDOA_CheckEutraBand

| Function name | f_POS_OTDOA_CheckEutraBand |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_POS_OTDOA_CheckEutraBand checks for the value of: v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 v_OtdoaProvideCapabilities.numberOfRXantennas_r14 without checking first that such fields are actually present. This could lead into fail verdicts if UE does not report any of the 3 aforementioned fields. |
| Summary of change | Introduce a first check of the presence of the aforementioned fields based on the value of the pics from change 1.1 |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment | Alternative solution proposed below. |

Before

<<SKIPPED CODE>>

function f_POS_OTDOA_CheckEutraBand(OTDOA_ProvideCapabilities  p_OtdoaProvideCapabilities) runs on EUTRA_PTC

{ // @sic R5-145736 sic@

var OTDOA_ProvideCapabilities v_OtdoaProvideCapabilities;

var integer i;

var integer v_BandEUTRA;

var integer v_BitStringIndex;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var bitstring v_SuppEutraBandsAccToPICS_v9a0 := f_DeriveSuppEutraBandsFromPics_v9a0(); // @sic R5-134907 sic@

var bitstring v_SuppEutraBandsAccToPICS_Chunk;         // @sic R5-150483 sic@

v_OtdoaProvideCapabilities := p_OtdoaProvideCapabilities;

if (not isvalue(v_OtdoaProvideCapabilities.supportedBandListEUTRA)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "SupportedBandListEUTRA not included in OTDOA capabilities"); // @sic R5s150589 sic@

}

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

// Check only the bands for which there are PICS; do not check other bands which may or may not be reported

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 0, 14); // bands 1 - 14

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 0, 14);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 16, 30); // bands 17 - 46   @sic R5s150589, R5-165506, R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 16, 30); // @sic R5s150589, R5-165506, R5s170733 sic@

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 47, 1); // band 48 @sic R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 47, 1); //@sic R5s170733 sic@

if (lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA) == maxFBI) {  // @sic R5-134907 sic@

//Check supportedBandListEUTRA-v9a0

if (isvalue (v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0)) {

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

for (i := 0; i < lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0); i := i + 1) { //@sic Band Issue sic@

v_BandEUTRA := v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0[i].bandEUTRA_v9a0;

v_BitStringIndex := v_BandEUTRA - 1;

if (not f_Bitstring_BitIsSet(v_SuppEutraBandsAccToPICS_v9a0, v_BitStringIndex)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_BandEUTRA));

}

}

}

}

//Checking of interFreqRSTDmeasurement-r10.  @sic R5-134907 sic@

if (pc_InterFreq_RSTD_WithGaps) {

if (not(v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10 == supported)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "interFreqRSTDmeasurement_r10 not reported")

}

}

//Checking of OTDOA measurements

//@sic R5-201624 sic@

if (pc_OTDOA_interFreqRSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interFreqRSTDmeasurement_r10")

}

if (pc_OTDOA_additionalNeighbourCellInfoList != ispresent (v_OtdoaProvideCapabilities.additionalNeighbourCellInfoList_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalNeighbourCellInfoList_r10")

}

if (pc_OTDOA_prs_id != ispresent (v_OtdoaProvideCapabilities.prs_id_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_id_r14")

}

if (pc_OTDOA_tp_separation_via_muting != ispresent (v_OtdoaProvideCapabilities.tp_separation_via_muting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for tp_separation_via_muting_r14")

}

if (pc_OTDOA_additional_prs_config != ispresent (v_OtdoaProvideCapabilities.additional_prs_config_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additional_prs_config_r14")

}

if (pc_OTDOA_prs_based_tbs != ispresent (v_OtdoaProvideCapabilities.prs_based_tbs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_based_tbs_r14")

}

if (pc_OTDOA_additionalPathsReport != ispresent (v_OtdoaProvideCapabilities.additionalPathsReport_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalPathsReport_r14")

}

if (pc_OTDOA_densePrsConfig != ispresent (v_OtdoaProvideCapabilities.densePrsConfig_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for densePrsConfig_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14) and (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 == pc_OTDOA_maxSupportedPrsBandwidth_r14))){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsBandwidth_r14")

}

if (pc_OTDOA_prsOccGroup != ispresent (v_OtdoaProvideCapabilities.prsOccGroup_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsOccGroup_r14")

}

if (pc_OTDOA_prsFrequencyHopping != ispresent (v_OtdoaProvideCapabilities.prsFrequencyHopping_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsFrequencyHopping_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14) and (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 == pc_OTDOA_maxSupportedPrsConfigs_r14))){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsConfigs_r14")

}

if (pc_OTDOA_periodicalReporting != ispresent (v_OtdoaProvideCapabilities.periodicalReporting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for periodicalReporting_r14")

}

if (pc_OTDOA_multiPrbNprs != ispresent (v_OtdoaProvideCapabilities.multiPrbNprs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for multiPrbNprs_r14")

}

if (pc_OTDOA_idleStateForMeasurements != ispresent (v_OtdoaProvideCapabilities.idleStateForMeasurements_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for idleStateForMeasurements_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.numberOfRXantennas_r14) and (v_OtdoaProvideCapabilities.numberOfRXantennas_r14 == pc_OTDOA_numberOfRXantennas_r14))){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for numberOfRXantennas_r14")

}

if (pc_OTDOA_motionMeasurements != ispresent (v_OtdoaProvideCapabilities.motionMeasurements_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for motionMeasurements_r15")

}

if (pc_OTDOA_interRAT_RSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interRAT_RSTDmeasurement_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interRAT_RSTDmeasurement_r15")

}

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_OTDOA_CheckEutraBand(OTDOA_ProvideCapabilities  p_OtdoaProvideCapabilities) runs on EUTRA_PTC

{ // @sic R5-145736 sic@

var OTDOA_ProvideCapabilities v_OtdoaProvideCapabilities;

var integer i;

var integer v_BandEUTRA;

var integer v_BitStringIndex;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var bitstring v_SuppEutraBandsAccToPICS_v9a0 := f_DeriveSuppEutraBandsFromPics_v9a0(); // @sic R5-134907 sic@

var bitstring v_SuppEutraBandsAccToPICS_Chunk;         // @sic R5-150483 sic@

v_OtdoaProvideCapabilities := p_OtdoaProvideCapabilities;

if (not isvalue(v_OtdoaProvideCapabilities.supportedBandListEUTRA)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "SupportedBandListEUTRA not included in OTDOA capabilities"); // @sic R5s150589 sic@

}

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

// Check only the bands for which there are PICS; do not check other bands which may or may not be reported

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 0, 14); // bands 1 - 14

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 0, 14);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 16, 30); // bands 17 - 46   @sic R5s150589, R5-165506, R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 16, 30); // @sic R5s150589, R5-165506, R5s170733 sic@

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 47, 1); // band 48 @sic R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 47, 1); //@sic R5s170733 sic@

if (lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA) == maxFBI) {  // @sic R5-134907 sic@

//Check supportedBandListEUTRA-v9a0

if (isvalue (v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0)) {

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

for (i := 0; i < lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0); i := i + 1) { //@sic Band Issue sic@

v_BandEUTRA := v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0[i].bandEUTRA_v9a0;

v_BitStringIndex := v_BandEUTRA - 1;

if (not f_Bitstring_BitIsSet(v_SuppEutraBandsAccToPICS_v9a0, v_BitStringIndex)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_BandEUTRA));

}

}

}

}

//Checking of interFreqRSTDmeasurement-r10.  @sic R5-134907 sic@

if (pc_InterFreq_RSTD_WithGaps) {

if (not(v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10 == supported)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "interFreqRSTDmeasurement_r10 not reported")

}

}

//Checking of OTDOA measurements

//@sic R5-201624 sic@

if (pc_OTDOA_interFreqRSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interFreqRSTDmeasurement_r10")

}

if (pc_OTDOA_additionalNeighbourCellInfoList != ispresent (v_OtdoaProvideCapabilities.additionalNeighbourCellInfoList_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalNeighbourCellInfoList_r10")

}

if (pc_OTDOA_prs_id != ispresent (v_OtdoaProvideCapabilities.prs_id_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_id_r14")

}

if (pc_OTDOA_tp_separation_via_muting != ispresent (v_OtdoaProvideCapabilities.tp_separation_via_muting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for tp_separation_via_muting_r14")

}

if (pc_OTDOA_additional_prs_config != ispresent (v_OtdoaProvideCapabilities.additional_prs_config_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additional_prs_config_r14")

}

if (pc_OTDOA_prs_based_tbs != ispresent (v_OtdoaProvideCapabilities.prs_based_tbs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_based_tbs_r14")

}

if (pc_OTDOA_additionalPathsReport != ispresent (v_OtdoaProvideCapabilities.additionalPathsReport_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalPathsReport_r14")

}

if (pc_OTDOA_densePrsConfig != ispresent (v_OtdoaProvideCapabilities.densePrsConfig_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for densePrsConfig_r14")

}

if (pc_OTDOA_maxSupportedPrsBandwidth != ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14)){

if (not (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 == pc_OTDOA_maxSupportedPrsBandwidth_r14)){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsBandwidth_r14")

}

}

if (pc_OTDOA_prsOccGroup != ispresent (v_OtdoaProvideCapabilities.prsOccGroup_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsOccGroup_r14")

}

if (pc_OTDOA_prsFrequencyHopping != ispresent (v_OtdoaProvideCapabilities.prsFrequencyHopping_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsFrequencyHopping_r14")

}

if (pc_OTDOA_maxSupportedPrsConfigs!= ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14)){

if (not (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 == pc_OTDOA_maxSupportedPrsConfigs_r14)){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsConfigs_r14")

}

}

if (pc_OTDOA_periodicalReporting != ispresent (v_OtdoaProvideCapabilities.periodicalReporting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for periodicalReporting_r14")

}

if (pc_OTDOA_multiPrbNprs != ispresent (v_OtdoaProvideCapabilities.multiPrbNprs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for multiPrbNprs_r14")

}

if (pc_OTDOA_idleStateForMeasurements != ispresent (v_OtdoaProvideCapabilities.idleStateForMeasurements_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for idleStateForMeasurements_r14")

}

if (pc_OTDOA_numberOfRXantennas != ispresent (v_OtdoaProvideCapabilities.numberOfRXantennas_r14)){

if (not (v_OtdoaProvideCapabilities.numberOfRXantennas_r14 == pc_OTDOA_numberOfRXantennas_r14)){ //@sic R5-214181 sic@

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for numberOfRXantennas_r14")

}

}

if (pc_OTDOA_motionMeasurements != ispresent (v_OtdoaProvideCapabilities.motionMeasurements_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for motionMeasurements_r15")

}

if (pc_OTDOA_interRAT_RSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interRAT_RSTDmeasurement_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interRAT_RSTDmeasurement_r15")

}

}

<<SKIPPED CODE>>

MCC TF160 proposed implementation:

function f_POS_OTDOA_CheckEutraBand(OTDOA_ProvideCapabilities  p_OtdoaProvideCapabilities) runs on EUTRA_PTC

{ // @sic R5-145736 sic@

var OTDOA_ProvideCapabilities v_OtdoaProvideCapabilities;

var integer i;

var integer v_BandEUTRA;

var integer v_BitStringIndex;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var bitstring v_SuppEutraBandsAccToPICS_v9a0 := f_DeriveSuppEutraBandsFromPics_v9a0(); // @sic R5-134907 sic@

var bitstring v_SuppEutraBandsAccToPICS_Chunk;         // @sic R5-150483 sic@

v_OtdoaProvideCapabilities := p_OtdoaProvideCapabilities;

if (not isvalue(v_OtdoaProvideCapabilities.supportedBandListEUTRA)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "SupportedBandListEUTRA not included in OTDOA capabilities"); // @sic R5s150589 sic@

}

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

// Check only the bands for which there are PICS; do not check other bands which may or may not be reported

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 0, 14); // bands 1 - 14

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 0, 14);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 16, 30); // bands 17 - 46   @sic R5s150589, R5-165506, R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 16, 30); // @sic R5s150589, R5-165506, R5s170733 sic@

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 47, 1); // band 48 @sic R5s170733 sic@

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 47, 1); //@sic R5s170733 sic@

if (lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA) == maxFBI) {  // @sic R5-134907 sic@

//Check supportedBandListEUTRA-v9a0

if (isvalue (v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0)) {

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

for (i := 0; i < lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0); i := i + 1) { //@sic Band Issue sic@

v_BandEUTRA := v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0[i].bandEUTRA_v9a0;

v_BitStringIndex := v_BandEUTRA - 1;

if (not f_Bitstring_BitIsSet(v_SuppEutraBandsAccToPICS_v9a0, v_BitStringIndex)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_BandEUTRA));

}

}

}

}

//Checking of interFreqRSTDmeasurement-r10.  @sic R5-134907 sic@

if (pc_InterFreq_RSTD_WithGaps) {

if (not(v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10 == supported)) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "interFreqRSTDmeasurement_r10 not reported")

}

}

//Checking of OTDOA measurements

//@sic R5-201624 sic@

if (pc_OTDOA_interFreqRSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interFreqRSTDmeasurement_r10")

}

if (pc_OTDOA_additionalNeighbourCellInfoList != ispresent (v_OtdoaProvideCapabilities.additionalNeighbourCellInfoList_r10)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalNeighbourCellInfoList_r10")

}

if (pc_OTDOA_prs_id != ispresent (v_OtdoaProvideCapabilities.prs_id_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_id_r14")

}

if (pc_OTDOA_tp_separation_via_muting != ispresent (v_OtdoaProvideCapabilities.tp_separation_via_muting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for tp_separation_via_muting_r14")

}

if (pc_OTDOA_additional_prs_config != ispresent (v_OtdoaProvideCapabilities.additional_prs_config_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additional_prs_config_r14")

}

if (pc_OTDOA_prs_based_tbs != ispresent (v_OtdoaProvideCapabilities.prs_based_tbs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_based_tbs_r14")

}

if (pc_OTDOA_additionalPathsReport != ispresent (v_OtdoaProvideCapabilities.additionalPathsReport_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalPathsReport_r14")

}

if (pc_OTDOA_densePrsConfig != ispresent (v_OtdoaProvideCapabilities.densePrsConfig_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for densePrsConfig_r14")

}

if (ispresent(v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14)) {

if (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 != pc_OTDOA_maxSupportedPrsBandwidth_r14) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsBandwidth_r14")

}

}

if (pc_OTDOA_prsOccGroup != ispresent (v_OtdoaProvideCapabilities.prsOccGroup_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsOccGroup_r14")

}

if (pc_OTDOA_prsFrequencyHopping != ispresent (v_OtdoaProvideCapabilities.prsFrequencyHopping_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsFrequencyHopping_r14")

}

if (ispresent(v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14)) {

if (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 != pc_OTDOA_maxSupportedPrsConfigs_r14) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsConfigs_r14")

}

}

if (pc_OTDOA_periodicalReporting != ispresent (v_OtdoaProvideCapabilities.periodicalReporting_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for periodicalReporting_r14")

}

if (pc_OTDOA_multiPrbNprs != ispresent (v_OtdoaProvideCapabilities.multiPrbNprs_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for multiPrbNprs_r14")

}

if (pc_OTDOA_idleStateForMeasurements != ispresent (v_OtdoaProvideCapabilities.idleStateForMeasurements_r14)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for idleStateForMeasurements_r14")

}

if (ispresent(v_OtdoaProvideCapabilities.numberOfRXantennas_r14)) { //@sic R5s211515 sic@

if (v_OtdoaProvideCapabilities.numberOfRXantennas_r14 != pc_OTDOA_numberOfRXantennas_r14) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for numberOfRXantennas_r14")

}

}

if (pc_OTDOA_motionMeasurements != ispresent (v_OtdoaProvideCapabilities.motionMeasurements_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for motionMeasurements_r15")

}

if (pc_OTDOA_interRAT_RSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interRAT_RSTDmeasurement_r15)){

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interRAT_RSTDmeasurement_r15")

}

}