3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211528

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2149 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR_Postamble |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function for the state STATE_EMERGENCY_RELEASE_CONNECTED, the SS transmit a De-registration REQ, and the expectation is that the UE will transmit a De-registration ACCEPT. This behavior is fine in situation where the UE has performed normal registration, and then has performed an emergency call establishment. However, for a situation like NR testcase 11.4.3, where the UE has only performed Registration for emergency, this is not fully applicable. Some devices in such situation(like NR testcase 11.4.3) will not consider the registration to be a “normal” one, and hence will transmit a 5GMM STATUS message upon reception of De-registration REQ. In other words, the UE may think that its not possible to De-register since the registration never happened. Moreover, the 24.50 cl 5.5.2.3 does not have any text that deals with a scenario where the UE is registered for emergency and has now received a De-registration REQ. This means that some UE implementations may consider that after the Registration for emergency, it is OK to expect a De-registration REQ from the SS, and thus transmit a De-registration Accept (38.523-3 cl 10.3.7 Step 3a1), but some may consider this to be a message received in incompatible protocol state and transmit a 5GMM STATUS message. The current TTCN des not handle this. This needs to be handled. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the implementation to handle the 5GMM STATUS message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function f_NR_Postamble	3

1.2	template cr_NG_GMM_STATUS	7

# Corrections required

## Correction to function f_NR_Postamble

| Function name | f_NR_Postamble |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function for the state STATE_EMERGENCY_RELEASE_CONNECTED, the SS transmit a De-registration REQ, and the expectation is that the UE will transmit a De-registration ACCEPT. This behavior is fine in situation where the UE has performed normal registration, and then has performed an emergency call establishment. However, for a situation like NR testcase 11.4.3, where the UE has only performed Registration for emergency, this is not fully applicable. Some devices in such situation(like NR testcase 11.4.3) will not consider the registration to be a “normal” one, and hence will transmit a 5GMM STATUS message upon reception of De-registration REQ. In other words, the UE may think that its not possible to De-register since the registration never happened. Moreover, the 24.50 cl 5.5.2.3 does not have any text that deals with a scenario where the UE is registered for emergency and has now received a De-registration REQ. This means that some UE implementations may consider that after the Registration for emergency, it is OK to expect a De-registration REQ from the SS, and thus transmit a De-registration Accept (38.523-3 cl 10.3.7 Step 3a1), but some may consider this to be a message received in incompatible protocol state and transmit a 5GMM STATUS message. The current TTCN des not handle this. This needs to be handled. A Prose CR will be raised for this. |
| Summary of change | Updated the implementation to handle the 5GMM STATUS message. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function //WA#9_1_5_1_15 f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); // @sic R5-195241 sic@ f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, -, -, p_SRB); //WA#9_1_5_1_15 } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

After Change:

function f_NR_Postamble(NR_CellId_Type p_NR_CellId, NG_ALL_STATE_Type p_UE_State, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { timer t_Wait := 5.0; if (p_UE_State == STATE_EMERGENCY_RELEASE_CONNECTED) { // @sic R5-214620 sic@ //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration not required". SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '0'B), omit)))); t_Wait.start; alt { // Does the UE transmit a DEREGISTRATION ACCEPT message? [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))){ t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,//WA#11_4_3 Prose CR needed p_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_GMM_STATUS(?)))) { t_Wait.stop; f_NR_RRCRelease(p_NR_CellId); } [] t_Wait.timeout{} } f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_DEREGISTERED); } else if (p_UE_State == STATE_EMERGENCY_RELEASE_IDLE) { // @sic R5-214620 sic@ if (pc_SwitchOnOff) { // The UE can actually be switched off f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); } [] t_Wait.timeout{} } } else { f_UT_PowerOffUE(UT); } } else { // All other options already handled in Deregister function //WA#9_1_5_1_15 f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State); // @sic R5-195241 sic@ f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, p_UE_State, -, -, p_SRB); //WA#9_1_5_1_15 } //And now remove the cell(s) f_NR_ReleaseAllCells(); }

## template cr_NG_GMM_STATUS

| Function name | cr_NG_GMM_STATUS |
| --- | --- |
| Reason for change | New template (Please see change 1.1 above) |
| Summary of change | New template |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Accepted |

template (present) NG_NAS_UL_Message_Type cr_NG_GMM_STATUS (template (present) GMM_GSM_Cause p_Cause) := { /*WA#11_4_3 */ /* 24.501 cl. 8.3.16 */ gmm_Status := { protocolDiscriminator := tsc_EPD_GMM, /* cl. 9.2 M V 1 */ spareHalfOctet := tsc_SpareHalfOctet, /* cl. 9.3 M V 1/2 */ securityHeaderType := tsc_SHT_NoSecurityProtection, messageType := tsc_MT_NG_GMM_Status, /* cl. 9.7 M V 1 */ gmmCause := p_Cause /* cl. 9.11.4.2 M V 1 IEI=59 */ } };