3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211536

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2152 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS Fallback test case 11.1.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EPS Fallback TC 11.1.9 MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to EPS Fallback TC 11.1.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	8

3	Branches Executed	9

4	Execution Log Files	9

4.1	Qualcomm SM 8450 + SDX65	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 11.1.9 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.9

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

Change 2 of R5s211309 is also required for the verification of 11.1.9 test case.

## Change 1

| Function name | f_TC_11_1_9_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | - As Expected in Prose Step 14 the re-dirict info of EUTRA_CELL1 need to include in RrcRelease and NR5GC PCT includes the re-dirict info in RrcRelease but Eutra PTC doesn’t sends |
| Summary of change | - Sent the Frequency info of eutra cell1 from Eutra PTC to NR5GC PTC |
| TTCN module | fl_TC_11_1_9_NR5GC_EUTRA_TestBody() |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_11_1_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); … //Postamble is in Eutra f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_9_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; var EUTRA_NR_CoOrd_SysInfo_Type v_SysInfoNR; f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // Send EUTRA frequency to NR //f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 15 - 20" siclog@ v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, noRrcConnectionRelease, mo_VoiceCall_v1280, -, -, -, -, true); // @sic R5s210149 sic@ … //Postamble is in Eutra f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 2

| Function name | f_TC_11_1_9_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | set voiceServiceCauseIndication in SIB2 of target EUTRA Cell signalled for setting MO-voice call cause |
| Summary of change | - Set voiceServiceCauseIndication in SIB2 |
| TTCN module | EUTRA_SysInfo_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_11_1_9_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); … f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change

function f_TC_11_1_9_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create and configure cell f_EUTRA_CellInfo_SetSysInfo_voiceServiceCauseIndication(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell1); …. f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

# Branches Executed

This test case was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n41 using the Ciphering Algorithm nea1 and Integrity Algorithm nia1.

# Execution Log Files

## Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_11_1_9_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\TC_11_1_9_PICS.txt

# References

| [1] | R5s211535 - Supporting information for addition of EPS Fallback test case 11.1.9 |
| --- | --- |