3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211566

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2163 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ENDC test case 7.1.1.4.2.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EN-DC test case 7.1.1.4.2.4 to the IWD_21wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.4.2.4 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.4.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	f_TC_7_1_1_4_2_4_ENDC_EUTRA	4

2.2	Correction to f_TC_7_1_1_4_2_4_ENDC_NR	5

2.3	f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody changes mentioned in R5s211564 (7.1.1.4.2.4.NR5GC submission) is required.	9

3	Branches executed	9

4	Execution Log Files	9

4.1	HiSi Balong 5000 UE	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of EN-DC test case 7.1.1.4.2.4 which is part of the EN-DC test suite in iWD_TTCN3-B2020-09_D21wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.4.2.4.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	HiSi Balong 5000

Verification Status:	PASS

# Corrections required

## f_TC_7_1_1_4_2_4_ENDC_EUTRA

| Function name | f_TC_7_1_1_4_2_4_ENDC_EUTRA |
| --- | --- |
| Reason for change | In function f_TC_7_1_1_4_x_x_ENDC_EUTRA the parameter is used to decide to perform RRCReconfiguration procedure for switching to use raType0. Similar to test case 7.1.1.4.2.3 the reconfiguration procedure can be decided based on pics pc_dynamicSwitchRA_Type0_1_PUSCH |
| Summary of change | Modified the parameter to use pc_ra_Type0_PUSCH and (not pc_dynamicSwitchRA_Type0_1_PUSCH to send RRCReconfiguration procedure for dynamic switching. |
| TTCN module | MAC_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_4_ENDC_EUTRA() runs on EUTRA_5GS_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled f_TC_7_1_1_4_x_x_ENDC_EUTRA(true );//@sic R5-202610 sic@ }

After Change:

function f_TC_7_1_1_4_2_4_ENDC_EUTRA() runs on EUTRA_5GS_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled f_TC_7_1_1_4_x_x_ENDC_EUTRA(pc_ra_Type0_PUSCH and (not pc_dynamicSwitchRA_Type0_1_PUSCH) );//@sic R5-202610 sic@ //WA#WI=zzz dynamic switch based on pics }

## Correction to f_TC_7_1_1_4_2_4_ENDC_NR

| Function name | f_TC_7_1_1_4_2_4_ENDC_NR |
| --- | --- |
| Reason for change | Accoring to 38.214 sec 5.1.3.1 when qam256 is configured the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and in Table 5.1.3.1-2 MCS is defined upto 27 and the rest are marked as reserved. |
| Summary of change | Updated the function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody to define a max mcs as a parameter and value 27 is passed for this test case |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType1); if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { //@siclog Steps 5aA1-5aa2 siclog@ f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); // Now send octetstrings to EUTRA f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType0); } f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType1, 27); // max mcs for table Table 5.1.3.1-2 is 27); if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { //@siclog Steps 5aA1-5aa2 siclog@ f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); // Now send octetstrings to EUTRA f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType0, 27 ); //WA#WI=878120 max mcs for table Table 5.1.3.1-2 is 27); } f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody

## Changes mentioned in R5s211564 (7.1.1.4.2.4.NR5GC submission) are required.

# Branches executed

## This EN-DC TC was executed in Band combination “3A_n78” using NEA2/NIA2  algorithms.

# Execution Log Files

## HiSi Balong 5000 UE

The HiSi Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_4_2_4_ENDC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s211567: Supporting information for addition of ENDC TC 7.1.1.4.2.4 in FR1. |
| --- | --- |