3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211595

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2179 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add RRC test case 8.1.6.1.1.1 in FR1 to the IWD_21wk37 ATS for NR5GC FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to RRC test case 8.1.6.1.1.1 required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-216826 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	7

4	Execution Log Files	7

4.1	Qualcomm SM 8450+SDX65	7

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.6.1.1.1 in FR1 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry

shaun.harry@keysight.com

Narendra Kalahasti

narendra.Kalahasti@ANRITSU.COM

## Verification Test Summary

Test Case: 	8.1.6.1.1.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450+SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_1_1_NR5GC |
| --- | --- |
| Reason for change | The postamble is incorrectly carried out on NR Cell 11 instead of NR Cell 1 |
| Summary of change | Changed the cell ID to NR Cell1 in the postamble function. |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC.ttcn |
| MCC160 Comment | Accepted |  |

Before Change

function f_TC_8_1_6_1_1_1_NR5GC() runs on NR5GC_PTC { //Imediate MDT / Measurement reporting / Location information … … f_NR_TestBody_Set(true); f_TC_8_1_6_1_1_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); } //end of f_TC_8_1_6_1_1_1_NR5GC

After Change

function f_TC_8_1_6_1_1_1_NR5GC() runs on NR5GC_PTC { //Imediate MDT / Measurement reporting / Location information … … f_NR_TestBody_Set(true); f_TC_8_1_6_1_1_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } //end of f_TC_8_1_6_1_1_1_NR5GC

## Change 2

| Function name | f_TC_8_1_6_1_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The report amount in the Report config is incorrectly set to infinity. It should be r1 as the test body just expects one measurement report - The Location Error IE is an optional UE and may or may not be included by the UE - At step 7, according to the Prose, the check is for the UE to not send any MR in 10s. The function used currently in the TTCN is incorrect, and is actually waiting for the UE to send an MR |
| Summary of change | - Changes the report amoint to r1 - Changed the expectation of Location Error in Location information Measurement Report to anyOrOmit - Changed the function to f_NR_Check_NoMeasurementReport() to check no MRs are sent by the UE for 10s Note: CR R5-216826 associated with changes 1 and 2 has been raised at RAN5#93 |
| TTCN module | RRC_MDT_IntraNR_Immediate_NR5GC.ttcn |
| MCC160 Comment | Change 1: Accepted, but Prose CR needed Change 2: Accepted, but Prose CR needed Change 3: Accepted |  |

Before Change

function f_TC_8_1_6_1_1_1_NR5GC_TestBody() runs on NR_BASE_PTC { var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -98; var NR_AbsoluteCellPower_Type v_CellPower_T2_FR1 := -78; var PhysCellId v_PhysCellId; var RSRP_Range v_RsrpThres; var RSRP_Range v_AbsThreshSS_Rsrp; var Hysteresis v_Hysteresis; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement // and reporting for event A2 with includeLocationInfo configured. // The UE transmits an RRCReconfigurationComplete message. v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_AbsThreshSS_Rsrp := 57; v_RsrpThres := 69; v_Hysteresis := 6; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfigEventA2(tsc_NR_IdReportConfigId1, v_RsrpThres, infinity_, v_Hysteresis)}; v_ReportConfigList[0].reportConfig.reportConfigNR.reportType.eventTriggered.includeCommonLocationInfo_r16 := true_; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 3" siclog@ // The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T1_FR1, v_CellPower_T1_FR1); // FR2 FFS //@siclog "Step 4" siclog@ // Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE commonLocationInfo included? v_MeasResults := cr_38508_MeasResults_LocationInfo(-, v_PhysCellId, -, -, ?, ?, ?, ?, ?, ?); f_NR_ReceiveOneMeasurementReport(nr_Cell1, v_MeasResults); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ // SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T2_FR1, v_CellPower_T2_FR1); // FR2 FFS //@siclog "Step 6" siclog@ // Wait and ignore MeasurementReport messages for 5 s to allow change of power levels for NR Cell 1. f_NR_IgnorePeriodicalMeasurmentReport(5.0); //@siclog "Step 7" siclog@ // Check: Does the UE attempt to transmit MeasurementReport message within the next 10s? // 21 Reports for event A2 to ensure ca 10 sec (21*0.48=5,28) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, 21); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); }//end of f_TC_8_1_6_1_1_1_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_1_1_NR5GC_TestBody() runs on NR_BASE_PTC { var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -98; var NR_AbsoluteCellPower_Type v_CellPower_T2_FR1 := -78; var PhysCellId v_PhysCellId; var RSRP_Range v_RsrpThres; var RSRP_Range v_AbsThreshSS_Rsrp; var Hysteresis v_Hysteresis; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (present) MeasResults v_MeasResults; //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement // and reporting for event A2 with includeLocationInfo configured. // The UE transmits an RRCReconfigurationComplete message. v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_AbsThreshSS_Rsrp := 57; v_RsrpThres := 69; v_Hysteresis := 6; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp)}; v_ReportConfigList := {cs_NR_ReportConfigEventA2(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, v_Hysteresis)}; v_ReportConfigList[0].reportConfig.reportConfigNR.reportType.eventTriggered.includeCommonLocationInfo_r16 := true_; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 3" siclog@ // The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T1_FR1, v_CellPower_T1_FR1); // FR2 FFS //@siclog "Step 4" siclog@ // Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE commonLocationInfo included? v_MeasResults := cr_38508_MeasResults_LocationInfo(-, v_PhysCellId, -, -, ?, ?, ?, *, ?, ?); f_NR_ReceiveOneMeasurementReport(nr_Cell1, v_MeasResults); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ // SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.6.1.1.1.3.2-1. f_NR_SetCellPower (nr_Cell1, v_CellPower_T2_FR1, v_CellPower_T2_FR1); // FR2 FFS //@siclog "Step 6" siclog@ // Wait and ignore MeasurementReport messages for 5 s to allow change of power levels for NR Cell 1. f_NR_IgnorePeriodicalMeasurmentReport(5.0); //@siclog "Step 7" siclog@ // Check: Does the UE attempt to transmit MeasurementReport message within the next 10s? // 21 Reports for event A2 to ensure ca 10 sec (21*0.48=5,28) f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); }//end of f_TC_8_1_6_1_1_1_NR5GC_TestBody

# Branches executed

## This test case was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n41 using the Ciphering Algorithm nea2 and Integrity Algorithm nia2.

# Execution Log Files

## Qualcomm SM 8450+SDX65

The Qualcomm SM 8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset platform. The documentation below is enclosed as evidence of the successful test case run [1]:

•	Test case execution log file:

Keysight/TC_8_1_6_1_1_1_LOG.html

•	PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_1_1_PIXIT.html

The Qualcomm SM 8450+SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR platform. The documentation below is enclosed as evidence of the successful test case run [1]:

•	Test case execution log file:

Anritsu/TC_8_1_6_1_1_1_LOGS.xml

•	PICS/PIXIT parameter file:

Anritsu/TC_8_1_6_1_1_1_PIXIT.txt

# References

| [1] | R5s211596: NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.1.1 |
| --- | --- |