3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211626

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4652 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_MTC_ConnectPTCs_LTE_IRAT_Common function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Function f_MTC_ConnectPTCs_LTE_IRAT_Common is used to connect al PTCs for LTE-IRAT test cases. It presents a problem in implementation for test cases which can be run in LTE-WCDMA and LTE-GSM mode while IMS is activated, as example, 9.2.3.3.5a. Test cases affected can be recognised due to usage of px_RATComb_Tested as 8th input parameter in function f_MTC_ConnectPTCs_LTE_IRAT. Test cases like 9.2.3.3.5a create both UTRAN and GERAN components: v_EUTRA := EUTRA_PTC.create("EUTRA") alive; if (px_RATComb_Tested != EUTRA_Only) { v_UTRAN := UTRAN_IRAT_PTC.create("UTRAN") alive; v_GERAN := GERAN_PTC.create("GERAN") alive; } The problem comes inside function f_MTC_ConnectPTCs_LTE_IRAT_Common function when: if (p_Utran != null) { /* @sic R5-142919: sic@ when we have test case specific IMS behaviour and there is a UTRAN PTC => we need coordination of IMS and UTRAN @sic R5s150057 change 1: sic@ to allow IMS de-registration on UTRAN at the end of test cases UTRAN shall always be connected to the IMS PTC */ connect(vc_ImsPdn1:OtherIPCAN, p_Utran:IMS[tsc_Index_IMS1]); } if (p_Geran != null) { /* @sic R5-152083: sic@ when we have test case specific IMS behaviour and there is a GERAN PTC => we need coordination of IMS and GERAN */ connect(vc_ImsPdn1:OtherIPCAN, p_Geran:IMS[tsc_Index_IMS1]); } One to many port connections are established which is a legal operation, but the issue comes later if the testcase is run with IMS enabled device. The IMS default handler is invoked for IMS registration in following sequence f_IMS_DefaultHandlerf_IMS_DefaultHandler_InitialRegistration fl_IMS_DefaultHandler_SendCoOrdMsg f_IMS_IPCAN_SendCoOrdMsg. function f_IMS_IPCAN_SendCoOrdMsg(IMS_IPCAN_CO_ORD_PORT p_Port, template (value) IMS_IPCAN_Coordination_MSG p_Msg := cms_IMS_IPCAN_Trigger) { p_Port.send(p_Msg); } Reason why "to ..." is required comes from "ETSI ES 201 873-1 TTCN-3 Core Language" specification "22.2.1 The Send operation" description. Sending unicast, multicast or broadcast Unicast, multicast and broadcast communication can be determined by the optional to clause in the send operation. A to clause can be omitted in case of a one-to-one connection where unicast communication is used and the message receiver is uniquely determined by the test system structure. And restriction "d": d) A to clause shall be present in case of one-to-many connections. If "OtherIPCAN" is connected to 2 different ports, receiver for simple OtherIPCAN.send() operation can no longer be uniquely determined by the test system and exception is generated. connect(vc_ImsPdn1:OtherIPCAN, p_Utran:IMS[tsc_Index_IMS1]); connect(vc_ImsPdn1:OtherIPCAN, p_Geran:IMS[tsc_Index_IMS1]); This will cause the test case to fail as both connections ( UTRAN and GERAN) are established but not port to receive the message is specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified initialization so test cases which present a dual nature will connect vc_ImsPdn1:OtherIPCAN only to the port required for testing, p_Utran:IMS[tsc_Index_IMS1] or p_Geran:IMS[tsc_Index_IMS1] based on px_RATComb_Tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | LTE-IRAT test cases having both LTE-WCDMA and LTE-GSM mode. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	5

1.1	Correction to function f_MTC_ConnectPTCs_LTE_IRAT_Common function	5

# Corrections required

## Correction to function f_MTC_ConnectPTCs_LTE_IRAT_Common function

| Function name | Correction to f_MTC_ConnectPTCs_LTE_IRAT_Common function |
| --- | --- |
| Reason for change | A One to many port connections are established which is a legal operation but alter during the test case a send operation is call without specying the destination port. |
| Summary of change | Modified initialization so test cases which present a dual nature will connect vc_ImsPdn1:OtherIPCAN only to the port required for testing, p_Utran:IMS[tsc_Index_IMS1] or p_Geran:IMS[tsc_Index_IMS1] based on px_RATComb_Tested. |
| TTCN module | Common\MTC\MTC_Main_LTE_IRAT.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> if (p_ImsPdn1 == null) { /* => we have no test case specific IMS implementation */ vc_ImsPdn1.start(f_IMS_DefaultHandler()); } if (p_Utran != null) { /* @sic R5-142919: sic@ when we have test case specific IMS behaviour and there is a UTRAN PTC => we need coordination of IMS and UTRAN @sic R5s150057 change 1: sic@ to allow IMS de-registration on UTRAN at the end of test cases UTRAN shall always be connected to the IMS PTC */ connect(vc_ImsPdn1:OtherIPCAN, p_Utran:IMS[tsc_Index_IMS1]); } if (p_Geran != null) { /* @sic R5-152083: sic@ when we have test case specific IMS behaviour and there is a GERAN PTC => we need coordination of IMS and GERAN */ connect(vc_ImsPdn1:OtherIPCAN, p_Geran:IMS[tsc_Index_IMS1]); } // IMS for PDN (emergency call) if (p_ImsPdn2 != null) { // test case specific IMS handling for PDN2 connect(p_ImsPdn2:IMS_CTRL, vc_IP_PTC:IMS_CTRL[tsc_Index_IMS2]); connect(p_ImsPdn2:IMS_Server, vc_IP_PTC:IMS_Server[tsc_Index_IMS2]); connect(p_ImsPdn2:IMS_Client, vc_IP_PTC:IMS_Client[tsc_Index_IMS2]); connect(p_ImsPdn2:MMI, mtc:PTC_Ut[tsc_MTC_PortIndex_IMS2]); /* @sic R5-134070: generic handling of UT commands for IMS PTC as well as for the other PTCs sic@ @sic R5s130987 sic@ */ if (p_Eutra != null) {connect(p_ImsPdn2:IPCAN, p_Eutra:IMS[tsc_Index_IMS2]);} } // ***** UTRAN PTC ***** if ((p_Utran != null) and ((not isvalue(p_RatCombination) or (valueof(p_RatCombination) == EUTRA_UTRA)))) { // if p_RatCombination == EUTRA_GERAN the UTRAN PTC is used as dummy only to get sys info f_UTRAN_PTC_Map(p_System, p_Utran); } // ***** GERAN PTC ***** if ((p_Geran != null) and ((not isvalue(p_RatCombination) or (valueof(p_RatCombination) == EUTRA_GERAN)))) { // if p_RatCombination == EUTRA_GERAN the UTRAN PTC is used as dummy only to get sys info f_GERAN_PTC_Map(p_System, p_Geran); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (p_ImsPdn1 == null) { /* => we have no test case specific IMS implementation */ vc_ImsPdn1.start(f_IMS_DefaultHandler()); } <<REMOVED CODE>> if (p_Utran != null) { /* @sic R5-142919: sic@ when we have test case specific IMS behaviour and there is a UTRAN PTC => we need coordination of IMS and UTRAN @sic R5s150057 change 1: sic@ to allow IMS de-registration on UTRAN at the end of test cases UTRAN shall always be connected to the IMS PTC */ connect(vc_ImsPdn1:OtherIPCAN, p_Utran:IMS[tsc_Index_IMS1]); } if (p_Geran != null) { /* @sic R5-152083: sic@ when we have test case specific IMS behaviour and there is a GERAN PTC => we need coordination of IMS and GERAN */ connect(vc_ImsPdn1:OtherIPCAN, p_Geran:IMS[tsc_Index_IMS1]); } <<REMOVED CODE>> // IMS for PDN (emergency call) if (p_ImsPdn2 != null) { // test case specific IMS handling for PDN2 connect(p_ImsPdn2:IMS_CTRL, vc_IP_PTC:IMS_CTRL[tsc_Index_IMS2]); connect(p_ImsPdn2:IMS_Server, vc_IP_PTC:IMS_Server[tsc_Index_IMS2]); connect(p_ImsPdn2:IMS_Client, vc_IP_PTC:IMS_Client[tsc_Index_IMS2]); connect(p_ImsPdn2:MMI, mtc:PTC_Ut[tsc_MTC_PortIndex_IMS2]); /* @sic R5-134070: generic handling of UT commands for IMS PTC as well as for the other PTCs sic@ @sic R5s130987 sic@ */ if (p_Eutra != null) {connect(p_ImsPdn2:IPCAN, p_Eutra:IMS[tsc_Index_IMS2]);} } // ***** UTRAN PTC ***** if ((p_Utran != null) and ((not isvalue(p_RatCombination) or (valueof(p_RatCombination) == EUTRA_UTRA)))) { // if p_RatCombination == EUTRA_GERAN the UTRAN PTC is used as dummy only to get sys info connect(vc_ImsPdn1:OtherIPCAN, p_Utran:IMS[tsc_Index_IMS1]); f_UTRAN_PTC_Map(p_System, p_Utran); } // ***** GERAN PTC ***** if ((p_Geran != null) and ((not isvalue(p_RatCombination) or (valueof(p_RatCombination) == EUTRA_GERAN)))) { // if p_RatCombination == EUTRA_GERAN the UTRAN PTC is used as dummy only to get sys info connect(vc_ImsPdn1:OtherIPCAN, p_Geran:IMS[tsc_Index_IMS1]); f_GERAN_PTC_Map(p_System, p_Geran); } <<SKIPPED CODE>>