3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211655

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2202 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of 5GMM Rel-16 test case 9.1.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd, Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add 5GMM Rel-16 RACS test case 9.1.9.5 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GMM Rel-16 RACS test case 9.1.9.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

3	Branches executed	10

4	Execution Log Files	10

4.1	SM-G996B	10

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.9.5 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry
	shaun.harry@keysight.com
Verification Test Summary

Test Case: 	9.1.9.5.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Keysight S8704A Protocol Conformance Toolset

UE used:	SM-G996B

Verification Status:	PASS

# Corrections required

Below tabular changes are required to PASS this TC.

## Change 1

| Function name | function fl_TC_9_1_9_5_Body() |
| --- | --- |
| Reason for change | 1) Incorerct TAC code sent in REGISTRATION ACCEPT step 10. 2) SRB2 is not established so steps 12-14 will take place on SRB1 |
| Summary of change | 1) TAI list is fetched using correct TAC code for cell B. 2) SRB2 has been changed to SRB1 for steps 12-14 |
| TTCN module | RACS_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted. 2. Accepted. |

Before Change

function fl_TC_9_1_9_5_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) UERadioCapIdDeletion v_UERadioCapIdDeletion1 := { iei := 'A'H, spare := '0'B, deleteReq := '001'B }; var template (value) UERadioCapIdDeletion v_UERadioCapIdDeletion2 := { iei := 'E'H, spare := '0'B, deleteReq := '001'B }; //@siclog "Step 1" siclog@ //The SS transmits CONFIGURATION UPDATE COMMAND including UE radio capability ID deletion indication. SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_UERadioCapIdDeletion1)))); // @siclog "Step 2" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //@siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 4" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Step 5" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000020'O) ); //R5s211170 SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 6" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 7" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 8" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 9" siclog@ //The UE initiates a Registration procedure for mobility registration update by transmitting a REGISTRATION REQUEST message on NGC Cell B and includes the UE radio capability ID assigned at step 5. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_UERadioCapIdDeletion2 ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 11" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB); //@siclog "Step 12" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellB, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS)){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 14" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB, tsc_NR_RbId_SRB2); //@siclog "Step 15" siclog@ f_NR_RRCRelease(v_NGC_NASCellB); //@sic R5-216279 sic@ } //End of fl_TC_9_1_9_5_Body }//End of Module

After Change

function fl_TC_9_1_9_5_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) UERadioCapIdDeletion v_UERadioCapIdDeletion1 := { iei := 'A'H, spare := '0'B, deleteReq := '001'B }; var template (value) UERadioCapIdDeletion v_UERadioCapIdDeletion2 := { iei := 'E'H, spare := '0'B, deleteReq := '001'B }; //@siclog "Step 1" siclog@ //The SS transmits CONFIGURATION UPDATE COMMAND including UE radio capability ID deletion indication. SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('1'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_UERadioCapIdDeletion1)))); // @siclog "Step 2" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //@siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 4" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Step 5" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000020'O) ); //R5s211170 SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 6" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 7" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellB);//KS@9.1.9.5 v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)});//KS@9.1.9.5 //@siclog "Step 8" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 9" siclog@ //The UE initiates a Registration procedure for mobility registration update by transmitting a REGISTRATION REQUEST message on NGC Cell B and includes the UE radio capability ID assigned at step 5. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_UERadioCapIdDeletion2 ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 11" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB); //@siclog "Step 12" siclog@ //Check: Does UE transmit a REGISTRATION REQUEST message and does not include any Network-assigned UE radio capability ID? SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellB, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;//KS@9.1.9.5 v_ReceivedMsg := v_ReceivedAsp.Signalling.Nas[0]; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_RACS)){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//KS@9.1.9.5 //@siclog "Step 14" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB, tsc_NR_RbId_SRB1);//KS@9.1.9.5 //@siclog "Step 15" siclog@ f_NR_RRCRelease(v_NGC_NASCellB); //@sic R5-216279 sic@ } //End of fl_TC_9_1_9_5_Body }//End of Module

## Change 2

| Function name | f_Get_NG_RegistrationAcceptMsg() |
| --- | --- |
| Reason for change | Following the deletion of NW-assigned UE Radio Capability IDs at step 10, the UE should not include any network-assigned UE radio capability ID in the subsequent REGISTRATION REQUEST at step 12, but it may include any configured manufacturer-assigned UE Radio Capability ID. The current implementation of f_Get_NG_RegistrationAcceptMsg() (in accordance with 38.508-1 table 4.7.1-7), will include any UE radio access capability ID from the REGISTRATION REQUEST in the REGISTRATION ACCEPT, if pc_5GS_RACS is set and no specific UE radio cap ID is being provided for the message.So if the UE is including manufacturer-assigned UE radio access capility ID in REGISTRATION REQUEST, the REGISTRATION ACCEPT will also include a manufacturer-assigned UE radio access capability ID, which will not be expected by the UE and can cause undefined behaviour. From 24.501 4.16: f) upon receiving a UE radio capability ID deletion indication IE set to "delete network-assigned UE radio capability IDs" in the REGISTRATION ACCEPT message or the CONFIGURATION UPDATE COMMAND message, the UE shall delete all network-assigned UE radio capability IDs stored at the UE for the serving network, initiate a registration procedure for mobility and periodic registration update and include an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration, if available at the UE, in the UE radio capability ID IE of the REGISTRATION REQUEST message; and a) the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the REGISTRATION ACCEPT message or in the CONFIGURATION UPDATE COMMAND message; |
| Summary of change | TTCN check in f_Get_NG_RegistrationAcceptMsg is modified to check the first digit (TF Field)of UE radio capability id, so that if the UE is using manufacturer-assigned radio cap ID this is not included in the REGISTRATION ACCEPT message. Note : Associated prose CR on 38.508-1 |
| TTCN module | NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | Decicsion postponed until outcome of prose CR at RAN5 |

Before Change

function f_Get_NG_RegistrationAcceptMsg(TypeOfRegistration_Type p_RegType, GMM_MobilityInfo_Type p_GMM_MobilityInfo, // @sic R5-206426 sic@ template (omit) NG_MobileIdentity p_GUTI, // @sic R5s190719 sic@ template (omit) NG_TrackingAreaIdList p_TAIList, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NSSAI p_AllowedNSSAI := cs_NSSAI_Allowed_Def, // @sic R5s190109, R5-216256 sic@ template (omit) RejectedNSSAI p_RejectedNSSAI := omit, template (omit) NSSAI p_ConfiguredNSSAI := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, template (omit) PDU_SessionStatus p_PDU_SessionStatus := omit, template (omit) PDU_SessionReactivationResult p_PDU_SessionReactResult := omit, template (omit) PDU_SessionReactivationError p_PDU_SessionReactError := omit, template (omit) LADN_Info p_LADN_Info := omit, template (omit) MICO_Ind p_MICO_Ind := omit, template (omit) NetworkSlicingInd p_NetworkSlicingInd := omit, template (omit) ServiceAreaIdList p_SAIList := omit, template (omit) GPRS_Timer3 p_T3512 := omit, template (omit) GPRS_Timer2 p_Non3GPPDereg := omit, template (omit) GPRS_Timer2 p_T3502 := omit, template (omit) EmergNumList p_EmergNumList := omit, template (omit) ExtdEmergNumList p_ExtdEmergNumList := omit, template (omit) SORTransparentContainer p_SOR := omit, template (omit) EAP_Message p_EAP := omit, template (omit) InclusionModeNSSAI p_InclusionModeNSSAI := omit, template (omit) OperatorAccessCatDefinitions p_AccessCatDefinition := omit, template (omit) NG_DRXparameter p_NG_DRXparameter := omit, template (omit) Non3GPP_NW_ProvidedPolicies p_Non3GPP_NW_ProvidedPolicies := omit, template (omit) EPS_BearerContextStatus p_EPS_BearerContextStatus := omit, // @sic R5s190543 sic@ RegistrationAccessType p_Access := Access_3GPP, B1_Type p_SMSOverNAS := '0'B, B1_Type p_EmergencyRegistered := '0'B, // @sic R5s201387 Baseline Moving sic@ B1_Type p_NssaaPerformed := '0'B, // @sic R5s201387 Baseline Moving sic@ template (omit) ExtdDRXParams p_NegotiatedExtdDRXParams := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) GPRS_Timer3 p_T3447 := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) GPRS_Timer3 p_T3448 := omit, template (omit) GPRS_Timer3 p_T3324 := omit, template (omit) UERadioCapId p_UERadioCapId := omit, template (omit) UERadioCapIdDeletion p_UERadioCapIdDeletion := omit, template (omit) NSSAI p_PendingNSSAI := omit, template (omit) NG_CipheringKeyData p_CipheringKeyData := omit, template (omit) CAGInfoList p_CAGInfoList := omit, template (omit) TruncatedS_TMSIConfig p_TruncatedS_TMSIConfig := omit, template (omit) WUSAssistInfo p_WUSAssistInfo := omit, template (omit) NB_N1ModeDRXParams p_NB_N1ModeDRXParams := omit) return template (value) NG_NAS_DL_Message_Type { ………….. if (isvalue(p_PDU_SessionStatus)) { // @sic R5-206426 sic@ v_PDU_SessionStatus := p_PDU_SessionStatus; } else { v_PDU_SessionStatus := p_GMM_MobilityInfo.SessionStatus; } if (isvalue(p_EPS_BearerContextStatus)) { // @sic R5-206426 sic@ v_EPS_BearerContextStatus := p_EPS_BearerContextStatus; } else { v_EPS_BearerContextStatus := p_GMM_MobilityInfo.EPSBearerStatus; } if (isvalue(p_UERadioCapId)) { // @sic R5-206419 sic@ v_UERadioCapId := p_UERadioCapId; } else if (pc_5GC_RACS) { v_UERadioCapId := p_GMM_MobilityInfo.UERadioCapId; } // if (p_GSM_MobilityInfo.PDUType == SST_URLLC or SST_V2X ) v_NwkFeatureSupport := omit FFS // if (p_GSM_MobilityInfo.PDUType == SST_URLLC and/or SST_V2X and/or SST_MIIoT) := one or more AllowedNSSAI FFS v_NASMsg := cs_NG_REGISTRATION_ACCEPT(cs_RegistrationResult (v_Access, p_SMSOverNAS, p_EmergencyRegistered, p_NssaaPerformed), p_GUTI, p_EPLMNs, p_TAIList, p_AllowedNSSAI, ……… }

After Change

function f_Get_NG_RegistrationAcceptMsg(TypeOfRegistration_Type p_RegType, GMM_MobilityInfo_Type p_GMM_MobilityInfo, // @sic R5-206426 sic@ template (omit) NG_MobileIdentity p_GUTI, // @sic R5s190719 sic@ template (omit) NG_TrackingAreaIdList p_TAIList, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NSSAI p_AllowedNSSAI := cs_NSSAI_Allowed_Def, // @sic R5s190109, R5-216256 sic@ template (omit) RejectedNSSAI p_RejectedNSSAI := omit, template (omit) NSSAI p_ConfiguredNSSAI := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, template (omit) PDU_SessionStatus p_PDU_SessionStatus := omit, template (omit) PDU_SessionReactivationResult p_PDU_SessionReactResult := omit, template (omit) PDU_SessionReactivationError p_PDU_SessionReactError := omit, template (omit) LADN_Info p_LADN_Info := omit, template (omit) MICO_Ind p_MICO_Ind := omit, template (omit) NetworkSlicingInd p_NetworkSlicingInd := omit, template (omit) ServiceAreaIdList p_SAIList := omit, template (omit) GPRS_Timer3 p_T3512 := omit, template (omit) GPRS_Timer2 p_Non3GPPDereg := omit, template (omit) GPRS_Timer2 p_T3502 := omit, template (omit) EmergNumList p_EmergNumList := omit, template (omit) ExtdEmergNumList p_ExtdEmergNumList := omit, template (omit) SORTransparentContainer p_SOR := omit, template (omit) EAP_Message p_EAP := omit, template (omit) InclusionModeNSSAI p_InclusionModeNSSAI := omit, template (omit) OperatorAccessCatDefinitions p_AccessCatDefinition := omit, template (omit) NG_DRXparameter p_NG_DRXparameter := omit, template (omit) Non3GPP_NW_ProvidedPolicies p_Non3GPP_NW_ProvidedPolicies := omit, template (omit) EPS_BearerContextStatus p_EPS_BearerContextStatus := omit, // @sic R5s190543 sic@ RegistrationAccessType p_Access := Access_3GPP, B1_Type p_SMSOverNAS := '0'B, B1_Type p_EmergencyRegistered := '0'B, // @sic R5s201387 Baseline Moving sic@ B1_Type p_NssaaPerformed := '0'B, // @sic R5s201387 Baseline Moving sic@ template (omit) ExtdDRXParams p_NegotiatedExtdDRXParams := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) GPRS_Timer3 p_T3447 := omit, // @sic R5s201387 Baseline Moving sic@ template (omit) GPRS_Timer3 p_T3448 := omit, template (omit) GPRS_Timer3 p_T3324 := omit, template (omit) UERadioCapId p_UERadioCapId := omit, template (omit) UERadioCapIdDeletion p_UERadioCapIdDeletion := omit, template (omit) NSSAI p_PendingNSSAI := omit, template (omit) NG_CipheringKeyData p_CipheringKeyData := omit, template (omit) CAGInfoList p_CAGInfoList := omit, template (omit) TruncatedS_TMSIConfig p_TruncatedS_TMSIConfig := omit, template (omit) WUSAssistInfo p_WUSAssistInfo := omit, template (omit) NB_N1ModeDRXParams p_NB_N1ModeDRXParams := omit) return template (value) NG_NAS_DL_Message_Type { ……….. if (isvalue(p_PDU_SessionStatus)) { // @sic R5-206426 sic@ v_PDU_SessionStatus := p_PDU_SessionStatus; } else { v_PDU_SessionStatus := p_GMM_MobilityInfo.SessionStatus; } if (isvalue(p_EPS_BearerContextStatus)) { // @sic R5-206426 sic@ v_EPS_BearerContextStatus := p_EPS_BearerContextStatus; } else { v_EPS_BearerContextStatus := p_GMM_MobilityInfo.EPSBearerStatus; } if (isvalue(p_UERadioCapId)) { // @sic R5-206419 sic@ v_UERadioCapId := p_UERadioCapId; } else if (pc_5GC_RACS and ((p_GMM_MobilityInfo.UERadioCapId.id and4b 'F0'O) == '10'O)) { v_UERadioCapId := p_GMM_MobilityInfo.UERadioCapId; } // if (p_GSM_MobilityInfo.PDUType == SST_URLLC or SST_V2X ) v_NwkFeatureSupport := omit FFS // if (p_GSM_MobilityInfo.PDUType == SST_URLLC and/or SST_V2X and/or SST_MIIoT) := one or more AllowedNSSAI FFS v_NASMsg := cs_NG_REGISTRATION_ACCEPT(cs_RegistrationResult (v_Access, p_SMSOverNAS, p_EmergencyRegistered, p_NssaaPerformed), p_GUTI, p_EPLMNs, p_TAIList, ……….. }

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance ToolsetUE and Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_9_1_9_5_LOG.xml

Keysight\TC_9_1_9_5_LOG.html

- PICS/PIXIT parameter file:

Anritsu\ TC_9_1_9_5_PIXIT.txt

Keysight\ TC_9_1_9_5_PIXIT.xml

# References

| [1] | R5s211656 : NR5GC FR1 : Supporting information for addition of 5GMM Rel-16 test case 9.1.9.5 |
| --- | --- |