3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211673

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2210 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NAS 5GMM test case 9.1.5.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - After deregistration in Preamble, UE has done a local release of the PDU session established and UE should be allowed to re-establish it during the Initial Registration inside the Test Body #associtated draft proseCR would be submitted during RAN5#93-e meeting - From Change#1, RRCrelease performed due to which Postamble is performed in IDLE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Handle PDU session establishment from UE after Registration Complete. - Postamble is in IDLE state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2            Change 2	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | fl_TC_9_1_5_1_15_TestBody() |
| --- | --- |
| Reason for change | After deregistration in Preamble, UE has done a local release of the PDU session established and UE should be allowed to re-establish it during the Initial Registration inside the Test Body #associtated draft proseCR would be submitted during RAN5#93-e meeting |
| Summary of change | Handle PDU session establishment from UE after Registration Complete. |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#94-e. The Change1 “f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA);” does not release the RRC Connection as the defaut parameter is “noRrcConnectionRelease”. It is recommened that “Steps 19a1” is the only step to be added in the prose CR for next RAN5 meeting. |

Before Change

function fl_TC_9_1_5_1_15_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var TrackingAreaCode v_TAC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_EmptyCAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-213385 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); } } //End of fl_TC_9_1_5_1_15_Body

After Change

function fl_TC_9_1_5_1_15_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; var TrackingAreaCode v_TAC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_EmptyCAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog Step 17a1-18a1 siclog@ // Steps 19a1-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA); v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-213385 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); } } //End of fl_TC_9_1_5_1_15_Body

  - Change 2

| Function name | f_TC_9_1_5_1_15_NR5GC() |
| --- | --- |
| Reason for change | From Change#1, RRCrelease performed due to which Postamble is performed in IDLE |
| Summary of change | Postamble is in IDLE state |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment | Not needed. As RRC connection is not released acc to Change1 requested. |

Before Change

function f_TC_9_1_5_1_15_NR5GC() runs on NR5GC_PTC { //Initial registration / Success / Extended and spare fields in UE Network Capability var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_5_1_15_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); }

After Change

function f_TC_9_1_5_1_15_NR5GC() runs on NR5GC_PTC { //Initial registration / Success / Extended and spare fields in UE Network Capability var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_5_1_15_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }