3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211674

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2211 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MDT TC 8.1.6.1.2.9 Logged MDT / Location information |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to MDT test case 8.1.6.1.2.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications 38.523-1 |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

3	Branches Executed	9

4	Execution Log Files	9

4.1	Samsung SM-G996B	9

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.9 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.9

ATS Version:	iWD-TTCN3-B2020-09_D21wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_9_NR5GC() |
| --- | --- |
| Reason for change | CellID should be corrected at step 5 and UE information request should be sent for logged measurement instead random access at step 19 |
| Summary of change | CellID 11 updated and UE information request updated for logged measurement. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted, see alternative optimised implementation Also a prose CR is needed to fix incorrect contents of locationInfo-r16 IE in Table 8.1.6.1.2.9.3.3-4 |

Before Change

function f_TC_8_1_6_1_2_9_NR5GC() runs on NR5GC_PTC { /* Logged MDT / Location information */ …. //@siclog "Step 4" siclog@ //The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.9.3.2-1.(Note 1) f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5-7" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) v_ReceivedMsg := f_NR_RRC_ConnEst_SON_MDT (nr_Cell1, logMeasAvailable); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 7: RRCSetupComplete message with logMeasAvailable included"); //@siclog "Steps 8-10" siclog@ //Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11. //NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11-18" siclog@ //Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1[4] subclause 4.5.4.2-3 are executed //to successfully complete the service request procedure on Cell 11 f_NR5GC_RRCIdleToConnected(nr_Cell11); //@siclog "Step 19" siclog@ //The SS transmits a UEInformationRequest message on Cell 11 SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_RLF)); //@siclog "Step 20" siclog@ //Check: Does the UE transmit a UEInformationResponse message where the IE CommonLocationInfo is present. v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(nr_Cell11); v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); v_CellIdentityServing := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell11); v_LogMeasInfo_Expected := cr_LogMeasInfo_r16_logMeasInfo(cr_NR_RLF_ReestablishmentCellId(v_CellIdentityServing), {cr_MeasResultLogging2NR_r16(v_ARFCN_Neighbour, {cr_MeasResultLoggingNR_r16(v_PhysCellIdNeighbour)})}); v_LogMeasInfo_Expected.locationInfo_r16 := ?; f_NR_ReceiveAndVerifyLogMeasInfoList_r16(nr_Cell11, v_LogMeasInfo_Expected, "Step 20"); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_2_9_NR5GC()

After Change

f function f_TC_8_1_6_1_2_9_NR5GC() runs on NR5GC_PTC { /* Logged MDT / Location information */ …. //@siclog "Step 4" siclog@ //The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.9.3.2-1.(Note 1) f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5-7" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) v_ReceivedMsg := f_NR_RRC_ConnEst_SON_MDT (nr_Cell11, logMeasAvailable); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 7: RRCSetupComplete message with logMeasAvailable included"); //@siclog "Steps 8-10" siclog@ //Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11. //NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11-18" siclog@ //Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1[4] subclause 4.5.4.2-3 are executed //to successfully complete the service request procedure on Cell 11 f_NR5GC_RRCIdleToConnected(nr_Cell11); //@siclog "Step 19" siclog@ //The SS transmits a UEInformationRequest message on Cell 11 SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_LOG)); //@siclog "Step 20" siclog@ //Check: Does the UE transmit a UEInformationResponse message where the IE CommonLocationInfo is present. v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(nr_Cell11); v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); v_CellIdentityServing := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell11); v_LogMeasInfo_Expected := cr_LogMeasInfo_r16_logMeasInfo(cr_NR_RLF_ReestablishmentCellId(v_CellIdentityServing), {cr_MeasResultLogging2NR_r16(v_ARFCN_Neighbour, {cr_MeasResultLoggingNR_r16(v_PhysCellIdNeighbour)})}); v_LogMeasInfo_Expected.locationInfo_r16 := ?; f_NR_ReceiveAndVerifyLogMeasInfoList_r16(nr_Cell11, v_LogMeasInfo_Expected, "Step 20"); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_2_9_NR5GC()

##### MCC160 Alternative implementation due to resolving of a function f_NR_RRC_ConnEst_SON_MDT:

function f_TC_8_1_6_1_2_9_NR5GC() runs on NR5GC_PTC { /* Logged MDT / Location information */ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_HigherRanked_FR1; var NR_AbsoluteCellPower_Type v_CellPower_HigherRanked_FR2; var NR_AbsoluteCellPower_Type v_CellPower_LowerRanked_FR1; var NR_AbsoluteCellPower_Type v_CellPower_LowerRanked_FR2; var PhysCellId v_PhysCellIdNeighbour; var ARFCN_ValueNR v_ARFCN_Neighbour; var CellIdentity v_CellIdentityServing; var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16; var PLMN_Identity v_PlmnID; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template LogMeasInfo_r16 v_LogMeasInfo_Expected; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template RRCSetupComplete_v1610_IEs v_RRCSetupComplete_v1610_IEs_UeMeas; f_NR5GC_Init(NR_2); //Table 8.1.6.1.3.1.3.2-1/2 v_CellPower_HigherRanked_FR1 := -78; v_CellPower_HigherRanked_FR2 := tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2; //@sic R5-216304 sic@ v_CellPower_LowerRanked_FR1 := -85; v_CellPower_LowerRanked_FR2 := tsc_NR_ServingCellSSS_EPRE_FR2; //@sic R5-216304 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_HigherRanked_FR1, v_CellPower_HigherRanked_FR2), cs_NR_CellPower(nr_Cell11, v_CellPower_LowerRanked_FR1, v_CellPower_LowerRanked_FR2)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_LowerRanked_FR1, v_CellPower_LowerRanked_FR2), cs_NR_CellPower(nr_Cell11, v_CellPower_HigherRanked_FR1, v_CellPower_HigherRanked_FR2)}; f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, v_CellPower_HigherRanked_FR1, v_CellPower_HigherRanked_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, v_CellPower_HigherRanked_FR1, v_CellPower_HigherRanked_FR2); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); //Preamble //The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); /*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation. *Otherwise, or in addition any other suitable method may also be used.*/ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_CONNECTED); f_NR_TestBody_Set(true); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. v_PlmnID := f_NR_CellInfo_GetPLMN (nr_Cell1); v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 5s to allow UE to activate logging f_Delay(5.0); //@siclog "Step 4" siclog@ //The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.9.3.2-1.(Note 1) f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 5-7" siclog@ //The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure. (Note 1) v_RRCSetupComplete_v1610_IEs_UeMeas := f_NR5GC_Initialise_RRCSetupComplete_v1610_IEs_UE_MeasurementsAvailable(logMeasAvailable); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4 (nr_Cell1, ?, ? , v_RRCSetupComplete_v1610_IEs_UeMeas); //@sic R5s211647 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 7: RRCSetupComplete message with logMeasAvailable included"); //@siclog "Steps 8-10" siclog@ //Steps 4 to 6 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 11. //NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo); //@siclog "Steps 11-18" siclog@ //Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1[4] subclause 4.5.4.2-3 are executed //to successfully complete the service request procedure on Cell 11 f_NR5GC_RRCIdleToConnected(nr_Cell11); //@siclog "Step 19" siclog@ //The SS transmits a UEInformationRequest message on Cell 11 SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_LOG)); //@sic R5s211647 sic@ //@siclog "Step 20" siclog@ //Check: Does the UE transmit a UEInformationResponse message where the IE CommonLocationInfo is present. v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(nr_Cell11); v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); v_CellIdentityServing := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell11); v_LogMeasInfo_Expected := cr_LogMeasInfo_r16_logMeasInfo(cr_NR_CGI_Info_Logging_r16_Def(v_CellIdentityServing), {cr_MeasResultLogging2NR_r16(v_ARFCN_Neighbour, {cr_MeasResultLoggingNR_r16(v_PhysCellIdNeighbour)})}); v_LogMeasInfo_Expected.locationInfo_r16 := ?; f(nr_Cell11, v_PlmnID, v_AbsoluteTimeInfo_r16, v_LogMeasInfo_Expected, "Step 20"); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_2_9_NR5GC()

## Change 2

| Function name | f_NR_ReceiveAndVerifyLogMeasInfoList_r16() |
| --- | --- |
| Reason for change | v_ReceivedLogMeasInfoList_r16 should be accessed with index ‘i’ while comparing with expected template p_ExpectedLogMeasInfo_r16 |
| Summary of change | v_ReceivedLogMeasInfoList_r16[i] used instead of v_ReceivedLogMeasInfoList_r16 |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Swap of parameters is not needed; alternative implementation of the function is proposed to verify basic contents of UEInformationResponse |

Before Change

function f_NR_ReceiveAndVerifyLogMeasInfoList_r16 ( NR_CellId_Type p_CellId, template (present)LogMeasInfo_r16 p_ExpectedLogMeasInfo_r16, charstring p_TestStep := "Step 20") runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedMsg; var template (present) LogMeasInfoList_r16 v_ReceivedLogMeasInfoList_r16; var LogMeasInfo_r16 v_ExpectedLogMeasInfo_r16; var integer i; var integer v_ListLengh; var boolean v_Match := false; v_ExpectedLogMeasInfo_r16 := valueof(p_ExpectedLogMeasInfo_r16); SRB.receive(car_NR_SRB1_RrcPdu_IND(p_CellId, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-,-,?,-,-,-))) -> value v_ReceivedMsg; v_ReceivedLogMeasInfoList_r16 := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16; v_ListLengh := lengthof(v_ReceivedLogMeasInfoList_r16); for (i:=0;i<v_ListLengh;i:=i+1) { if(match(v_ExpectedLogMeasInfo_r16, v_ReceivedLogMeasInfoList_r16[i])) { v_Match := true; break; } } if(v_Match) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } }

After Change

function f_NR_ReceiveAndVerifyLogMeasInfoList_r16 ( NR_CellId_Type p_CellId, template (present)LogMeasInfo_r16 p_ExpectedLogMeasInfo_r16, charstring p_TestStep := "Step 20") runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedMsg; var template (present) LogMeasInfoList_r16 v_ReceivedLogMeasInfoList_r16; var LogMeasInfo_r16 v_ExpectedLogMeasInfo_r16; var integer i; var integer v_ListLengh; var boolean v_Match := false; v_ExpectedLogMeasInfo_r16 := p_ExpectedLogMeasInfo_r16; SRB.receive(car_NR_SRB1_RrcPdu_IND(p_CellId, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-,?,-,-,-,-))) -> value v_ReceivedMsg; v_ReceivedLogMeasInfoList_r16 := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16; v_ListLengh := lengthof(v_ReceivedLogMeasInfoList_r16); for (i:=0;i<v_ListLengh;i:=i+1) { if(match(v_ReceivedLogMeasInfoList_r16[i],v_ExpectedLogMeasInfo_r16 )) { v_Match := true; break; } } if(v_Match) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } }

##### MCC160 Alternative implementation of f_NR_ReceiveAndVerifyLogMeasInfoList_r16:

function f_NR_ReceiveAndVerifyLogMeasInfoList_r16 ( NR_CellId_Type p_CellId, PLMN_Identity p_PlmnID, AbsoluteTimeInfo_r16 p_AbsoluteTimeInfo_r16, template LogMeasInfo_r16 p_ExpectedLogMeasInfo_r16, charstring p_TestStep := "Step 20") runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedMsg; var template (present) LogMeasInfoList_r16 v_ReceivedLogMeasInfoList_r16; var LogMeasInfo_r16 v_ExpectedLogMeasInfo_r16; var integer i; var integer v_ListLengh; var boolean v_Match := false; v_ExpectedLogMeasInfo_r16 := valueof(p_ExpectedLogMeasInfo_r16); SRB.receive(car_NR_SRB_RrcPdu_IND(p_CellId, tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-,cr_LogMeasReport_r16_LogMeasInfo_Def(p_AbsoluteTimeInfo_r16, cr_TraceReference_r16(p_PlmnID)),-,-,-))) -> value v_ReceivedMsg; v_ReceivedLogMeasInfoList_r16 := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16; v_ListLengh := lengthof(v_ReceivedLogMeasInfoList_r16); for (i:=0;i<v_ListLengh;i:=i+1) { if(match(v_ExpectedLogMeasInfo_r16, v_ReceivedLogMeasInfoList_r16[i])) { v_Match := true; break; } } if(v_Match) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " " & p_TestStep); } }

# Branches Executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B 5G UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_6_1_2_9_LOG.xml

- PICS/PIXIT parameter file:

TC_8_1_6_1_2_9_PIXIT.txt

# References

R5s211675 - NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.9