3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211676

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2212 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MDT TC 8.1.6.1.2.7 Logged MDT / Logging and reporting / Indication of logged measurements at NR reestablishment |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to MDT test case 8.1.6.1.2.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

3	Branches Executed	9

4	Execution Log Files	10

4.1	Samsung SM-G996B	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.7 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.7

ATS Version:	iWD-TTCN3-B2020-09_D21wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR
	Keysight 5G Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_7_NR5GC() |
| --- | --- |
| Reason for change | Set initial max reference power for both NR cell1 and cell 2. |
| Summary of change | Power level for both NR cell1 and cell2 is set to -85 during the test case. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Rejected. According to the associated draft prose CR, the change is not needed. |

Before Change

function f_TC_8_1_6_1_2_7_NR5GC() runs on NR5GC_PTC { // Logged MDT / Logging and reporting / Indication of logged measurements at NR reestablishment f_NR5GC_Init(NR_2); //Added initial cell ref Power //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_6_1_2_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell2, STATE_IDLE_1A); }

After Change

function f_TC_8_1_6_1_2_7_NR5GC() runs on NR5GC_PTC { // Logged MDT / Logging and reporting / Indication of logged measurements at NR reestablishment f_NR5GC_Init(NR_2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -85, -85); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_6_1_2_7_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell2, STATE_IDLE_1A); }

## Change 2

| Function name | f_TC_8_1_6_1_2_7_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Power levels of cell 1 is set to -88 initially and on time T1 is being changed to -85 which will stop UE to trigger trigger re-establishment procedure at step 13. |
| Summary of change | Cell 1 power changed to non suitable cell at T1 time instance. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement. Note: Cell 1 power is actually changed to non suitable off cell. |

Before Change

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var float v_WaitToLogging := 30.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, -85, -82),//@sic R5-216303 sic@ cs_NR_CellPower (nr_Cell2, -91, -91) }; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ -- -- f_NR_RRCRelease(nr_Cell2); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var float v_WaitToLogging := 30.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell2, -91, -91) }; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ -- -- f_NR_RRCRelease(nr_Cell2); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody

## Change 3

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution() |
| --- | --- |
| Reason for change | Cell 2 is not initialized with condition of rrcsetup to be triggred with msg4 |
| Summary of change | Added TC 8.1.6.1.2.7 condition |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted Note: Contrary to the above description, this is needed for the RRCReestablishment message at step 14. |

Before Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } //Added TC 8.1.6.1.2.7 here case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } //Anritsu TTCN CR R5s211593 } return false; }

After Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_2_7_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } //Anritsu TTCN CR R5s211593 } return false; }

# Branches Executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B 5G UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_8_1_6_1_2_7_LOG.xml

Keysight/TC_8_1_6_1_2_7_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_8_1_6_1_2_7_PIXIT.txt

Anritsu/TC_8_1_6_1_2_7_PIXIT.xml

# References

R5s211677 - NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.7