3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211691

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2216 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC testcase 9.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Due to OS limitation, e.g. Andriod S, the current data “2010 12 April” is out of date and is not allowed to be set in UE when checking the NITZ by MMI. Therefore it is suggested to update the date to current local date. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Set the date to local date in step 12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_9_1_4_1_TestBody	2

# Corrections required

## Correction to function fl_TC_9_1_4_1_TestBody

| Function name | fl_TC_9_1_4_1_TestBody |
| --- | --- |
| Reason for change | - Due to OS limitation, e.g. Andriod S, the current data “2010 12 April” is out of date and is not allowed to be set in UE when checking the NITZ by MMI. Therefore it is suggested to update the date to current local date or a new time in future. Note :it is supported with a prose CR. |
| Summary of change | - Modify 2010 to 2030. |
| TTCN module | ConfigurationUpdate_NR5GC.ttcn |
| MCC160 Comment | Implemented as per R5-221451 |

Before Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC { …… //@siclog "Steps 5-11" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(v_NGC_NASCellA); f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); f_UT_AT_SubscribeTimeZoneReporting(UT); // @sic R5s200573 sic@ f_UT_AT_TriggerTimezoneReport (UT, "2010", "04", "12", "13", "??", "??", "\+04", "1"); // @sic R5s200573 sic@ //@siclog "Step 12" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, -, -, cs_NetworkName ('43'O, v_FullNameString), cs_NetworkName ('45'O, v_ShortNameString), // @sic R5-198186 sic@ cs_TimeZone('40'O), // @sic R5-198186 sic@ cs_TimeZoneAndTime('01'O, '40'O, '21'O, '31'O, '83'O, '25'O,'40'O), cs_DaylightSavingTime_tlv('01'B))))); // @sic R5-198186 sic@ //@siclog "Steps 13" siclog@ t_T3555.start; alt{ [] SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13"); } [] t_T3555.timeout {} } v_AT_Response := f_UT_AT_CheckTimezoneReport (UT); // CTZE response expected @sic R5s200573 sic@ //@siclog "Steps 14" siclog@ f_AT_ResponseCheck(v_AT_Response, "2010/04/12,14:[0-9]#(2):[0-9]#(2)", // TZ GMT+1, dst (1 char) ignored, check date and hour, ignore mins, secs @sic R5s200903 sic@ "Step 14 (UT and local time zone)", v_Expression_CTZR, 2); f_AT_ResponseCheck(v_AT_Response, "1", // DST=1 "Step 14 DST", v_Expression_CTZR, 1); v_AT_Response := f_UT_ChkOperatorName (UT, LONG_NAME); f_AT_ResponseCheck (v_AT_Response, v_FullNameString, "Step 14 (long network name)"); v_AT_Response := f_UT_ChkOperatorName (UT, SHORT_NAME); f_AT_ResponseCheck (v_AT_Response, v_ShortNameString, "Step 14 (short network name)"); v_AT_Response := f_UT_ChkTimeZone (UT); f_AT_ResponseCheck(v_AT_Response, "*[0-9]#(2)/[0-9]#(2)/[0-9]#(2),[0-9]#(2):[0-9]#(2):[0-9]#(2)\+04*", // only TZ to be checked "Step 14 (local time zone)"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Steps 15" siclog@ // Switch off the UE f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_CONNECTED_3A); …… }

After Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC { …… //@siclog "Steps 5-11" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(v_NGC_NASCellA); f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); f_UT_AT_SubscribeTimeZoneReporting(UT); // @sic R5s200573 sic@ f_UT_AT_TriggerTimezoneReport (UT, "2030", "04", "12", "13", "??", "??", "\+04", "1"); // @sic R5s200573 sic@ //@siclog "Step 12" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, -, -, cs_NetworkName ('43'O, v_FullNameString), cs_NetworkName ('45'O, v_ShortNameString), // @sic R5-198186 sic@ cs_TimeZone('40'O), // @sic R5-198186 sic@ cs_TimeZoneAndTime('03'O, '40'O, '21'O, '31'O, '83'O, '25'O,'40'O), cs_DaylightSavingTime_tlv('01'B))))); // @sic R5-198186 sic@ //@siclog "Steps 13" siclog@ t_T3555.start; alt{ [] SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13"); } [] t_T3555.timeout {} } v_AT_Response := f_UT_AT_CheckTimezoneReport (UT); // CTZE response expected @sic R5s200573 sic@ //@siclog "Steps 14" siclog@ f_AT_ResponseCheck(v_AT_Response, "2030/04/12,14:[0-9]#(2):[0-9]#(2)", // TZ GMT+1, dst (1 char) ignored, check date and hour, ignore mins, secs @sic R5s200903 sic@ "Step 14 (UT and local time zone)", v_Expression_CTZR, 2); f_AT_ResponseCheck(v_AT_Response, "1", // DST=1 "Step 14 DST", v_Expression_CTZR, 1); v_AT_Response := f_UT_ChkOperatorName (UT, LONG_NAME); f_AT_ResponseCheck (v_AT_Response, v_FullNameString, "Step 14 (long network name)"); v_AT_Response := f_UT_ChkOperatorName (UT, SHORT_NAME); f_AT_ResponseCheck (v_AT_Response, v_ShortNameString, "Step 14 (short network name)"); v_AT_Response := f_UT_ChkTimeZone (UT); f_AT_ResponseCheck(v_AT_Response, "*[0-9]#(2)/[0-9]#(2)/[0-9]#(2),[0-9]#(2):[0-9]#(2):[0-9]#(2)\+04*", // only TZ to be checked "Step 14 (local time zone)"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Steps 15" siclog@ // Switch off the UE f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_CONNECTED_3A); …… }