3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211721

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2232 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC MDT test case 8.1.6.1.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC test case 8.1.6.1.4.3 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.6.1.4.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 8.1.6.1.4.3	3

2.1	function f_TC_8_1_6_1_4_3_NR5GC	3

3	Branches executed	4

4	Execution Log Files	4

4.1	MediaTek MT6983	4

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.6.1.4.3 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	TC_8_1_6_1_4_3_NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required to test case 8.1.6.1.4.3

## function f_TC_8_1_6_1_4_3_NR5GC

| Function name | function f_TC_8_1_6_1_4_3_NR5GC |
| --- | --- |
| Reason for change | At the end of the test procedure the UE is connected to NR Cell 3 and therefore the postamble procedure will be on Cell3 |
| Summary of change | Modified the first parameter in f_NR_Postamble to nr_Cell3 |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC |
| MCC160 Comment | Accepted. |

Before Change:

function f_TC_8_1_6_1_4_3_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover f_NR5GC_Init(NR_4); //System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2. f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -73, -73); // Power levels are FFS for FR2 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A);//The UE is in state 1N-A as defined in TS 38.508-1, subclause 4.4A, on cell 1 f_NR_TestBody_Set(true); f_TC_8_1_6_1_4_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_4_3_NR5GC()

After Change:

function f_TC_8_1_6_1_4_3_NR5GC() runs on NR5GC_PTC { //Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover f_NR5GC_Init(NR_4); //System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.2. f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -73, -73); // Power levels are FFS for FR2 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A);//The UE is in state 1N-A as defined in TS 38.508-1, subclause 4.4A, on cell 1 f_NR_TestBody_Set(true); f_TC_8_1_6_1_4_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell3, STATE_CONNECTED_3A); //UE is connected to Cell3 }// End of f_TC_8_1_6_1_4_3_NR5GC()

## Additional MCC160 implementation

function f_TC_8_1_6_1_4_3_NR5GC_TestBody() runs on NR5GC_PTC

{…

//@siclog "Step 16" siclog@

//The SS changes cell 1 and cell 3 parameters according to the row "T1"in table 8.1.6.1.4.3.3.2-1

f_NR_SetCellPower (nr_Cell3, -78, -78); //@sic R5s211721 sic@

…

}//End of f_TC_8_1_6_1_4_3_TestBody()

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NR ciphering algorithm nea2 and integrity algorithm nia2 on Rohde & Schwarz 5G Protocol Conformance Test platform

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_6_1_4_3_NR5GC_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

| MCC160 Comment | Log file in R5s211722 does not include change 2.2, so it has been replaced by new SS log file in R5s220189, which is accepted. |
| --- | --- |

# References

| [1] | R5s211722: Supporting information for addition of NR5GC TC 8.1.6.1.4.3. |
| --- | --- |