3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220014

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4657 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE EMM testcase 9.2.3.1.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-12-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Testcase 9.2.3.1.25 fails due to insufficient guardtimer value, when run on CE mode B UE. As per, NOTE 1 from Table 9.2.3.1.25.3.2-1, CE mode B UE would wait for 87s at 8 different steps, along with 6min & 12min at Step 12 & 23 respectively. The overall wait time throughout the testcase procedure is 1776s, which results in insufficient time for remaining steps when guardtimer is 1900s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Guard timer value increased from 1900s to 2700s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | TC_9_2_3_1_25 |
| --- | --- |
| Reason for change | Testcase 9.2.3.1.25 fails due to insufficient guardtimer value, when run on CE mode B UE. As per, NOTE 1 from Table 9.2.3.1.25.3.2-1, CE mode B UE would wait for 87s at 8 different steps, along with 6min & 12min at Step 12 & 23 respectively. The overall wait time throughout the testcase procedure is 1776s, which results in insufficient time for remaining steps when guardtimer is 1900s. |
| Summary of change | Guard timer value increased from 1900s to 2700s. |
| TTCN module | LTE_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before change:

testcase TC_9_2_3_1_25() runs on MTC_LTE system SYSTEM_LTE {

// @purpose

//   Normal tracking area update / Abnormal case / Failure after 5 attempts due to no network response

var EUTRA_PTC        v_EUTRA      := null;

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(1900);

v_EUTRA := EUTRA_PTC.create("EUTRA") alive;

f_MTC_ConnectPTCs_LTE(system, v_EUTRA, v_IMS1, v_IMS2);

v_EUTRA.start(f_TC_9_2_3_1_25_EUTRA());

t_GuardTimer.start;

f_MTC_MainLoop_LTE(t_GuardTimer);

}

After change:

testcase TC_9_2_3_1_25() runs on MTC_LTE system SYSTEM_LTE {

// @purpose

//   Normal tracking area update / Abnormal case / Failure after 5 attempts due to no network response

var EUTRA_PTC        v_EUTRA      := null;

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(2700);

v_EUTRA := EUTRA_PTC.create("EUTRA") alive;

f_MTC_ConnectPTCs_LTE(system, v_EUTRA, v_IMS1, v_IMS2);

v_EUTRA.start(f_TC_9_2_3_1_25_EUTRA());

t_GuardTimer.start;

f_MTC_MainLoop_LTE(t_GuardTimer);

}