3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220015

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2249 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In step 20a the ACK SN needs to be 2 because the step 9A already used ACK SN=1, since then the SN incremented by 1. Otherwise, UE shall RLC re-transmit the last UL PDU which will cause test failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incremented ACK SN for RLC PDU in step 20A by one. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_9_1_Step16Ato21 |
| --- | --- |
| Reason for change | In step 20a the ACK SN needs to be 2 because the step 9A already used ACK SN=1, since then the SN incremented by 1. Otherwise, UE shall RLC re-transmit the last UL PDU which will cause test failure. |
| Summary of change | Incremented ACK SN for RLC PDU in step 20A by one. |
| TTCN module | NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted. |

Before change:

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC

{

...

//@siclog "Step 20A" siclog@

//@sic R5s201457 sic@

f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id);

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@

}

After change:

function f_TC_7_1_1_9_1_Step16Ato21(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC

{

...

//@siclog "Step 20A" siclog@

//@sic R5s201457 sic@

f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 2, p_NR_DRB_Id);

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

deactivate(v_IgnoreNRSR);//@sic R5-204418 sic@

}